/*! \page events Events

\addindex events

The TACO control system was originally based on synchronous remote

procedure calls (RPCs) and the client-server model. Clients and servers which

required asynchronism made use of the data collector (a distributed online

buffer of device command results) or the servers implemented their own mini-buffers

locally and the clients polled the server. This is not always efficient in terms

of time, network bandwidth and CPU usage. Therefore an asynchronous call was added

and has been available for over a year now. The asynchronous call implements the

mechanisms necessary to add events without much effort. It was logical therefore

with the recent move towards Linux on frontends to take advantage of the excellent

TCP/IP stack implementation on Linux to offer programmers and clients events.

The present implementation offers a simple model for user events which will permit

device server programmers to add their own events (user events) to their code

thereby providing adding value to their device servers. The present implementation

is ideal for device servers which have a small number of clients. A full

implementation with sophisticated system and user events which provides efficient

mechanisms for distributing events to large numbers of clients will be implemented

in TANGO (next generation TACO). The present implementation in TACO is simply an

avantgout of TANGO events and allows TACO programmers to gain experience using

events. This chapter presents the user event api, examples of how to program them

and a discussion on performance and problems which can arise.

\section Events

Events are short messages which are sent to clients asynchronously. The origin of

the messages is a device server. Clients only receive messages if they have solicited

them. Events are classified according to type. Event types are specific to the

device server and should be defined as unique long integers. The most obvious way

to do so is to use the device class unique base as offset and number events starting

from 1 e.g. :

\code

#define D_EVENT_AGPS_STATE 		DevAgpsBase + 1

#define D_EVENT_OMS_STATE_CHANGE 	DevOmsBase + 1

\endcode

*/

\section event_api API

The event API consists of three additional calls which are distributed as part of

the DSAPI. The API consists of a client part and a server part. The client part

allows a client to register its interest in events , to receive events and to

unregister once it is finished. The server part allows servers to dispatch events

to clients. The server has to program how to trigger events.

\subsection client_event_api Client side

- dev_event_listen() - register a callback for an event type

long dev.event.listen (devserver ds, long event.type,

DevArgument argout, DevType argout.type DevCallbackFunction *callback, void *user.data, long *event.id.ptr, long *error)

devserver ds - device from which client wants to receive events long event.type - type of event to receive DevArgument argout - pointer to argout data (if any) which will be sent with event DevType argout.type - argout type DevCallbackFunction *callback - pointer to callback function void *user.data - pointer to user data to pass to callback function long *event.id.ptr - pointer to event id (returned by dev.event.listen()) long *error - pointer to error code (if any)

ffl dev event unlisten() - unregister a callback for an event type

long dev.event.unlisten (devserver ds, long event.type,

long event.id, long *error)

devserver ds - device from which to unregister client's interest in event long event.type - event type to unregister long event.id - event id (returned by dev.event.listen()) long *error - pointer to error code (if any)long dev.event.fire

ffl dev synch() - poll network to check if any events have arrived and trigger

callback

long dev.synch (struct timeval *timeout, long *error) struct timeval *timeout - pointer to maximum time to wait while polling long *error - pointer to error code (if any)

14.3.2 Server side

ffl dev event fire() - a server call to diispatch a user event to all clients which

have registered their interest in that event with this server

- C using Objects in C :

long dev.event.fire (DevServer ds, long event.type,

DevArgument argout,DevType argout.type, long event.status, long event.error)

14.4. IMPLEMENTATION 185

long event.type - event type to dispatch DevArgument argout - pointer to argout to dispatch with event DevType argout.type - argout type long event.status - status of event to dispatch to client long event.error - error code of event to dispatch to client (if status != DS.OK)

- C++ using the Device class :

long dev.event.fire (Device *device, long event.type,

DevArgument argout,DevType argout.type, long event.status, long event.error)

long event.type - event type to dispatch DevArgument argout - pointer to argout to dispatch with event DevType argout.type - argout type long event.status - status of event to dispatch to client long event.error - error code of event to dispatch to client (if status != DS.OK)

/**

\page events

\section Implementation

User events have been implemented in TACO DSAPI v7.0. They have been tested on

Linux/x86, Linux/m68k, HP-UX and Solaris 2.5. They should work in principle on

OS-9 but because of its flaky TCP/IP stack implementation programmers are urged

to port their device servers to one of the Unix derivatives e.g. Linux, where

they will not be plagued by sockets closing when they shouldn't or not closing

when they should ! No port has been undertaken so far for Windows or VxWorks.

\section Timeouts

Events depend on detecting the server or client going down in order to work

correctly. This is treated as a timeout in the client. If the client does not

receive any events during a period exceeding the asynchronous timeout value

(set using \c dev_asynch_timeout()) it will ping the server to see if it is

still alive. If not it will trigger the event callback with status = DS_NOTOK

and error = DevErr_RPCTimedout. The event will be unregistered on the client

side. If the server detects a client is not there anymore it will silently

remove it from the list of registered clients.

\section Examples

How best to generate events in a device server ? The most obvious way is to create

an event thread whose job it is to poll a variable (state or value) to detect the

event. Once the event is detected the event thread calls \c dev_event_fire() to

dispatch the event. Here is a simple example to generate a periodic event using

Posix threads :

\code

void *events_thread(void * arg)

{

long event = 1;

long counter=0;

struct timespec t100ms;

fprintf(stderr, "\nfire_events(): starting thread %s\n", (char *)arg);

for (;;)

{

dev_event_fire(ds, event,&counter,D_LONG_TYPE,DS_OK,0);

counter++;

/*

* sleep for 90 ms

*/

t100ms.tv.sec = 0;

t100ms.tv.nsec = 90000000;

nanosleep(&t100ms, NULL);

}

return NULL;

}

int event_thread_start()

{

int retcode;

pthread_t th_a, th_b;

void * retval;

#if defined(linux) || defined(solaris)

retcode = pthread_create(&th_a, NULL, fire_events, "a");

#else

retcode = pthread_create(&th_a, pthread_attr_default,

(pthread_startroutine_t)fire_events,

(pthread_addr_t)"a");

#endif /* linux || solaris */

if (retcode != 0)

fprintf(stderr, "create a failed %d\n", retcode);

\endcode

The function \c event_thread_start() has to be called at an appropiate point in

the device server e.g. during class_initialise() or object_create().

\section Performance

The performance of events depends naturally on what type of system the device

server is running on. Tests have been caried out on Linux/x86, Linux/68k, HP-UX

and Solaris running on Pentiums, 68030s, s700s and SPARC CPUs. They all showed

similar performance with variations due to the scheduler. Firing of events uses

the one-way ONC/RPC mechanism which means it is immediately copied to the system

buffer without waiting. This means there is very little overhead introduced in

the device server. Generating events at maximum speed shows that the minimum

time between events is about 25 microseconds with an average of 500 microseconds

over a long (seconds) time scale. This is due to scheduler stopping the device

server at regular intervals (presumably to dispatch the events). Using the example

code above a number of tests were done on different platforms. The results were

all roughly the same i.e. the server could generate events at regular time

intervals of 100 millseconds wih a jitter of less than 10 microseconds. The

jitter goes up as a function of the number of clients e.g. jitter of 25 microseconds

for 10 clients on Linux/m68k. Here is an example output log from a client

(Linux/x86 + Pentium) which accepts the events from a device server running on a

tacobox (Linux/x86 + Pentium) and prints out their times :

\verbatim

counter = 3362 , server time = {924772119 s,342170 us} delta time = 99974 us

counter = 3363 , server time = {924772119 s,442169 us} delta time = 99999 us

counter = 3364 , server time = {924772119 s,542169 us} delta time = 100000 us

counter = 3365 , server time = {924772119 s,642169 us} delta time = 100000 us

counter = 3366 , server time = {924772119 s,742169 us} delta time = 100000 us

counter = 3367 , server time = {924772119 s,842169 us} delta time = 100000 us

counter = 3368 , server time = {924772119 s,942169 us} delta time = 100000 us

counter = 3369 , server time = {924772120 s,042173 us} delta time = 100004 us

counter = 3370 , server time = {924772120 s,142169 us} delta time = 99996 us

counter = 3371 , server time = {924772120 s,242169 us} delta time = 100000 us

counter = 3372 , server time = {924772120 s,342169 us} delta time = 100000 us

counter = 3373 , server time = {924772120 s,442169 us} delta time = 100000 us

counter = 3374 , server time = {924772120 s,542169 us} delta time = 100000 us

\endverbatim

\section event_problems Known problems

Known problems so far are that when the server or client die then HP-UX and

Solaris servers and clients have difficult to detect this due to the way sockets

are handled. The next release will fix this by implementing an event heartbeat

which will reactivate the event channel. Failure to do so will result in the

event timing out and the client being removed from the list of registered clients

in the server.

*/