/*! \page whatis What is TACO ?

\addindex whatis

TACO is an object oriented control system originally developed at the European Synchroton Radiation

Facility. The beasic idea behind TACO is to treat every control element as an object on which commands

can executed. The objects are called devices and they are available network wide. Devices are created and

stored in device servers. The command which can be executed on a device are implemented in the device

class. Device classes can be written in C(using a methodology called OIC) or C++. The commands are

accessed via a small set of C calls referred to as the application programmer's interface(DSAPI).

\section History

Device servers were first developed at the European Synchrotron Radiation Facility

(ESRF) for controlling the 6 GeV synchrotron radiation source. This document is a

Programmer's Manual on how to write device servers. It will not go into the details

of the ESRF, nor its Control System nor any of the specific device servers in the

Control System. Various papers describe these topics already. Readers are referred

to Laclare (1983) for a description of the ESRF, to G&ouml;tz et al (1991) for the

Control System and the Device

Server User Guides (or DSUGs) for specific device servers. The role of this document

is to help programmers faced with the task of writing device servers. Device servers

have been developed at the ESRF in order to solve the main task of the Control System

viz. provide read and write access to all devices in a distributed system. The

problem of distributed device access is only part of the problem however. The other

part of the problem is providing a programming framework for a large number of devices

programmed by a large number of programmers each having different levels of experience

and style. Device servers have been written at the ESRF for a large variety of

different devices. Devices vary from serial line devices to devices interfaced by

field-bus to memory mapped VME cards to entire VME/VXI data acquisition systems.

The definition of a device depends very much on the user's requirements. In the

simple case a device server can be used to hide the serial line protocol required

to communicate with a device. For more complicated devices the device server can

be used to hide the entire complexity of the device timing, configuration and

acquisition cycle behind a set of high level commands. A model (referred to as

the Device Server Model or DSM) has been developed to satisfy the main two

requirements. In order to do this the DSM has a number of parts to it. It defines

the concept of a generic device which is created and managed in a server - a device

server. The device is accessed by an application programmers interface (api) which

is network transparent. Device specific details get treated in the device servers

thereby freeing applications to do applicationoriented work. Multiple access is

implemented by queuing requests - the queuing is handled automatically by the

network software.

The concept of using servers to access devices was first proposed by W.-D.Klotz

in 1989. To demonstrate this a simulation of a powersupply was implemented which

ran as a server. Clients wanting to use the powersupply connected to the server

which then forked a copy of the server for the client. Unique data was stored in

shared memory. This first version was based on Berkeley sockets and suffered from

the drawback that no machine independent data format was used and that for large

numbers of clients the number of forked process soon became a limiting factor.

The original version was released to application programmers and served a useful

purpose. A.G&ouml;tz (the author) took over the original server in the late Spring

of 1990. The first goals were to replace the Berkeley sockets with the CERN NC/RPC

interface, to write servers for real devices, and to setup a team of programmers

who would write the servers. This was just the time that X11 and MIT Widgets

started appearing on commercial platforms.  The Widget model (implemented by MIT's

Intrinsics Toolkit) struck the author as being very appealing. It is easy to use,

very powerful and manages to hide the complexity of the implementation from the

user. It also demonstrated how Classes and Objects can be implemented in C. Armed

thus with the original powersupply api and the Widget model from MIT work begun

(mid-1990) in earnest on the device server concept. Assistance was provided by

R.Wilcke (who ported the CERN NC/RPC software to OS9) and H.Witsch (who acted as

the first guinea-pig device server programmer). The first device server implemented

the same functionality as the WDKPowerSupply. The server ran on OS9 and the client

on HPUX. Today (almost three years later) more than 500 device servers exist for

the ESRF's Machine and Beamline Control Systems and for Data Acquisition Systems.

They run on a range of Operating Systems i.e. OS9, HPUX, Linux, Windows, and SunOS.

There are approximately 16 programmers involved in writing device servers.

The CERN NC/RPC has been replaced by the SUN NFS/RPC thanks to J.Meyer. A resource

database has been added which is accessible via a standard set of rpc calls

developed by E.Taurel. Device servers are implemented using classes and clients

access devices via a standardised api. If the powersupply server process is

considered as the first prototype and the NC/RPC based device servers as the first

generation, then it would be true to say that device servers are now well into

their second generation. The term "device server" first appeared in an internal

ESRF document by W.-D.Klotz and S.M.Keogh in June 1989. It reappeared in a paper

written for a GULAP (Group for Upper Level Applications Programming) Meeting in

January 1990 and has been a common word in the ESRF daily vocabulary ever since.

\section system_arch System architecture

TACO is based on a client-server model. All devices are created and served by device servers.

Clients access devices via a network transparent application programmer's interface (DSAPI).

In addition to device servers there are so-called system serversthe manager and database which

provide system services. There is no a-priori limit to the number of device servers and clients.

This makes TACO very scalable.

\image html tacoarch.gif "TACO system architecture "

\image latex tacoarch.eps "TACO system architecture " width=14cm

\subsection manger Manager

The manager is the only fixed point in the whole TACO control system. It is used as a single

entry point to start and stop the control system. All clients (including device servers) of

the control system connect to the Manager before anything else.

\subsection database Database

TACO supports a simple database called the resource database where all configuration parameters

for devices are stored. The database is served by a database server. All values are stored as

ascii strings which are then converted to the correct types at runtime in the calling process

(device server). All C simple types and array of simple types are supported. The GNU ndbm

database available under Unix and Windows is used as database.

\section dsm Device Server Model

One of the most fundamental aspects of TACO can be found in the implementation of device access

in the device servers. All device control (input/output) is implemented in the device servers.

Device servers are implemented according to a model known as the device server model (DSM). In

the DSM all devices (physical and logical) are treated as objects. Each object belongs to a device

class. The class implements the actions necessary for each device type. The actions (referred to

as commands) can be executed locally or via the network. The device class implements methods and

actions. The actions can be considered as special methods which can be executed by local and remote

clients. They have a fixed number of input and output parameters where the parameters can be simple

or complex (self-defined) types.

\image html tacodsm.gif "TACO Device Server Model for a typical PowerSupply "

\image latex tacodsm.eps "TACO Device Server Model for a typical PowerSupply " width=14cm

\section api Application Programmer's Interface

The device server model is used for implementing device access in TACO. Users of the control system

on the other hand have a "black box" view of the control system. They access the control system

either via a high-level programming language (C, C++, Tcl, Spec) using the device server Application

Programmer's Interface (DSAPI) or using one of the graphical applications which have been written.

The DSAPI consists the following basic calls :

-# dev_import()- import or build up a connection to a device

-# dev_putget() - execute a command on a device

-# dev_putget_async() - execute a command n a device asynchronously

-# dev_free() - free the device

In addition to these calls there are a number of calls for modifying the network communication

parameters, interrogating the state of an asynchronous command execution and for managing device

security. All network calls to and from the device server are implemented using the Sun Open Network

Computing / Remote Procedure Call (ONC/RPC). The ONC/RPC is available on all platforms where the

Network File System (NFS) is implemented. The ONC/RPC uses the eXternal Data Representation (XDR)

format to encode data sent on the network.

\section multi Multiple Control Systems

TACO supports the concept of multiple control systems. Each control system has its own database and

device servers. Clients and servers of different control systems can communicate with each other as

if they were part of the same control system. To specify a device in a diiferent control system a

device must be specified with its full name :

\verbatim

//nethost/domain/family/member

\endverbatim

Where nethost is the name of the host where the database of the second control system is running.

This concept is referred to as multi-nethost in the documentation.

\section uses Uses of TACO

TACO is a toolkit for building distributed objects. Any application which can profit from encapsulating

functionality into objects and distributing them over more than one host on the network can find a use

for TACO. Control systems are one very good example of this and TACO was developed mainly for doing

distributed control. All control systems need to control hardware. The hardware can be in a the same

computer or more often than not in a variety of computers and black-boxes. It is the job of the control

system to coordinate the different hardware. Examples of hardware are stepper motors, cameras,

powersupplies, detectors, adc's, dac's but could even be coffee machines or light switches in the case

of home automation. TACO is ideal for encapsulating hardware functionality in a device server and

exporting it on the network e.g. for embedded controllers. These are called tacoboxes amongst TACO

users. GNU/Linux is an ideal candidate as underlying operating system. TACO can also be used to

distribute pure logic where no hardware is involved e.g. for doing image processing, or for sharing

data between applications. TACO has been used in the research environment (synchrotron radiation

sources, reactors and telescopes) but is also being used to control robots and soon in the home to

automate light switches, heaters, messaging systems etc.

\section interfaces Interfaces to TACO

TACO has been interfaced to a number of other languages and programs. The main

interfaces are C and C++ and are described in a separate chapter. In addition

to these two languages the following languages/programs have been interfaced to

TACO :

- Python - contact Jens Meyer (meyer@esrf.fr) or Marie-Christine Dominguez

(domingue@esrf.fr)

- Tcl - contact Gilbert Pepellin (pepellin@esrf.fr)

- MathLab - contact Laurent Farvacque (laurent@esrf.fr) or Francis Epaud

(epaud@esrf.fr)

- LabView - contact Andy G&ouml;tz (goetz@esrf.fr)

- SPEC - contact Gerry Swislow (info@certif.com) i

For more information refer to the website or contact the person involved directly.

\section platforms Supported Platforms

\addindex platforms

TACO is actively supported and used on the following platforms :

<ul>

<li>Linux/x86 - following distributions have been tested

<ul>

<li>SuSE</li>

<li>Mandrakei</li>

<li>RedHat</li>

<li>Debian</li>

<li>Ubuntu</li>

</ul>

but there is no reason why TACO shouldn't compile and run on any Linux distribution.

</li>

<li>Linux/68k - using the Debian distribution on MVME-162's and MVME167's Motorola's

<li>Solaris - versions 2.5 and 2.7 are supported using the native Solaris compilers

and GNU gcc compilers</li>

<li>FreeBSD</li>

<li>Mac OS X</li>

<li>HP-UX - version 9.x and 10.20</li>

<li>OS9 - version V3.03 on VME</li>

<li>Windows

<ul>

<li>95/98</li>

<li>NT, 2k, XP</li>

</ul>

</li>

</ul>

The following platforms have been ported to in the past but are not used anymore

and are therefore not uptodate :

- VxWorks - version 5.x

- LynxOS - version ?

- Irix - version 6.5 The latter platforms could be updated if need arises.

*/

*/