/*! \page dsapi DSAPI

The DSAPI is the TACO Device Server Application Programmer's Interface for C and

C++ programs. It is used by clients and servers to import, execute commands on,

free and explore TACO devices. It uses the ONC-RPC (SUN remote procedure call)

as underlying communication protocol. This document describes the latest version

of the DSAPI V6.0. This document is split into the following sections :

- "Getting Started" describes how to write a simple client which uses DSAPI, should

be read by beginner's who want to get a quick start;

- "Changes" describes what are the main changes in the different major releases;

Beginners should read "Getting Started" first, other programmers should read what

changes have taken place in the latest version and use the reference guide.

\section what_s_dsapi What is DSAPI ?

DSAPI is a C (and C++) programmer's interface for accessing device in a TACO

control system. Devices in a TACO control system are network objects created

and served by processes called device servers.

A device is identified by its ASCII name :

\verbatim

[//facility/]domain/family/member

\endverbatim

Each device understands a set of commands. The commands enable a remote client

to execute actions on a device e.g. for a powersupply switch it on or off, read

the state, read the current. The DSAPI gives remote and local clients access to

device commands. Using DSAPI it is possible to execute any command on any device

(assuming the client has the necessary permission) in a TACO control system. Data

is passed from the client to the device via the input and output parameters of

the DSAPI. Devices are organised into classes. Each class implements and understands

a fixed set of commands. The list of commands for a device class is documented

in the Device Server User's Guide (DSUG). The set of C functions which implement

the DSAPI are archived in static or shared libraries for all platforms supported.

\section getting_started Getting Started

This section will take you through the steps of writing a simple application using

DSAPI. Two versions of a simple "Hello World" in C for sending a string to a

"hello world" device will be presented The first version demonstrates using the

DSAPI to execute commands synchronously while the second version demonstrates

asynchronous command execution. The user will be taken through the stages of

compiling, linking, debugging and running. The section will terminate with tips on

common pitfalls encountered by DSAPI beginner's (and even old-timer's sometimes).

\subsection hello_worl_sync "Hello World" (synchronous) example

This example will take you through the steps of writing a simple program to send

a "Hello World" string to a device synchronously.

Step 1

The first step is to find out which commands the device understands. If you don't

know them off by heart then get hold of the user guide (DSUG) for that device class

and read it. The DSUG will list all commands implemented for the device and their

input and output arguments. The command we will use in this example is DevHello.

Step 2

The next step is to write the program. This assumes we know what we have to control

and how. In the case of this example we want a program which sends a string to a

device and reads one back. The program is written in C and uses a simple ascii

interface to interact with the user. The program listing can be found below (cf.

section "Code Example"). ALL device access is done using DSAPI (of course). The main

statements to note are :

- \verbatim #include <API.h>\endverbatim - include file required by all clients (and servers). Necessary

to prototype all DSAPI functions, and to define symbols and types. API.h will

include other include as necessary.

- devserver hs - variable which will contain the device handle. Every device has

to have a device handle. It is passed as input parameter to every DSAPI call. It

contains all information necessary to communicate with the device on the network

(network address, protocol, security etc.) as well as various bookkeeping

information (device name). The device handle is initialised on the first successful

call to \c dev_import() (cf. below).

- dev_import() - initialise the device handle. This call takes as input the device

name and permission level requested. It checks the database to see if the device

is defined and if so it asks the database for the device's network address. Then

it tries to contact the device server. All this information is stored in the device

handle and returned to the user. If the device is not defined in the database or

the user does not have the necessary permission to use the device \c dev_import()

will return an error and the device handle will be NULL. The import is stateless

this means the routine will not fail if the device server is not running. The 2nd

parameter is used for security (this is discussed in the C library reference).

- dev_putget() - execute a command on the device. This call is the workhorse of

DSAPI. It is used to execute a command on a device synchronously i.e. the client

sends her request to the device and then waits for the command to be executed and

for the answer to be returned before continuing. For the asynchronous version see

below. The client has to specify the input and output arguments and their types.

This information is normally obtained from the DSUG but can be constructed dynamically

(using \ref dev_cmd_query). All parameters are passed as pointers. If the output

arguments contain any pointers in them the client can choose to allocate space for

the result himself or let DSAPI allocate space. (by setting the pointer to NULL)

In the latter case it is up to the client to free the space allocated by DSAPI.

The question of when to allocate and when to free is a tricky one and is treated

in more detail in the section \ref common_pitfalls "Common Pitfalls".

- dev_free() - free the device handle. This call will try to inform device server

that the client is not connected to this device anymore. If this is the client's

last network connection to the device server it will free the socket connection

to the device server. Finally it will free the device handle structure allocated

by \ref dev_import().

Step 3

The next step is to compile and link the client. This is different depending

whether you are using a Unix-like (HP-UX, Solaris, Linux, VxWorks), OS9 or a

Windows-NT system.

Unix and OS9

To compile under Unix and OS9 you have to tell the compiler where to find the

DSAPI include files and which libraries to link with. Assuming the your program

is called helloworld, DSHOME is an environment variable which points to the root

directory of your TACO installation and OS the operating system type (s700 for

HP-UX 9.x, hpux10.2 for HP-UX 10.2, solaris for Solaris, linux for Linux,

vxworks for VxWorks, os9 for OS9) then simply type :

\verbatim

$CC $CFLAGS -I$DSHOME/include -L$DSHOME/lib/$OS -ldsapi -ldbapi -ldsxdr helloworld.c -o helloworld.

\endverbatim

CC and CFLAGS have to be positioned for each platform (refer to the example

Makefile).

Windows-NT

To compile under Visual C++ 4.2 you need to set the following options using the

graphical interface :

to be filled in ...

Step 4

The final step is to run your program. Make sure you are in a shell interpreter

(e.g. bash, ksh, tcsh, csh for Unix and MSDOS for Windows-NT) and simple type

the name of the client program plus the name of the device i.e.

\verbatim

helloworld exp/hello/world.

\endverbatim

If you forget to provide a device name the program will prompt you for one.

Example code - helloworld.c

\include helloworld.c

\subsection hello_world_async "Hello World" (asynchronous) example

This example is a repeat of the above but using the asynchronous version of DSAPI.

Asynchronism in this case means the client requests a command to be executed but

does not wait for the server to respond. Instead it continues on to the next

statement immediately. The request is put into the server's buffer of incoming

requests. After the server has executed the command it returns an acknowledge

plus any output arguments to the client asynchronously. The reply is buffered in

the clients queue of incoming replies. When the client is ready it polls its

input queue to see if there are any replies pending (using the dev synch() call).

Asynchronous command execution is more difficult to program than synchronous.

However it is more efficient and is particularly useful for windowing programs

and for programs which want to start multiple commands on multiple devices

executing simultaneously and don't want to wait for the command to finish execution.

This example is identical to the above example excepting for the fact that DevHello

command is executed asynchronously. A callback function specified. This makes

the code longer and more slightly more complicated to read.

Step 1

Understanding the device - same as Step 1 above.

Step 2

Writing the program - in principal same as Step 2 above however this time round

use the asynchronous version of DSAPI. The new calls are :

- callbacks - functions to be called when client receives a reply. Every reply

received by the client has to be signalled to the client and unpacked. The

callback functions serve this purpose. One callbacks functions has been implemented

for this example - \c hello_callback(). The client can pass its own data with every

asynchronous call which can be used to identify each reply during the callback

(user data parameter).

- dev_putget_asyn() - execute a command asynchronously on a device. As explained

above the client does not wait for the server to accept the request for the reply.

The input arguments are the same as for \c dev_putget() (synchronous) plus three

additional arguments. The additional arguments specify the callback function (to

be triggered during a call to \c dev_synch()), a pointer to user data and an

asynchronous id (returned by dev_putget_asyn()).

- dev_synch() - check to see if any asynchronous replies have been received. If

so they are unpacked and the corresponding callback is triggered. dev_synch()

takes as input the amount of time it should wait for pending replies before

continuing.

Step 3

Compiling and linking - same as Step 3 above for Unix and OS9. The asynchronous

calls are part of the standard library. Not support under Windows-NT (yet).

Step 4

Running - same as Step 4 above for Unix and OS9. Not supported under Windows-NT

(yet).

Example code - helloworld_asyn.c

\include helloworld_async.c

\subsection common_pitfalls Common Pitfalls

Using an API is easy once you know how. For beginner's this is not the case. This

section will list the common pitfalls encountered by beginner's (and old-timers

too!) when they start using DSAPI.

\subsection Nethost

Every TACO control system is managed by a NETHOST. The NETHOST is the name of

the host where the TACO Manager has been started. It is referred to as the

facility in the device name. The Manager is the entry point for all TACO

clients and servers. A common error when starting an application (e.g. helloworld)

is to forget to specify the NETHOST environment variable. In this case you will

get an error similar to this :

\verbatim

Thu Nov 6 13:56:42 1997 environmental variable NETHOST not defined

\endverbatim

The solution is to set the environment variable to the name of a host where a

TACO control system Manager is running e.g. "setenv NETHOST libra" for csh or

"export NETHOST=libra" for ksh or bash. An alternative to specifying the

NETHOST environment variable is to qualify the device name with the facility

field which is the same as the NETHOST e.g. //libra/exp/hello/world.

If the Manager is not running you will get the following error :

\verbatim

Thu Nov 6 14:03:26 1997 no network manager available

\endverbatim

If you don't know which host is your NETHOST then ask your TCO system

administrator/guru. If you are supposed to be the guru then start the Manager.

If you don't know how then send an email to the TACO help-line taco@esrf.fr

\subsubsection shared_libraries Shared Libraries

Another common error is not finding the DSAPI shared libraries. If your application

dies with the following message :

\verbatim

./helloworld: can't load library 'libtaco.so'

\endverbatim

You must add the DSAPI library directory for your platform to the shared library

path searched by your system.

For Solaris and Linux use :

setenv LD_LIBRARY_$PATH:$DSHOME/lib/$OS for csh and tcsh,

export LD_LIBRARY_PATH=$LD_LIBRARY_PATH:$DSHOME/lib/$OS for ksh and bash.

For HP-UX use :

setenv SHLIB_PATH $DSHOME/lib/$OS for csh and tcsh,

export SHLIB_PATH=$SHLIB_PATH:$DSHOME/lib/$OS for ksh and bash.

Where DSHOME is an environment variable pointing to the TACO home directory and

OS the operating system flavour. Shared libraries are not supported on OS9 and

Windows/NT (yet).

\subsection Makefiles

Although the compile+link instructions listed above can be typed every time you want to recompile+relink it is much more efficient to write a makefile with the necessary instruct

ions. The TACO makefiles are multi-platform and make use of the conditional statements supported by GNU make (also known as gmake). gmake supports statements of the kind ifdef $(symbol), else and

endif. Most TACO conditional makefiles use the same symbols. These are :

- __hp9000s700 - for HPPA 1.0 systems running HP-UX 9.x

- __hpux10 - for HPPA 1.0 systems running HP-UX 10.2

- _solaris - for Solaris

- linux - for Linux

- vw68k - for Motorola 68k systems running VxWorks

- vwx86 - for Intel 68k systems running VxWorks

- _UCC - for OS9 system using the Ultra C and C++ Compiler

- unix - for HP-UX, Solaris, Linux and VxWorks platforms

A simple example Makefile for the helloworld program could look like this :

\verbatim

#

#

# Makefile for helloworld - a simple DSAPI client

#

#

# TACO home directory

#

DSHOME = $(LOCAL.DSHOME)

#

# library home direcory - platform dependant

#

ifdef __hpux10

LIBHOME = $(DSHOME)/lib/hpux10.2

endif # __hpux10

ifdef _solaris

LIBHOME = $(DSHOME)/lib/solaris

endif # _solaris

ifdef linux

LIBHOME = $(DSHOME)/lib/linux

endif # linux

ifdef _UCC

LIBHOME = $(DSHOME)/lib/os9

endif # _UCC

ifdef vw68k

LIBHOME = $(DSHOME)/lib/vw68k

endif # vw68k

ifdef vwx86

LIBHOME = $(DSHOME)/lib/vwx86

endif # vwx86

#

# include files home directory

#

INCLDIRS = -I$(DSHOME)/include \

-I$(DSHOME)/include/private

#

# compiler flags - platform dependant

#

ifdef __hpux10

CC = /bin/cc

CFLAGS = -Aa -g -DEBUG -Dunix -D.HPUX.SOURCE -D..hpux10 -DBSD=199704 \

-c $(INCLDIRS)

endif # __hpux10

ifdef _solaris

CC = /opt/SUNWspro/SC4.0/bin/cc

CFLAGS = -Xa -g -Dsolaris -DEBUG -c $(INCLDIRS)

endif # _solaris

ifdef linux

CC = gcc

CFLAGS = $(INCLDIRS) -Dlinux -Duniy -ansi -DEBUG -g -c

endif # linux

ifdef _UCC

CC = xcc CFLAGS = -mode=c89 -g -D EBUG -to osk -tp 020 -x il -e as=. $(INCLDIRS)

endif # _UCC

ifdef vw68k

CC = cc68k

CFLAGS = -Dvxworks -Dunix -DCPU=MC68020 -ansi -m68030 \

-msoft-float -DEBUG -e $(INCLDIRS) -g

endif # vw68k

ifdef vwx86

CC = cc386

CFLAGS = -v -c -Dvxworks -Dunix -DCPU=I80386 -ansi \

-DEBUG $(INCLDIRS) -g

endif # vwx86

#

# library flags

#

ifdef __hpux10

LFLAGS = -L$(LIBHOME) -ldsapi -ldsxdr -ldbapi -lm

endif # __hpux10

ifdef _solaris

LFLAGS = -L$(LIBHOME) -ldsapi -ldsxdr -ldbapi -lnsl -lsocket -lm

endif # _solaris

ifdef linux

LFLAGS = -L$(LIBHOME) -ldsapi -ldsxdr -ldbapi -lm

endif # linux

ifdef _UCC

LFLAGS = -L$(LIBHOME) -l dsapi -l dsxdr -l dbapi -l rpclib -l netdb_small \

-l socklib.l -l sys_clib.l -l unix.l

endif # _UCC

#

#------------------------main-target-to-make------------------------------

#

all : helloworld

helloworld : helloworld.c

$(CC) $(CFLAGS) helloworld.c -o helloworld $(LFLAGS)

\endverbatim

\note don't forget to start all rules with a tabulation mark ! Although even this simple example looks complicated keeping all platform dependancies in one file can prove to be a time saver when developing on multiple platforms.

\subsubsection memory_allocation Memory Allocation

Probably the trickiest part for beginners to DSAPI is memory allocation. DSAPI

uses the memory allocation of the XDR library of the ONC-RPC. The difficulties

come from the fact that all procedure calls are to remote servers and pointers

to memory areas have to be copied to the (remote) server and vice versa. The

rules for memory allocation in DSAPI can be summarised as follows :

-# arguments are either outgoing (input) or incoming (output) from the client

to the server,

-# all input and output arguments are passed via pointers,

-# memory for input arguments have to allocated by the client (of course !),

-# memory for pointers in output arguments can be allocated either by the client

or by the DSAPI (actually the XDR layer),

-# if memory in output arguments is to be allocated by DSAPI then initialise

pointers in output arguments to NULL,

-# if pointers to memory in output arguments are NOT initialised to NULL DSAPI

assumes the client has allocated the necessary memory and will try to use it

(with catastrophic consequences if this is not the case !),

-# any memory allocated by DSAPI has to be freed by the client using

\c dev_xdrfree().

-# in order to avoid nasty bugs or strange core dumps therefore clients MUST

initialise all incoming pointers to NULL or to locally allocated memory.

If you understand the above rules and follow them you should not have any

problems. The problems come from not understanding and following these rules.

The XDR types supported by DSAPI are covered in the section on "XDR Types".

To illustrate the above rules here are some examples :

- simple C types

\verbatim

devserver ps;

long status, error;

float readvalue;

.

.

.

status = dev_putget(ps, DevReadCurrent, NULL, D_VOID_TYPE,

&readvalue, D_FLOAT_TYPE, &error);

printf("current %6.3f\n",readvalue);

.

.

.

\endverbatim

This is a simple example of using a simple C type to receive output from the

server. Simply pass the pointer to the simple type to DSAPI.

\note DSAPI cannot allocated memory for simple types because it expects a

pointer to a value and not a pointer to a pointer to a value and it therefore

has no way of distinguishing between a pointer to the value ZERO and a pointer

to NULL (if you know what I mean ...)

- output arguments - memory allocated by client

\verbatim

devserver ps;

long status, error, i;

float readvalues[MAX_READVALUES];

DevFloatVarArray float_vararr;

.

.

.

floatr_.vararr.length = MAXr_.READVALUES;

float_vararr.sequence = readvalues;

status = dev_putget(ps, DevReadAll, NULL, D_VOID_TYPE,

&float.vararr, D.VAR.FLOATARR, &error);

printf("read %d value\n\n",float.vararr.sequence);

for (i=0; i!float_vararr.sequence; i++)

{

printf(" current[%d] %6.3f\n", i, readvalues[i]);

}

.

.

.

\endverbatim

In this example the client receives a variable length array of floats. The client

has allocated memory for the array of floats itself. It is the responsibility of

the client to ensure that sufficient memory is allocated for the return argments

and that the server does not send more values than the client expects.

- output arguments - memory allocated by DSAPI

\verbatim

devserver ps;

long status, error, i;

DevFloatVarArray float.vararr;

.

.

.

float.vararr.length = 0;

float.vararr.sequence = NULL;

status = dev.putget(ps, DevReadAll, NULL, D.VOID.TYPE,

&float.vararr, D.VAR.FLOATARR, &error);

printf("read %d value"n"n",float.vararr.sequence);

for (i=0; i!float.vararr.sequence; i++)

{

printf(" current[%d] %6.3f"n", i, float.vararr.sequence[i]);

}

dev.xdrfree(D.VAR.FLOATARR, &float.vararr, &error);

.

.

.

\endverbatim

In this example the client sets the sequence to NULL and lets DSAPI allocate memory

for the output arguments. The client has to free the allocated memory.

\subsection advanced_features Advanced Features

Before leaving the "Getting Started" section we would like to mention some advanced

features of the DSAPI which are very useful.

\subsection Timeouts

The DSAPI is managed by timeouts. Both synchronous and asynchronous calls have a

timeout. A client will receive a timeout error (DevErr_RPCTimedOut) if the server

has not sent an answer within the timeout period. The default timeout for

synchronous calls is 3 seconds. The default timeout for asynchronous calls is 25

seconds. The client can modify the timeout per device using the dev_rpctimeout()

call (cf. the C library reference). This can be necessary if the request is know

to take longer than the default timeout to execute. If a client gets lots of

timeouts there could be a network problem i.e. lots of network traffic. This can

fixed by simply changing from UDP to TCP protocol (see next section).

\subsection Protocol

The DSAPI is based on the ONC-RPC and makes use of UDP and TCP (the two main IP

protocols). The difference between the two protocols is :

- UDP is a connectionless unreliable protocol. UDP has the advantage that it

does not require a dedicated file descriptor per client-server connection and it

is (sometimes) faster than TCP. It has the disadvantage that it does not retry

if a request fails and it is limited in maximum packet size to 8 kilobytes. All

device imports are done using UDP. UDP is the default protocol

- TCP is a connection-oriented reliable protocol. It has the advantage that it

is reliable i.e. it will retry if a request fails to be acknowledge, and can

transfer unlimited packet sizes (in reality limited by the receiving computer to

a few megabytes). It has the disadvantage that it requires a file descriptor per

client-server connection and it is a more complicated protocol to implement.

To change from UDP to TCP or vice-versa use the dev rpc protocol() call (cf. below).

\section Changes

\subsection part1 Version 8.0

Version 8 introduces support for TANGO. TANGO (http://www.esrf.fr/tango) is the new Version of TACO based

on CORBA (instead of RPC) and with support for C++ and Java. The TANGO interface

allows TACO clients to do a dev putget() call on a TANGO device in a transparent

manner - simply add "tango:" in front of the device name to switch protocol from

RPC to CORBA. To use the TACO-TANGO interface link your C or C++ program with the

C++ linker and the libtaco++ library.

\subsection part2 Version 7.0

Version 7 introduces events. Events use the same mechanism as the asynchronous

call for dispatching. They allow servers to be programmed to generate true

asynchronous events to clients.

\subsection part3 Version 6.0

The main changes in the new version are the inclusion of true asynchronous

dev putget() calls - dev putget asyn() and related calls (cf.

"C library reference" above).

\subsection part4 Version 5.1

The main changes to this version were the following - support for multi-nethost,

ports to Windows (95 and NT), Linux and VxWorks.

\subsection part5 Version 4.1

The main changes here were security was implemented, and port to Ultra C for OS9.

\subsection part6 Version 3.37

\subsubsection An Asynchronous dev_put()

The new function dev_put_asyn() is similar to the ancient

dev_put(). The only difference is, that dev_put_asyn() sends a request to execute

a command to a device server and returns immediately when the command was received.

The only errors which can be returned by dev_put_asyn() are errors during the

sending of the command. A correct return status only indicates that the command

execution was started. No failures during command execution can be reported back

to the client.

\code

long dev_put.asyn (ds, cmd, argin, argin.type, error)

devserver	ds;		/// client handle to the device

long		cmd;		/// command to execute

DevArgument	argin;		/// pointer to input arguments

DevType		argin_type	/// input argument data type

long		*error; 	/// error

\endcode

\subsubsection Destroying Objects

With the function ds_destroy() a proper interface was created to destroy objects

in a device server. Ds_destroy() searches for a destroy method (DevMethodDestroy)

in the object class. If no destroy method is implemented in the object class, its

superclasses are searched. Arriving at the end of the class tree, the destroy method

of the general device server class will be executed. The general destroy method

will free the object correctly only, if no memory allocation was done for object

fields outside the DevServerPart structure of the object. The device name, as a

field of DevServerPart will be freed correctly bye the general device server

class destroy method. Also exported objects can be destroyed. They will be

deleted from the list of exported devices and all client accesses will be stopped.

\code

long ds__destroy (ds, error)

DevServer	ds; 		/// pointer to the object

lond		*error 		/// error

\endcode

Attention:

To destroy an exported object, ds_destroy() must be used. Executing only the

destroy method will not delete the device from the list of exported devices. With

the next client access a nice core will be generated.

\subsubsection Accessing Process Internal Devices

Until version 3.37 the only possibility to access devices internally was the

function dev_cmd(). That was not enough to handle the coming security features.

Out of this reason the functionality of the functions:

\code

dev_import()

dev_putget()

dev_put()

dev_free()

\endcode

was enlarged. They can be used now on all exported devices, remote via RPCs or

internally just via function calls. Dev_import() will detect automatically whether

a device is internal and will avoid all overhang of the remote access on the

client handle. Also memory treatment was unified. All outgoing arguments (remote

or intern) are allocated by XDR. Dev_xdrfree() must be used to free the memory.

Attention:

This unified interface for device access works on all exported devices. Objects

which are not exported, can be accessed only be dev_cmd(). To access process

internal devices the unified interface must be used to avoid access and security

problems in the coming releases.

\subsubsection Dynamic Memory Allocation

The general structures handling exported devices and client connections to the

devices

\code

typedef struct _DevServerSec {

long	 security_key;

long  	 access_right;

long	 single_user_flag;

} DevServerSec;

typedef struct _DevServerAccess {

DevServer	 ds;

char		 export_name[80];

long 		 export_status;

long		 export_counter;

long 		 single_user_flag;

long		 max_no_of_clients;

DevServerSec	 *client.access;

} DevServerDevices;

DevServerDevices 	*devices	///Exported devices; in DevServer.c

\endcode

are no longer static arrays. The are allocated dynamically in data blocks. The

BLOCK SIZE is defined in ApiP.h and set to 5 structures per data block. To avoid

the growth of a device server, all client connections should be freed correctly.

\subsubsection The Device ID

Every device in a server is referenced by a device ID. The ID is send with every

client call to identify the device and is hidden to the user in the client handle

to the device. Up to version 3.37 the device ID was a simple number. Indicating

the position of the device in the list of exported devices. Now the device ID

was split up into several information fields.

\verbatim

| 31 | 30             20 | 19         12 | 11             0 |

-------------------------------------------------------------

|     |                    |              |

|     |                    |              |- Position in the

|     |                    |                 list of exported

|     |                    |                 devices.

|     |                    |

|     |                    |- Position in the

|     |                       list of client

|     |                       connections to

|     |                       the device.

|     |

|     |- Export counter

|

|- Local access flag

\endverbatim

The export counter field becomes interesting only if you destroy an exported

object and reexport another or the same object again. In the case of a destroyed

object, the export counter is increased and all client connections on the old value

are no longer valid. A newly exported device might take the place in the list of

exported devices afterwards. The Local access flag is set if the dev_import()

detects a local device. The split up of the device ID limits a device server to

the following values:

\code

Maximum number of exported devices = 4096

Maximum number of client connections per device = 256

\endcode

*/