/*! \page labview  LabView for TACO

This part describes the LabView interface to TACO control systems. It describes

the DSAPI client interface and how to write TACO device servers in the LabView

graphical programming language G.

\section labview_started Getting started

In order to get started you need access to the following tools :

-# LabView on Unix (Linux, HP-UX, Solaris or Windows)

-# for Unix : the LabView-TACO shared libraries lv_dsapi.so and lv_dsclass.so

(only needed if you plan to write a device server in TACO) in your shared library

path (LD_LIBRARY_PATH for Linux, Solaris, BSD, and SHLIB_PATH for HP-UX)

-# for Windows : the LabView-Taco shared library lv dsapi.dll and oncrpc.dll in

your $PATH e.g. in c:\\Windows (you can download these two libraries as a zip file)

\note ftp://ftp.esrf.fr/pub/cs/taco/lv_taco.zip

-# start LabView (normally by typing labview) and open one of the example VI's

or wire your own following the instructions in this document.

\section labview_interface LabView-TACO interface

\subsection Clients

The following shared library calls have been implemented to interface TACO clients

to LabView :

-# lv_dev_putget() - will execute a command on a device, one input and output

argument is passed, an error code is returned and a status. The call has the

following calling syntax :

\code

long lv_dev putget (char *name, char *cmd, void *argin, void *argout, long *error);

\endcode

- name : device name e.g. "id11/oregon/1" (passed as C string type)

- cmd : command to execute e.g. "DevMoveRelative" (passed as C string

type)

- argin : input argument e.g. array of floats (passed as "Adapt to Type"

(specify "Handles by Value" on Labview 6i) type, this means the G program

has to wire the correct input type expected by the command to the input

argument e.g. if a double array is expected the LabView program has to

provide a double array as input, failure to do so can result in a core

dump of the LabView program ! NOTE: the input has to be wired even it

is not used to a dummy type of the correct type; this is because

LabView does not differentiate between not used input types when calling

library functions and will not allow the program to run if an input

is not wired - the RUN arrow is broken)

- argout: output argument e.g. array of floats (passed as "Adapt to Type"

(specify "Handles by Value" on Labview 6i), see text above for advice

on how to use this type. NOTE: the input has to be wired even it is an

output to a dummy type of the correct type; this is because LabView

does not differentiate between output only types when calling library

functions and will not allow the program to run if an input is not

wired - the RUN arrow is broken)

- error : output error if any (passed as pointer to 32 bit integer. See

NOTE above for wiring the output error)

- returns : a 32 bit integer indicating the status of the command

(0 = OK, -1 = NOT OK)

- NOTE : refer to section on types to know which are supported for argin

and argout

-# lv_dc_devget() - will retrieve the result of a command on a device from the

data collector (the TACO data cache), one output argument is passed, an error

code is returned and a status. The call has the following calling syntax :

\code

long lv_dc_devget(char *name, char *cmd, void *argout, long *error)

\endcode

- name : device name e.g. "sr/d-ct/1" (passed as C string type)

- cmd : command to execute e.g. "DevReadSigValues" (passed as C string

type)

- argout: output argument e.g. array of floats (passed as "Adapt to Type"

(specify "Handles by Value" in Labview 6i), see text above for advice

on how to use this type. NOTE: the input has to be wired even it is an

output to a dummy type of the correct type; this is because LabView does

not differentiate between output only types when calling library functions

and will not allow the program to run if an input is not wired - the RUN

arrow is broken)

- error : output error if any (passed as pointer to 32 bit integer. See NOTE

above for wiring the output error)

- returns : a 32 bit integer indicating the status of the command

(0 = OK, -1 = NOT OK)

- NOTE : refer to section on types to know which are supported for argin

and argout

-# lv_dev_protocol() - change the RPC protocol on a device to TCP or UDP.

NOTE: the default is UDP. Changing to TCP will make client connections more reliable

and allow correct error detection (e.g. server down error). The call has the following

syntax :

\code

long lv_dev_protocol(char *name, char*protocol, long *error);

\endcode

- name : name of device e.g. "id11/oregon/1" (passed as C string type)

- protocol : RPC protocol e.g. "tcp" or "udp" (passed as C string type)

- error : error if any (passed a pointer to 32 bit integer)

- returns : a 32 bit integer indicating the status of the command

(0 = OK, -1 = NOT OK)

-# lv_dev_timeout() - change RPC timeout on a device. The call has following syntax :

\code

long lv_dev_timeout(char *name, long timeout, long *error);

\endcode

- name : name of device e.g. "id11/oregon/1" (passed as C string type)

- timeout : new timeout in milliseconds (pass as 32 bit integer)

- error : error code if any (passed as pointer to 32 bit integer)

- returns : a 32 bit integer indicating the status of the command

(0 = OK, -1 = NOT OK)

-# lv_dev_free() - close down all connections to a device. This call is useful for

releasing access to devices not used anymore. It closes all open network connections

to the device thereby also saving open file descriptors. The first call to the

device will reimport the device. The call has following syntax :

\code

long lv_dev_free(char *name, long *error);

\endcode

- name : name of device e.g. "id11/oregon/1" (passed as C string type)

- error : error code if any (passed as pointer to 32 bit integer)

- returns : a 32 bit integer indicating the status of the command

(0 = OK, -1 = NOT OK)

-# lv_dev_error_str() - return TACO error string corresponding to error no.

The call has following syntax :

\code

char *lv_dev_error_str( long error_no);

\endcode

- error_no : error number

- returns : text to the error number error_no (passed as C string type)

-# lv_dev_cmd_query() - return the list of commands supported by a device and

their types. This call is mainly useful as info for the LabView programmer to

know what commands and what data types are supported for a device . The call

has following syntax :

\code

long lv_dev_cmd_query(char *device, void* cmd list, long *error);

\endcode

- device : name of device e.g. "id11/oregon/1" (passed as C string type)

- cmd_list : list of commands and their input/output types returned as an

array of strings (passed as "Adapt to Type", specify "Handles by Value"

in Labview 6)

- error :

- returns : a 32 bit integer indicating the status of the command

(0 = OK, -1 = NOT OK)

-# lv_dc_cmd_query() - return the list of commands polled by the data collector

for a device and their types. This call is mainly useful as info for the LabView

programmer to know what commands and what data types are supported for a device

in the data collector. The call has following syntax :

\code

long lv_dc_cmd_query(char *device, void* cmd list, long *error);

\endcode

- device : name of device e.g. "id11/oregon/1" (passed as C string type)

- cmd list : list of commands and their input/output types returned as an

array of strings (passed as "Adapt to Type", specify "Handles by Value"

in Labview 6)

- error : error code if any (passed as pointer to 32 bit integer)

- returns : a 32 bit integer indicating the status of the command

(0 = OK, -1 = NOT OK)

-# lv_db_getdevexp() - return the list of exported devices whose names satisfy

the filter. The call has following syntax :

\code

long lv_db_getdevexp(char *filter, void* device list, long *error);

\endcode

- device : name filter of format D/M/F where either of D, F or M can be

the wildcard *

- device list : list of device names returned as an array of strings (passed

as "Adapt to Type", specify "Handles by Value" in Labview 6)

- error : error code if any (passed as pointer to 32 bit integer)

- returns : a 32 bit integer indicating the status of the command

(0 = OK, -1 = NOT OK)

\subsection Servers

It is possible to write TACO device servers in LabView (described below). HOWEVER

the preferred method to make Labview callable from TACO clients is to generate a

shared library (DLL in Windows parlance) of your Labview program using the

Application Builder and to call the shared library from a TACO device server. If

you cannot generate a shared library from Labview then use the technique below.

TACO device servers in Labview work by implementing a device server loop in a part

of the G program which polls the network periodically (using the lv_ds_cmd_get()

call) to see if there are any client requests. Once a request is detected the

LabView program has to execute it and then return the answer to the client (using

the lv_ds_cmd_put()) call. Clients and servers communicate using the TACO RPC

protocol on the network. The TACO devices have to be defined beforehand in the

TACO database. The following shared library calls have been implemented to write

TACO device servers in G :

-# lv_ds_init() - create and initialise a LabView TACO device server, to be called

once in a LabView device server before calling lv_ds_cmd_get(). The call has the

following calling syntax :

\code

long lv_ds_init(char *server, char*name);

\endcode

- server : device server executable name e.g. "StressRigds" (passed as C

string type)

- name : device server instance/personal name e.g. "id11" (passed as C

string type)

-# lv_ds_cmd_get() - poll network to see if a client request has arrived, if yes

command returned is non-zero. Can only be called after lv_ds_init() has been

called. The call has the following syntax :

\code

long lv_ds_cmd_get(long *command, void *argin);

\endcode

- command : command received, can be one of

-# DevLVIOStringDevState - takes array of strings as input and

returns array of strings as output

-# DevLVIODouble - takes array of doubles as input and returns

array of doubles as output

-# DevReadValue - returns an array of doubles as output

-# DevSetValue - takes array of doubles as input

-# DevState

-# DevStatus

-# 0 if no client request.

DevState and DevStatus are handled automatically by the LabView device

server

- argin : passed as "Adapt to Type" (specify "Handles by Value" on Labview 6).

-# lv_ds_cmd_put() - return result to client after completing executing of command.

Must only be called after a lvr_ds_cmd_get() has returned a non-zero command value.

The call has the following calling syntax :

\code

long lv_ds_cmd_put(long command, void *argout);

\endcode

- command : command returned by lv_ds_cmd_get() (passed as 32 bit integer)

- argout : double array to be passed back to client if command was

DevIOLVString, DevIOLVDouble or DevReadValue (passed as "Adapt to Type"

(specify "Handles by Value" on Labview 6))

\subsection Debugging

An additional command exists to set the debugging flag in the library in order to

display debugging information in a graphical window. The command is :

\code

void lv_ds_debug(long debug);

\endcode

sets the debug flag. Setting the debug flag to a non-zero value switches on the

debugging, zero switches it off.

\section Types

All TACO kernel types and motor types are supported. The following types are supported

as input and output :

-# D_VOID_TYPE

-# D_SHORT_TYPE

-# D_LONG_TYPE

-# D_FLOAT_TYPE

-# D_DOUBLE_TYPE

-# D_STRING_TYPE

-# D_VAR_STRINGARR

-# D_VAR_SHORTARR

-# D_VAR_LONGARR

-# D_VAR_ULONGARR

-# D_VAR_FLOATARR

-# D_VAR_DOUBLEARR

The following types are supported as input only :

-# D_MOTOR_FLOAT

-# D_MULMOVE_TYPE

The following arguments are supported in output only :

-# D_LONG_READPOINT

-# D_FLOAT_READPOINT

-# D_DOUBLE_READPOINT

-# D_STATE_FLOAT_READPOINT

-# D_VAR_LRPARR

-# D_VAR_FRPARR

-# D_VAR_SFRPARR

-# D_OPAQUE_TYPE

\subsection Examples

There are examples of calling all the functions as well as examples of a TACO LabView

device server (device server.vi) and client (device client.vi) are available. Study

them to find out how to write your own clients and servers in LabView.

\section problems  Known Problems

Some of the known problems with the TACO LabView interface are :

-# out of memory - sometimes LabView gives this error message when calling the

lv_dev_putget() function. I don't know what this is due to. It seems to be occur when

a string is passed as output. The only solution I have found is to rewire the

corresponding (dummy) input for the output to another string type or to recreate it

as a constant. This problem has been mostly occurring on HP-UX. If anyone finds a

better explaination/solution for this problem let me know.

-# limited number of devices - the present interface is limited to 1000 TACO

devices in a LabView session

-# not unregistered LabView device servers - there is no routine yet for unregistering

LabView device servers. This is not a major problem. It simply means the number of

reserved program numbers will go up every time a LabView device server is started.

-# blocked lv_dev_putget() call - this can happen on HPUX when trying to access

a device running on a host which is not reachable from the host on which LabView

is running. The present implementation will block. This problem does not occur on Linux.

-# client cannot run in same Labview session as server - the present version does

not support running clients in the same Labview session as the device server.

-# too many files open - this problem can be encountered in Labview applications

which have too many devices open. The latest (V1.5) version of the TACO interface

sets the limit of the number of open files to the maximum operating system allowed

value at the first call. If you still have problems then your application is too big!

Previous versions (<V1.5) are limited by the default limit for no. of open files

e.g. 60 on HP-UX.

-# Labview hangs - this can happen when accessing a device with UDP protocol and

the server/host are down. The solution is to switch to TCP protocol.

-# Windows port is based on DSAPI V5.15. Data collector interface and lv_dsclass

have not been ported (yet). Could be if the need arises.

-# the resizing of variable length string arrays as output does not work properly.

It will downsize the input array but not upsize it. To get around this problem pass

a string array initialised with more elements than you know will be returned. The

array will then come back with the correct size. Apparently there is a leak with

string arrays. If you see this please let me know so I can reproduce it and try

to fix it.

\section future_development Future developments

This is the sixth release of the LabView TACO interface. A number of improvements

been made e.g. adding support for the data base, dynamic device management,

increasing limit on open files. In the future we plan to offer a VI library for

TACO which will reduce the programming effort on clients even more. If any readers

have ideas for other improvements they should send their comments to the author

(goetz@esrf.fr) or even better add them to the source code themselves and then

send the code to the author!

*/