/*! \page testtool A tool to test a TACO control system

\addindex testtool

testcs is a TACO tool built to test a control system. It is able to test from a

single device server to a complete TACO control system. Testing a device server

is done by sending a network request to it and waiting for the answer. It does

not test the device served by the device server but only the device server ability

to answer to netwok request.

The tool takes its input directly from the TACO device server database and must

run on the same computer than the database. It is available for HP-UX, SunOS and

Solaris.

\section Usage

Five option are available :

- -k to test a TACO control system kernel servers. The kernel servers are :

- The manager

- The database server

- The data collector server(s) if the control system is running with a

data collector

- -d to test a device server. The full device server name must be specified

(device server executable name/personal name)

- -h to test all the device server running on a specific host. The host name must

be specified.

- -a to test a complete control system. In this case, the tool will test the kernel

servers and all the device servers running on all the hosts used in the control

system.

- The last option -v is a verbose option. This option has a meaning only with the

-k,-h and -a options. In verbose mode, the tool displays the answer of all the

tested device server. In non verbose mode, only the faulty device server are

reported to the user.

If the option -a is used, the tool will inform you of :

- All the missing device servers which have not been started.

- All the started but dead device servers. A manual page is available under UNIX.

\section  usage_example Usage example

\section testing_a_device_server Testing a device server

Test of a running device server called PneumValves started with the personal name

sr_c02.

\verbatim

$testcs -d pneumvalves/sr_c02

DS pneumvalves/sr_c02 : UDP version 1 ==> OK

DS pneumvalves/sr_c02 : TCP version 1 ==> OK

DS pneumvalves/sr_c02 : UDP version 4 ==> OK

DS pneumvalves/sr_c02 : TCP version 4 ==> OK

$

\endverbatim

If the device server is badly killed (with a kill -9 under UNIX or if the device

server has crashed).

\verbatim

$testcs -d pneumvalves/sr_c02

DS pneumvalves/sr_c02 : UDP version 1 ==> NOK, leaving test

DS process PID found in database : 17185

$

\endverbatim

If the device server is nicely killed.

\verbatim

$testcs -d pneumvalves/sr_c02

DS pneumvalves/sr_c02 defined in database on host libra but not started

$

\endverbatim

If the device server is unregistered from the database (dbset_servunreg or

dbm_servunreg command) or has never been started.

\verbatim

$testcs -d pneumvalves/sr_c02

Device server is not running (PN in db = 0)

$

\endverbatim

If the device server is deleted from the database (dbset_servdel or dbm_servdel

command

\verbatim

$testcs -d pneumvalves/sr_c02

Device server not defined in database

$

\endverbatim

\section test_kernel Testing control system kernel servers

Example of the testcs answer started with option -k and -v on the ESRF machine

control system:

\verbatim

$ testcs -k -v

Manager : UDP version 1 ==> OK

Manager : UDP version 4 ==> OK

Database server : UDP version 1 ==> OK

Database server : UDP version 2 ==> OK

Database server : UDP version 3 ==> OK

Database server : TCP version 1 ==> OK

Database server : TCP version 2 ==> OK

Database server : TCP version 3 ==> OK

Data collector read server 1 on gemini : TCP version 1 ==> OK

Data collector read server 1 on gemini : UDP version 1 ==> OK

Data collector read server 2 on gemini : TCP version 1 ==> OK

Data collector read server 2 on gemini : UDP version 1 ==> OK

Data collector read server 3 on gemini : TCP version 1 ==> OK

Data collector read server 3 on gemini : UDP version 1 ==> OK

Data collector read server 4 on gemini : TCP version 1 ==> OK

Data collector read server 4 on gemini : UDP version 1 ==> OK

Data collector read server 5 on gemini : TCP version 1 ==> OK

Data collector read server 5 on gemini : UDP version 1 ==> OK

Data collector write server 1 on gemini : TCP version 1 ==> OK

Data collector write server 1 on gemini : UDP version 1 ==> OK

Data collector write server 2 on gemini : TCP version 1 ==> OK

Data collector write server 2 on gemini : UDP version 1 ==> OK

Data collector write server 3 on gemini : TCP version 1 ==> OK

Data collector write server 3 on gemini : UDP version 1 ==> OK

Data collector write server 4 on gemini : TCP version 1 ==> OK

Data collector write server 4 on gemini : UDP version 1 ==> OK

Data collector read server 1 on aries : TCP version 1 ==> OK

Data collector read server 1 on aries : UDP version 1 ==> OK

Data collector read server 2 on aries : TCP version 1 ==> OK

Data collector read server 2 on aries : UDP version 1 ==> OK

Data collector read server 3 on aries : TCP version 1 ==> OK

Data collector read server 3 on aries : UDP version 1 ==> OK

Data collector read server 4 on aries : TCP version 1 ==> OK

Data collector read server 4 on aries : UDP version 1 ==> OK

Data collector read server 5 on aries : TCP version 1 ==> OK

Data collector read server 5 on aries : UDP version 1 ==> OK

Data collector write server 1 on aries : TCP version 1 ==> OK

Data collector write server 1 on aries : UDP version 1 ==> OK

Data collector write server 2 on aries : TCP version 1 ==> OK

Data collector write server 2 on aries : UDP version 1 ==> OK

Data collector write server 3 on aries : TCP version 1 ==> OK

Data collector write server 3 on aries : UDP version 1 ==> OK

Data collector write server 4 on aries : TCP version 1 ==> OK

Data collector write server 4 on aries : UDP version 1 ==> OK

$

\endverbatim

\section testing_all_device_servers Testing all the device server running on a host

This is a copy of the output of testcs started with the -h and -v option for one

of the ESRF machine control system VME:

\verbatim

$ testcs -h vme006 -v

Test host : vme006

DS plc/sy_s678 and pneumvalves/sy_s678 : UDP version 1 ==> OK

DS plc/sy_s678 and pneumvalves/sy_s678 : TCP version 1 ==> OK

DS plc/sy_s678 and pneumvalves/sy_s678 : UDP version 4 ==> OK

DS plc/sy_s678 and pneumvalves/sy_s678 : TCP version 4 ==> OK

DS ripc/sy_s678 and ripc-channel/sy_s678 : UDP version 1 ==> OK

DS ripc/sy_s678 and ripc-channel/sy_s678 : TCP version 1 ==> OK

DS arun/sy_s678 and pg.arun/sy_s678 : UDP version 1 ==> OK

DS arun/sy_s678 and pg.arun/sy_s678 : TCP version 1 ==> OK

DS arun/sy_s678 and pg.arun/sy_s678 : UDP version 4 ==> OK

DS arun/sy_s678 and pg.arun/sy_s678 : TCP version 4 ==> OK

DS magvaccoolingilds/sy and cellmagil/sy : UDP version 1 ==> OK

DS magvaccoolingilds/sy and cellmagil/sy : TCP version 1 ==> OK

DS thctrl/sy and srthc/sy : UDP version 1 ==> OK

DS thctrl/sy and srthc/sy : TCP version 1 ==> OK

DS thctrl/sy and srthc/sy : UDP version 4 ==> OK

DS thctrl/sy and srthc/sy : TCP version 4 ==> OK

$

\endverbatim

On this output, you can remark that device server with several embedded classes

are tested as one server (plc/sy_s678 and pneumvalves/sy_s678 are part of the same

device server process). It is also possible to detect old device server which are

registered in the RPC layers with version 1 only (ripc/sy_s678 and magvaccoolingilds

servers).

\section testing_complete_control_system Testing a complete control system

The following is a result of testcs started on a ESRF beam line control system

with the -a option

\verbatim

$testcs -a

Testing control system kernel components

Getting information from the whole control system

On large control system, this may needs time !

Getting information for : id101

Getting information for : id102

Getting information for : id106

Getting information for : tina

Control system with 34 server process(s) distributed on 4 host(s)

Testing device server(s) running on id101

Testing device server(s) running on id102

DS gpib/dummy and mcamb/id10 : UDP version1 ==> NOK !!!!!!

DS process PID found in database : 66

DS wxbpm/mcd defined in database on host id102 but not started

Testing device server(s) running on id106

Testing device server(s) running on tina

DS ud_daemon/ud_atte defined in database on host tina but not started

$

\endverbatim

This exmaple does not use the verbose mode of testcs. From the output, you can

conclude that

- All the kernel conponents are running well (manager, database server and data

collector).

- The control system is distributed on 4 hosts and uses 34 device servers.

- The deice server gpib/dummy is not running

- The device servers wxbpm/mcd and ud_daemon/ud atte have not been started.

*/