DServerSerialSoftware Documentation Version courante du document : 1.0Dernière modification  le 03/10/2003Historique des modificationsDateRevisionDescriptionAuthor24/09/20031.0Initial Version X. Elattaoui03/10/2003Read line is OKX. ElattaouiSoftware ArchitectureHistory  Our work was to take the Tango Serial Line DeviceServer wrote by ESRF for LINUX , and port it  on Win32 platform. We use the old TACO Serial Line WIN32 DeviceServer as a code basis to avoid the numerous WIN32 API pitfalls.Basic philosophy : We kept the existing interface for compatibility reasonsWe don’t have modify the code under Linux (or just a little bit to correct some bugs !)We have added some methods to the existing interface to improve ergonomics (ergonomy ?) .Design considerationFile organization	There are three files. The main is “Serial.cpp” which contains all the methods used on Linux and Win32 platform. And the particular methods for both platforms are in :“SerialWin32.cpp” for Win32“SerialLinux.cpp” for LinuxMiscellaneous changesReplacement of cout and coutx by new Tango log streamsAdd new commands to simplify the utilisation from the different clients :about read :  DevSerNcharReadStringabout settings :  SetTimeOut, SetParity, SetBaudRate, …The asynchronous mode for writing is no more used. Only synchronous writes (CreateFile function calls) are now done on the COM port . For read and write, flag OVERLAPPED is in the CreateFile function call not used but pointer to the structure OVERLAPPED is given for further ReadFile calls (seems to work better like this even if doc says it’s not necessary). So Read and Write command don’t return until they have finished.Timeout is now handled for the ReadLine command Current known limitationsTest under LinuxTill now, the server has not be tested  under Linux . A non regression test python script is provided with the Dserver to allow a quick functionnal test on linux.PerformancesNon regression test procedureInstall the Tango/Python bindingWith a simple cable, just connect the 2 com ports of your CPU. Then declare 2 devices : com1 and com2 Run the test_serial_line.py python script . If it runs , your Dserver is probably working !!