----- file created by makedoc on: Thu Jul 20 11:12:46 2000

----- Documentation for /pgexp/applications1/XRayOptics/xop/src_idl/dabax/crystal_fh.pro -----

NAME:

CRYSTAL_FH

PURPOSE:

Calculates the crystal structure factor and other parameters.

CATEGORY:

X-Ray optics. DABAX database.

CALLING SEQUENCE:

crystal_fh,str,phot[,theta,text=text,out=out]

INPUTS:

str: structure with inputs (as produced by bragg_calc)

phot: Photon energy in eV

OPTIONAL INPUTS:

theta: Scattering angle in degrees.

If theta is nod defined, crystal_fh considers the

bragg angle for the given configuration.

KEYWORDS:

text: a named variable where to write out a text with the

results of the calculations.

str: a named variable where to write out a structure with the

results of the calculations.

OUTPUT:

The output is written in the keyword variables.

PROCEDURE:

Adapted from crystal_fh.F (SHADOW 2.2 ray-tracing package).

EXAMPLE:

inp = bragg_defaults()

out = bragg_calc(inp)

text = ''

crystal_fh,out,8000.,text=text

MODIFICATION HISTORY:

96-07-10 Written by M. Sanchez del Rio (srio@esrf.fr)