================================   MAMON   =====================================

MAMON (Multiple beAM diffractiON) is an widget based graphical interface to

calculate the Umweganregung peak location plot (the diffracted wavelength

lambda vs. the azimutal angle angle Psi) for a given primary reflection in

perfect cubic crystals.

The program computes first all Miller indices hkl associated to the

different crystal planes which meet the diffraction condition (multiple

diffraction events) in the two-beam multiple diffraction approximation.

Translated into the reciprocal space this means that two or more knots of the

reciprocal lattice come across the Ewald's sphere of radius 1/lambda (where

lambda is the wavelength) at the same time when this is rotated of an angle

Psi, say, about one diffraction vector P = (H,K,L).

The azimuthal angle Psi is calculated relative to an arbitrarily chosen

reference direction perpendicular to the scattering vector of the primary

reflection.  The user can specify the energy and the angular range as well

as the max hkl values considered

The theoretical frame of the program is essentially provided in:

1) B.H. Cole et al, "Simultaneous Diffraction: Indexing Umweganregungen

Peaks in Simple Cases", Acta Cryst. (1962), 15, 138-144

The peak intensity is calculated with the approximated method described in:

2) K. Yvon et al., "LAZY PULVERIX, a computer program for calculating X-ray

and neutron diffraction powder patterns", J. Appl. Cryst. (1977), 10, 73-74

Basically, the intensity I is calculated as I= M L P F**2, being

M: multiplicity factor,

P: Polarization factor,

L: Lorentz factor (L=1/[sin_theta**2  cos_theta])

Theta is the Bragg angle.

F is calculated considering the scattering factor f0 calculated using

using the coefficients tabulated in the file mamon.f0.

Obviously, this intensity is not correct because:

- The formula is not valid for perfect crystals, only for powders.

- The total intensity must be the sum of the primary reflection

plus the intensity of the so-called operative reflections plus the

contribution of the cooperative ones.

- Anomalous scattering factors are not considered.

The crystal group symmetries represented in the program are: simple cubic,

body-centered cubic, face-centered cubic. In particular the selection rules

for reflections peculiar of the diamond structure are also built in in the

code.

DESCRIPTION OF THE CONTROLS IN THE MAIN WINDOW:

File:

MAMON input parameters: This option allows to save the current

parameters to a file for later loading. It also allows

to save the current parameters as defaults for being

used when the application is initialized. In the last

case, the file is named "application".xop (where

"application " is the name of the current XOP

application) and is written in the directory pointed

by the XOP_DEFAULTS_DIR environment variable (which

must be set). The parameter file is ASCII and can be

read and edited with care.

Quit: to exit from the program

Set_Parameters:

Set Parameters: to define the parameters for the calculation.

The same result is obtained pressing the "Set Parameters"

button in the main BM window.

Please refer to the information under the HELP

button for a complete description of the parameters.

Set Defaults: Sets the default parameters.

Show: Display results

Plot Results: Plots the MAMON results.

View Results: Displays the MAMON output text file mamon.dat, which

contains:

h,k,l     the Miller indices of the current Umweganregung event

theta     the cooresponding Bragg angle

F(hkl)    the absolute value of the calculated structure factor.

The Debye-Waller factor is not taken into acount

Intensity the reflection intensity

Beta      angle between the projectons of the vectors r = (h,k,l) (which

meets the Bragg condition at the same time as P) and

k0 = (e0x,e0y,e0z)/lambda on the plane perpendicular to P

[e0 is the unit vector]. Let's call these projections r_perp

and k0_perp, respectively

Psi0      angle between a reference axis perpendicular to P

(calculated by the program) and r_perp

Psi       rotation angle around the main diffraction vector

calib     (not relevant for its use in this context)

lambda [Angstrom] as calculated from eq. 4 of ref. 1.

This eq. gives the relationship between primary as well as

operative reflection and k0 for simultaneous diffraction

to occur.

Help:   Shows the BM help (this text).

COPYRIGHT:

MAMON belongs to XOP package and it is distributed within XOP.

PLEASE REFER TO THE XOP COPYRIGHT NOTICE BEFORE USING IT.

CREDITS:

Published calculations made with XOP should refer:

M. Sanchez del Rio and R. J. Dejus "XOP: Recent Developments"

SPIE proceedings vol. 3448, pp.340-345, 1998.

LAST MODIFICATION: msr/msr/99-03-19