XCOM: Photon Cross Sections on a Personal Computer

M. J. Berger and J. H. Hubbell

Center for Radiation Research

National Bureau of Standards

Gaithersburg, MD 20899

A computer  program and data base  are presented

which can be used to calculate, with  a personal

computer, photon cross  sections for scattering,

photoelectric absorption and pair production, as

well as total  attenuation  coefficients, in any

element,  compound or mixture, at  energies from

1 keV to 100 GeV.

1. Introduction

Data  on the scattering and absorption of photons (x-rays,  gamma rays,

bremsstrahlung)  are  required for many scientific,  engineering and medical

applications.  The  number  of materials for which photon cross sections are

needed is large and ever increasing. Available tables [1-11] usually include

cross sections for many (but not all) elements.  Some tables [1,2,6,11] also

contain data for a limited number of compounds and mixtures.  In practice it

is  not  possible to meet all cross-section requirements adequately by means

of printed tables.  Moreover,  the cross sections are often needed at photon

energies other than those included in the tables.

Photon  cross  sections  for compounds can of course be obtained rather

accurately  (except  at energies close to absorption edges) as weighted sums

of  the  cross sections for the atomic constituents.  However,  the required

numerical work  is tedious,  and the task is further complicated by the fact

that  photoabsorption  cross sections and total attenuation coefficients are

discontinuous  at  absorption  edges.  The presence of these discontinuities

makes  it  desirable  that cross section tables for compounds include photon

energies  immediately  above  and below all the absorption edges for all the

atomic constituents, and this requires much additional interpolation.

A convenient alternative approach is to generate the cross sections and

attenuation  coefficients  for  compounds   and mixtures as needed,  using a

personal  computer.  This  paper  describes   a computer program called XCOM

which  carries  out  this task quickly on IBM-compatible personal computers,

for any element, compound or mixture, at energies between 1 keV and 100 GeV.

The  program makes use of a database of cross sections for the elements that

is stored in compressed form on a single floppy disk.

The  XCOM program can generate cross sections on a standard energy grid

(spaced  approximately  logarithmically), or on a grid selected by the user,

or for a mix of both grids. Cross sections at energies immediately above and

below  all  absorption edges are automatically included.  XCOM provides  two

forms of output:  (a) tables which  correspond closely in format to existing

tables in the literature;  (b) user-selected arrays which are convenient for

further computer calculations.

The  program provides total cross sections and attenuation coefficients

as  well  as partial cross sections for the following processes:  incoherent

scattering,   coherent  scattering,   photoelectric  absorption,  and   pair

production in the field of the atomic nucleus and in the field of the atomic

electrons.  For  compounds,  the  quantities tabulated are partial and total

mass  interaction  coefficients,  which   are   equal  to the product of the

corresponding  cross  sections times the number of target molecules per unit

mass of the material.  The reciprocals of these interaction coefficients are

the  mean free paths between scatterings,  between photo-electric absorption

events,  or  between  pair  production   events.  The sum of the interaction

coefficients  for the individual processes is equal to the total attenuation

coefficient.  Total  attenuation  coefficients without the contribution from

coherent scattering are also given, because they are often used in gamma-ray

transport calculations.

The  interaction  coefficients  and  total attenuation coefficients for

compounds  or  mixtures are obtained as sums of the corresponding quantities

for the atomic constituents.  The weighting factors,  that is, the fractions

by  weight  of  the  constituents,  are calculated by XCOM from the chemical

formula entered by the user. For mixtures, however, the user must supply the

fractions by weight of the various components.

Some  limitations  should be noted.  The cross sections for elements in

the  XCOM  database pertain to isolated neutral atoms,  and do not take into

account  molecular and solid-state effects  which modify the cross sections,

especially  in the  vicinity  of absorption  edges.  Relatively  small cross

sections, such as those for Delbruck scattering, two-photon Compton scatter-

ing or photo-meson production, are not included. Also omitted is the nuclear

photoeffect  which, in the giant-dipole  resonance region from  5 to 30 MeV,

can contribute a few percent to the total attenuation coefficient.  Finally,

XCOM does not calculate  energy  absorption  coefficients that represent the

conversion of photon energy to kinetic energy of secondary Compton-, photo-,

and pair-electrons.

2. Database for Elements

A comprehensive database for all elements over a wide range of energies

was constructed through the combination of incoherent and coherent scattering

cross  sections  from  Refs. [12]  and  [13], photoelectric absorption  from

Scofield  [14],  and  pair  production  cross  sections  from Ref. [8].  For

scattering and pair production, the same cross sections are used as in other

recent tabulations in Refs. [6, 8, and 11], whereas for photoelectric absorp-

tion  there is a small difference (omission of a renormalization correction)

which is discussed below.

The  incoherent  (Compton) scattering  cross sections in Ref. [12] were

obtained from a combination of the Klein-Nishina formula and nonrelativistic

Hartree-Fock incoherent scattering functions.  Radiative and double Compton-

scattering   corrections  were  also  included.   The   coherent  (Rayleigh)

scattering cross sections in Ref. [13] were calculated from a combination of

the Thompson formula and relativistic Hartree-Fock atomic form factors.  The

photoelectric cross sections were obtained by Scofield [14] by a phase-shift

calculation  for  a  central  potential  and a Hartree-Slater atomic  model.

Scofield's results extend only up to 1.5 MeV.  At higher energies, where the

photoelectric  cross section is quite small, a  semi-empirical  formula from

Ref. [2] connects Scofield's values at 1.5 MeV to the asymptotic high-energy

limit calculated by Pratt [15].  Cross sections for pair production given in

Ref. [8] are  based on  complicated  combinations  of formulas  from  Bethe-

Heitler  theory with various other  theoretical models to  take into account

screening, Coulomb, and radiative  corrections.  Different combinations were

used in the  near-threshold, intermediate and high-energy  regions to obtain

the best possible agreement with experimental cross sections.

For  elements with atomic numbers from 2 to 54, Scofield [14] presented

correction  factors   for  individual  atomic  subshells,   with  which  the

photo-effect cross  sections  can be  renormalized so  that they  correspond

approximately  to a relativistic Hartree-Fock model rather than the Hartree-

Slater model used in the original calculation.  This renormalization is most

significant for the outer atomic shells;  the total cross section is lowered

by  no  more  than  10  percent   at energies above 1 keV.  Scofield did not

actually  apply  the  renormalization   to   the cross sections given in his

tables.  The  renormalization   was   used,  however,  in the tabulations in

Refs. [6, 8, and 11].  Recent reviews [16,17]  indicate that,  on the whole,

agreement with experiment is better when the renormalization is not done. We

have  therefore  omitted  the renormalization  in the database  for the XCOM

program.

3. Interpolation and Combination

For  the  purpose  of interpolation with respect to photon energy,  the

coherent  and incoherent scattering cross sections and the total attenuation

coefficients  are  approximated by log-log cubic-spline fits as functions of

energy.  For the pair-production cross sections,  the fitted quantity is the

logarithm of the quantity ((1-E'/E)**3)*SPAIR(E), where E is the photon energy,

E' the  threshold  energy  for pair  production,  and SPAIR(E)  is the cross

section.  The fitting is done separately for pair production in the field of

the atomic nucleus (E' = 1.022 MeV) and in the field of the atomic electrons

(E' = 2.044 MeV).

The  combined  photoelectric absorption cross section for all shells is

similarly  interpolated  with use of log-log cubic-spline fits,  but only at

energies above the K-shell absorption edge. Below this energy, interpolation

is  applied  to  the logarithm of the photoelectric absorption cross section

for each separate shell, fitted as a linear function of the logarithm of the

photon energy. The separate fitting for each shell is necessary to avoid the

error  that  would  be   incurred  by interpolating across absorption edges.

Linear  log-log  fitting  is  equivalent  to assuming that the photoelectric

cross section is proportional to a power of the photon energy, and was found

to  provide  more satisfactory fits than a log-log cubic-spline fit near the

absorption edges.

The  interaction  coefficients  and  total attenuation coefficients for

compounds  are obtained as weighted sums over the corresponding coefficients

for  elements.  XCOM automatically calculates the weight  factors, i.e., the

fractions  by weights of the atomic constituents,  from the chemical formula

for the compound entered by the user.  For mixtures, the user must enter the

fractions by weight of the components.

4. Overview of the XCOM Program

The hardware requirements for running the XCOM program are moderate. It

is  sufficient  to have an IBM-compatible personal computer with a memory of

at least 256K bytes,  and with at least one 5.25-inch floppy-disk drive.  It

is  assumed  that  the  computer  is   operated with the PC-DOS or an MS-DOS

operating system,  version 2.0 or later.  A mathematical co-processor (Intel

8087  or  80287 chip) is not required,  but is highly desirable,  because it

speeds up to execution of XCOM by a factor of twenty or better.

The  XCOM program is distributed on two 5.25-inch floppy disks.  One of

these  is  the  Database  Disk,  and   contains 100 data files designated as

MDAT.001,  MDAT.002, ..., MDAT.100.  These  files,  generated  by  a Fortran

program and written in binary format, contain the cross-section database for

the  elements  with  atomic  numbers  Z = 1 to 100.  The  other disk  is the

Program Disk, and contains an executable file called XCOM.EXE.

The Program Disk also contains a copy of this report, in file XCOM.DOC,

and  the Fortran source code for XCOM.  The source code is not needed to run

XCOM.  It  is  included  to make it possible for the user to make modify the

program,  and  to  adapt  the  program to for use with a different operating

system or computer.

The main program, XCOM, uses the following subroutines:

SPEC    Allows the user to specify the composition of the material,

the energy list, desired output etc.

FORM    Translates the chemical symbols for elements or chemical

formulas for compounds into the composition of the material

specified in terms of atomic numbers, atomic weights, and

fractions by weight of the atomic constituents.

MERGE   Creates a merged energy list arranged according to magnitude.

This list combines a standard energy grid (approximately

logarithmic) with the set of absorption-edge energies for

all the atomic constituents for a given compound or mixture,

and with the set of additional energies which the user wishes

to add.

REV     Utility routine for reversing the order of certain arrays.

SCOF    Routine for generating coefficients for cubic-spline fits.

BSPOL   Interpolation routine (based on binary search) making use of

cubic-spline interpolation coefficients from SCOF.FOR.

BLIN    Linear interpolation routine, based on binary search.

Also  included  on the Program Disk are five additional files which are

"included" in the main program or in subroutines at the time of compilation.

ENB.DAT and INDEX.DAT are for inclusion in XCOM,  HASH1.DAT and HASH2.DAT in

FORM, and ATWTS.DAT in XCOM and FORM.

5. How to Run the XCOM Program

If  the program is to be run using only floppy disks,  the Program Disk

should be inserted into drive A.  After the appearance of the A> prompt, the

program can be started by entering "XCOM".

If the program is to be run from a hard disk,  it is recommended that a

special  subdirectory  be created for running XCOM.  First,  the user should

copy  the  file  XCOM.EXE  from  the  Program  Disk  and  the   data   files

MDAT.001, MDAT.002, ..., MDAT.100  from the  Database Disk  into the special

directory.  After  the special directory is made the current  directory, the

program can be started by entering XCOM.

From this point on,  the program proceeds interactively.  The user must

respond to prompts (indicated by -->) by entering requested input data or by

making  choices from various options.  The requested information pertains to

the name and characterization of the substance of interest,  the desired set

of energies at which cross sections are to be computed,  and the form of the

output.

The  prompts are largely self-explanatory.  In order to indicate to the

user  what  to  expect,  we show  in this  report  a record  of prompts  and

responses  that  appeared  on the  monitor screen  in three  sample runs  of

XCOM,  for  an  element  (lead)  in  Appendix A,  for  a  compound  (calcium

tungstate) in Appendix B, and for a mixture (pyrex glass) in Appendix C. The

XCOM program could probably be run without further explanations.  Additional

information  is provided in  the remainder of this  Section, which will help

the user to formulate his responses to the prompts.

5.1. Name of Substance.

The  name can be freely chosen,  may include imbedded blanks,  and must

consist  of  no more than 72 characters.  The name will appear on the top of

the output table.

5.2. Elements, Compounds and Mixtures.

The  substance  for  which  cross   sections  are to be computed can be

designated to be an element, compound or mixture. Elements can optionally be

indicated  by  their   atomic   number,  or by their chemical  symbol. These

symbols,  or chemical formulas for compounds,  should be entered in standard

chemical notation,  with appropriate upper and lower case.  However, because

of  hardware limitations,  subscripts must be written on line.  For example,

the formula for calcium tungstate must be entered as CaWO4.

Substances  consisting of molecules with only a single species of atoms

can be designated as either as elements or compounds. For example, molecular

nitrogen  could  be treated as an "element" with symbol N,  or as a compound

with formula N2 (entered as N2).

5.3. Mixtures.

Mixtures  are  assumed  to  consist of two or more components,  each of

which can be either an element or a compound.  Whether the mixture is broken

down  into  "elemental"  components  or "compound" components is a matter of

convenience  (depending on the readily available information),  but does not

change the results.

The user  must indicate  how many components  there are in the mixture,

and must specify the chemical symbol or formula, as well as  the fraction by

weight,  for each component,  as prompted. The program then uses these input

data  to  compute  the   fractions   by  weight of for the individual atomic

constituents,  as  well  as the sum of these fractions.  This information is

displayed on the monitor screen. The input data might be faulty in the sense

that  the  sum of the fractions by weight does not add up to unity.  In this

case the user is given two choices:  1) The input data can be "accepted", in

which  case  the program renormalizes all of the fractions by weight so that

they add up to unity;  2) Another set of fractions by weight for the mixture

can be entered.

5.4. Quantities and Units.

For  elements,  the  user is  given  three  choices:  1) the output can

consist  of  cross  sections  for  individual  processes,  and  total  cross

sections, in units of  barns/atom, where 1 barn =10E-24 cm2;  2) the output

can consist of cross sections for individual processes,  in barns/atom,  and

mass  attenuation  coefficients,  in  cm2/g;  3)   the output can consist of

partial  mass  interaction  coefficients   and,  total   mass    attenuation

coefficients,  in  cm2/g.  For   compounds   and  mixtures the output always

consists  of  partial  mass interaction coefficients,  and total attenuation

coefficients, in cm2/g.

5.5. Energy List.

The user can 1) limit the output to the standard energy grid, 2) add to

the standard grid selected energies of his choice, or 3) request output only

for the set of energies selected by him.  In case 2) the additional energies

are merged into the standard energy grid according to magnitude. In case 3),

the  energies  will  appear  in  the output table exactly in the sequence in

which they were entered by the user.

If  additional  energies  are  entered  by the user,  this can be done,

optionally,  either  from the keyboard,  or from a previously prepared input

file.  This  file (stored in any desired directory on a floppy or hard disk)

should contain, as first item, the number of additional energies, and then a

list of energies, which items separated by blank spaces.

5.6.  Database Input Data

The  program  will  prompt  the   user  as to the  location of the file

containing the database files.  The user can request that these data be read

from  a  floppy  disk drives (A or B),  or from the current directory in the

hard disk.  Even  when the program is run from the  special directory on the

hard disk,  the data can still come from drives A or B.  If the data  are to

come from  the  hard  disk,  they  must have been  previously  stored in the

special directory, i.e.,  the current directory  from which the XCOM run was

started.

5.7.  Output Table

The  user  is  asked  by a prompt where the file with the cross section

table is to be stored.  The file specification can include the letter of the

drive,  the subdirectory,  and the file name.  If the current directory on a

hard  disk is to be used,  only the file name need be supplied.  If the user

wants  to  see the output table only on the monitor screen,  the output file

should  be designated as "CON".  If the user wants neither to inspect nor to

save  the  output table (because only the output arrays discussed in section

5.8 are wanted), the output file should be designated as "NUL".

On the top of each page of the table the name of the substance is given,

followed  by a listing of the atomic numbers and fractions by weight of  the

atomic  constituents.  The  main body of the  table is supplied  with enough

headings  to be  self-explanatory.  This left-most column gives the designa-

tions of  the  absorption  edges  (K, L1, L2, L3, M1, M2,..) as  well as the

atomic number Z  of the  pertinent  atomic constituent.  Data  for  energies

immediately below and above each edge, are given on two lines separated by a

blank  line.  It should be noted that the standard energy grid automatically

includes  at  least one  other  energy between any two successive absorption

edges.  For  materials  of low atomic number,  there are no absorption edges

above 1 keV, and the column indicating the names of edges is absent.

Typical output tables are shown in Table 1 (for an element,  lead),  in

Table  2 (for a compound,  calcium tungstate) and in Table 3 (for a mixture,

pyrex glass:  0.807 SiO2, 0.129 B2O3, 0.038 Na2O, 0.022 Al2O3, and 0.004 K2O

by  weight).  On  the top of each page,  the name of the substance is given,

followed  by  a listing of the atomic numbers and fractions by weight of the

atomic constituents.  Fig. 1 shows some of the results from Table 2, namely,

the  total  mass attenuation coefficient plotted as a function of the photon

energy.

5.8 Additional Output Arrays

The cross section tables described  in Section 5.7 are directly compar-

able in format  to printed  tables found  in the  literature,  and  are most

convenient for visual inspection of the data.  If these  data are to be used

in subsequent  computer  calculations,  the appropriate cross section arrays

must be extracted for the tables.  XCOM provides an option which facilitates

this task.

After  the  production  of the  cross  section table  is completed, the

program prompts the  user to indicate whether  additional output in the form

of arrays of selected quantities is  desired.  If the answer is affirmative,

a selection menu is presented.  The arrays which can be  selected correspond

to the columns in the  cross section table.  After each selection, this menu

is presented again.

The user  is also  asked to  provide the name  of the file in which the

additional arrays are to be stored. At the beginning of this file, preceding

the arrays, the following information  is stored.  The first record consists

of the name of the substance.  The second record gives  the number of atomic

constituents,  the  third  contains a  list of atomic numbers of the consti-

tuents and the fourth a list of fractions by weight.

The fifth  record gives the number of  sub-arrays into which each array

is divided, and the sixth contains the lengths of all successive sub-arrays.

Each  sub-array  contains   data  for  all  energies  between  two  adjacent

absorption edges.  This partitioning  facilitates the setting up of interpo-

lation schemes in which interpolation  across  absorption edges is  avoided.

Subsequent records  contain, for  each of the  quantities  selected, an

identifier (such as "energy list," "total attenuation coefficient") followed

by one or more sub-arrays.

Even though the arrangement of the file, as described above, may appear

somewhat complicated,  it can easily  be understood  by comparison  with the

corresponding  output table.  A typical set of output arrays,  consisting of

photon  energies  and  total   mass  attenuation  coefficients  for  calcium

tungstate,  is given  Table 4.   These arrays  are the same  as the  data in

columns 2 and 7 in Table 2, and were used to produce the plot in Fig. 1.

Acknowledgement.

This work  was supported by the  NBS Office of Standard Reference Data,

and by the U.S. Department of Energy (OHER).

References

1.  Hubbell, J.H. and Berger, M.J., Sections 4.1 and 4.2 in Jaeger, R.G.

(ed.): Engineering Compendium on Radiation Shielding (IAEA, Vienna),

Vol. 1, Ch. 4, pp. 167-202, Springer, Berlin (1968).

2.  Hubbell, J.H., Photon Cross Sections, Attenuation Coefficients and Energy

Absorption Coefficients  from 10 keV to 100 GeV, Natl. Stand. Ref. Data

Ser. 29 (1969).

3.  McMaster, W.H., Del  Grande,  N.K.,Mallett,  J.H,  and  Hubbell,  J.H.

Compilation of  X-ray Cross  Sections, Lawrence  Livermore Lab., Report

UCRL-50174, (1969).

4.  Storm, E. and Israel, H.I., Photon Cross Sections from 1 keV to 100 MeV

for Elements Z=1 to Z=100, Nucl. Data Tables A7, 565-681 (1970).

5.  Veigele, W.J., Photon Cross Sections from 0.1 keV to 1 MeV for Elements

Z=1 to Z=94, Atomic Data 5, 51-111 (1973).

6.  Hubbell, J.H., Photon Mass Attenuation and Mass Energy-Absorption

Coefficients for H,C,N,O,Ar and Seven Mixtures from 0.1 keV to 20 MeV,

Radiat. Res. 70, 58-81 (1977).

7.  Leroux, J, and Thinh, T.P., Revised Tables of X-ray Mass Attenuation

Coefficients, Corporation Scientifique Classique, Quebec (1977).

8.  Hubbell, J.H.,  Gimm., H.A.,  and Overbo,  I., Pair,  Triplet and Total

Atomic  Cross Sections (and Mass Attenuation Coefficients) for 1 MeV-100

GeV Photons in Elements Z=1 to 100, J. Phys. Chem. Ref. Data 9, 1023-1147

(1980).

9.  Plechaty, E.F., Cullen, D.E., and Howerton, R.J., Tables and Graphs of

Photon-Interaction Cross Sections from 0.1 keV to 100 MeV Derived from

the LLL Evaluated-Nuclear-Data Library, Report UCRL-50400, Vol. 6,

Rev. 3 (1981).

10.  Henke, B.L., Lee, P., Tanaka, T.J., Shimabukuro, R.L. and Fujikawa, B.K.,

Low Energy X-ray Interaction Coefficients:  Photoabsorption, Scattering

and Reflection, Atomic Data and Nuclear Data Tables, 27,1-144 (1982).

11.  Hubbell, J.H., Photon Mass Attenuation and Energy Absorption Coefficients

from 1 keV to 20 MeV, Int. J. Appl. Radiation & Isotopes, 33, 1269-1290

(1982).

12.  Hubbell, J.H.,  Veigele, W.J., Briggs, E.A., Brown, R.T., Cromer, D.T.,

and Howerton, R.J.,Atomic Form Factors, Incoherent Scattering Functions,

and Photon Scattering Cross Sections, J. Phys. Chem. Ref. Data 4, 471-538

(1975); erratum in 6, 615-616 (1977).

13.  Hubbell, J.H. and Overbo, Relativistic Atomic  Form Factors  and Photon

Coherent  Scattering Cross Sections, J. Phys. Chem. Ref. Data 8,

69-105, (1979).

14.  Scofield, J.H.,  Theoretical Photoionization  Cross Sections  from 1 to

1500 keV, Lawrence Livermore National Laboratory Rep. UCRL-51326 (1973).

15.  Pratt, R.H., Atomic Photoelectric Effect at High Energies, Phys. Rev.

117, 1017-1028 (1960).

16.  Saloman, E.B.  and Hubbell, J.H., Critical Analysis of Soft X-ray Cross

Section Data, Nucl. Instr. Meth. A255, 38-42 (1987).

17.  E.B. Saloman and J.H.Hubbell, X-ray Attenuation Coefficients (Total Cross

Sections): Comparison of the Experimental Data Base with Recommended Values

of Henke and the  Theoretical Values  of Scofield  for Energies between

0.1-100 keV, National Bureau of Standards Report NBSIR 86-3431 (1986).