----- file created by makedoc on: Thu Jul 20 11:13:08 2000

----- Documentation for /pgexp/applications1/XRayOptics/xop/src_idl/optics/xcrystal_bent.pro -----

=============================   Xcrystal_bent   ===============================

Xcrystal_bent is a common widget based graphical interface to different

codes that perform curved crystal diffraction calculations.

The idea is to perform bent crystal calculations with different

modules (computer codes) using the same interface. The different

modules use different theories. The initial considered theories are:

1) Lamellar theory.

2) Penning Polder theory.

3) Takagi-Taupin theory.

In addition, and for comparison purposes, a module to calculate

flat perfect crystals, based on the Zachariasen's theory has been

included. For FLAT crystals calculations it is recommended to use

the xcrystal application in XOP>

Plase note that:

The Penning-Polder model is only available for LAUE crystals

The Takagi-Taupin model is under testing for BRAGG crystals, and

it may not produce correct results.

XCrystal_bent uses the codes crystal_za (Zachariasen), crystal_ml

(Multi-lamellar) crystal_pp (Penning-Polder) and crystal_tt (Takagi-Taupin).

DESCRIPTION OF THE CONTROLS IN THE MAIN WINDOW:

File:

xcrystal_bent input parameters: This option allows to save the current

parameters to a file for later loading. It also allows

to save the current parameters as defaults for being

used when the application is initialized. In the last

case, the file is named "application".xop (where

"application " is the name of the current XOP

application) and is written in the directory pointed

by the XOP_DEFAULTS_DIR environment variable (which

must be set). The parameter file is ASCII and can be

read and edited with care.

Quit: to exit from the program

Set_Parameters:

Set Parameters: to define the parameters for the calculation.

The same result is obtained pressing the "Set Parameters"

button in the main crystal_bent window.

Please refer to the information under the HELP

button for a complete description of the parameters. After

pressing the ACCEPT button, the corresponding code starts

running.

Set Defaults: Sets the default parameters.

Takagi-Taupin specific parameters: two items (Set Parameters &

Set Defaults) allows to define the specific parameters

for Takagi-Taupin calculations. They are placed in different

windows to allow a too complicated main window.

Show: Plot, View or Print the results.

Diffraction Curves: display calculated diffraction curves.

Bent Crystal Parameters: shows numerical parameters of the

calculations.

Crystallographic Parameters: shows numerical parameters used

for the calculation of the crystal structure factor.

Takagi-Taupin: Some specific outputs only available if using

the Takagi-Taupin theory.

Integration Network: Shows the integration network with the

electric field corresponding to y=0.

This option is only available when the calculations is

requested in the TAKAGI-TAUPIN/SET PARAMETERS option.

All Reflectivity Curves: Shows an array of diffraction

profiles corresponding, each one, to one knot in the

incident surface.

This option is useful to check that the "window" parameter

is large enough to avoid border effects in the main (central)

diffraction profile.

This option is only available when the calculations is

requested in the TAKAGI-TAUPIN/SET PARAMETERS option.

Help:   Shows the xcrystal_bent help (this text).

COPYRIGHT:

xcrystal_bent  belongs to XOP package and it is distributed within XOP.

PLEASE REFER TO THE XOP COPYRIGHT NOTICE BEFORE USING IT.

CREDITS:

Published calculations made with XOP should refer:

M. Sanchez del Rio and R. J. Dejus "XOP: Recent Developments"

SPIE proceedings vol. 3448, pp.340-345, 1998.

In addition, published calculations with the xcrystal_bent aplication

should also cite:

M. Sanchez del Rio, C. Ferrero and V. Mocella.

"Computer simulations of bent perfect crystal diffraction profiles"

SPIE proceedings vol. 3151, pp.312-323, 1997.

LAST MODIFICATION: msr/msr/99-03-16