----- file created by makedoc on: Thu Jul 20 11:13:09 2000

----- Documentation for /pgexp/applications1/XRayOptics/xop/src_idl/optics/xpower.pro -----

==============================  Xpower   =====================================

Xpower is a widget based graphical interface to calculate the transmission

in terms of energy vs flux or power of a set of filters and mirrors.

It uses the DABAX files to get reflectivities and transmissions.

DESCRIPTION OF THE CONTROLS IN THE MAIN WINDOW:

File:

Xpower input parameters: This option allows to save the current

parameters to a file for later loading. It also allows

to save the current parameters as defaults for being

used when the application is initialized. In the last

case, the file is named "application".xop (where

"application " is the name of the current XOP

application) and is written in the directory pointed

by the XOP_DEFAULTS_DIR environment variable (which

must be set). The parameter file is ASCII and can be

read and edited with care.

Quit: to exit from the program

Set_Parameters:

Set Parameters: to define the parameters for the calculation.

The same result is obtained pressing the "Set Parameters"

button in the main XPower window.

Please refer to the information under the HELP

button for a complete description of the parameters. After

pressing the ACCEPT button, XPower starts running.

If you whish to calculate values for a single energy point

instead of an array of energy points [default], set the energy

minimum equal to the energy maximum. In this case, the

results will be presented in a common text window for all the

three options in the "Show" menu.

Set Defaults: Sets the default parameters.

Show: Produces a graphic of the results (Local properties, cumulated

properties) and shows text parameters. In the case that a single

energy point is calculated (by setting the minimum energy equal

to the maximum energy), the three menu entries will display the

same text parameters.

Help:   Shows the xpower help (this text).

COPYRIGHT:

xpower  belongs to XOP package and it is distributed within XOP.

PLEASE REFER TO THE XOP COPYRIGHT NOTICE BEFORE USING IT.

CREDITS:

Published calculations made with XOP should refer:

M. Sanchez del Rio and R. J. Dejus "XOP: Recent Developments"

SPIE proceedings vol. 3448, pp.340-345, 1998.

LAST MODIFICATION: msr/msr/99-03-16