----- file created by makedoc on: Thu Jul 20 11:12:51 2000

----- Documentation for /pgexp/applications1/XRayOptics/xop/src_idl/dabax/crl_calc_rt.pro -----

NAME:

crl_ray_parabol

PURPOSE:

This function calculate the final position (in the phase space)

and the attenuation of an incoming ray, for a CRL with parabolic

holes of specified dimensions

CATEGORY:

optics

CALLING SEQUENCE:

Result = parabol(X, Xprime, Intens, Aperture, Delta, Nholes, $

Rholed, d, Xmax, Slendist, Lfindist )

INPUTS:

X, Xprime: coordinates in phase space of ray at source (um and urad)

Intens: relative intensity at coordinate X, Xprime (arb. unit)

Aperture: aperture of lens (mm) Rif. ESRF/MACH ID 97/33

Delta: coefficient delta of refr. index (n = 1 - delta - i * beta)

Nholes, Rholes: Number of holes and radius (mm) of curvature of lens

d: minimum distance between holes (mm)

Xmax: half of max lateral dimension of hole (mm)

Slendist, Lfindist: distances source-lens, lens-final plane (plane

of interest) in meters

KEYWORD PARAMETERS:

Group: the id of the called widget (to manage error window).

OUTPUTS:

This function return the coordinates (X1, X1') in the

phase space and the relative intensity of the ray at the final plane

(plane of interest), in form of a 3-tags structure

PROCEDURE:

Calculate the deflection of the ray and the transversal displacement

due to drift-spaces

Reference. P.Elleaume - - NIM A412 (1998) pp 483-506 (formula 9)

EXAMPLE:

out=ray_calc(45,12,.5,.3,5.9e-7,51,.4,.1,1,24,8.2)

(out is the position in phase space and the relative intensity of the

ray at the exit of the lens (at a distance 8.2 meters from it))

MODIFICATION HISTORY:

Written by:	G. Apostolo (apostolo@esrf.fr), 98-3-4

98/12/22 srio@esrf.fr adapts to XOP2.0. Vectorize the

calculation loop. Renamed from crl_Parabol.

NAME:

crl_ray_cyl

PURPOSE:

This function calculate the final position (in the phase space)

and the attenuation of an incoming ray, for a CRL with cylindrical

holes of specified dimensions

CATEGORY:

Optics

CALLING SEQUENCE:

Result = crl_ray_cyl(X, Xprime, Intens, Aperture, Delta, Nholes, $

Rholed, d, Slendist, Lfindist )

INPUTS:

X, Xprime: coordinates in phase space of ray at source (um and urad)

Intens: relative intensity at coordinate X, Xprime (arb. unit)

Aperture: aperture of lens (mm) Rif. ESRF/MACH ID 97/33

Delta: coefficient delta of refr. index (n = 1 - delta - i * beta)

Nholes, Rholes: Number and radius (mm) of lens holes

d: minimum distance between holes (mm)

Slendist, Lfindist: distances source-lens, lens-final plane (plane

of interest) in meters

KEYWORD PARAMETERS:

Group: the id of the called widget (to manage error window).

OUTPUTS:

This function return the coordinates (X1, X1') in the

phase space and the relative intensity of the ray at the final plane

(plane of interest), in form of a 3-tags structure ({x,xp,int})

PROCEDURE:

Calculate the deflection of the ray and the transversal displacement

due to drift-spaces

Reference: P.Elleaume - NIM A412 (1998) pp 483-506 (formula 32)

EXAMPLE:

EXAMPLE1:

out=ray_cyl(45,12,.5,.3,5.9e-7,51,.4,.1,24,8.2)

(out is the position in phase space and the relative intensity of the

ray at the exit of the lens (at a distance 8.2 meters from it))

MODIFICATION HISTORY:

Written by:	G. Apostolo (apostolo@esrf.fr), 98-3-4

98/12/22 srio@esrf.fr adapts for XOP2.0. Renamed from crl_ray_calc.

NAME:

crl_ray_tracing

PURPOSE:

This function ray-traces a source trough a Compound

Refractive Lense (either cylindrical or parabolic).

CATEGORY:

Optics

CALLING SEQUENCE:

Result = crl_ray_tracing(source, Delta, Mu, $

Nholes, Rholed, d, Xmax, Slendist, Lfindist)

INPUTS:

source: a structure with the source (as created by CRL_Tay_Source)

Delta: The delta component of the refraction index of the CRL material.

Mu; attenuation coeficient od the CRL material.

Nholes: Number of lens holes

Rholes: Radius (mm) of lens holes

d: minimum distance between holes (mm)

Xmax: half of max lateral dimension of hole (mm) (only for parabolic

holes, Cil = 0)

Slendist: distance source-lens in meters

Lfindist: distances lens-final plane (plane of interest) in meters

KEYWORD PARAMETERS:

Cil: flag (1 = cylindrical holes (default), 0 = parabolic holes)

OUTPUTS:

a structure of same type as source with the image {z,zp,int}

PROCEDURE:

Calculate the parameters of the lens (Focal distance, Aperture),

generate Nrays rays and calculate the deviation of each ray through

the lens

EXAMPLE:

EXAMPLE1:

space=crl_ray_tracing(.039,2.5,24,1.6,'Be',1.85,51,.4,.1,1,0,0,24,8.2,$

100000,Inttot,1,1)

(generate the flux versus the lateral displacement for a lens in

Beryllium (51 cylindricalholes of radius 0.4 mm) at a distance

8.2 meters from the lens)

MODIFICATION HISTORY:

Written by:	G. Apostolo (apostolo@esrf.fr), 98-3-4

98/05/18 srio@esrf.fr adapts for XOP.

98/12/22 srio@esrf.fr adapts for XOP2.0. Renamed from crl_ray_stat1.

NAME:

crl_calc_rt

PURPOSE:

main interface for performing ray tracing with CRLs

CATEGORY:

Optics

CALLING SEQUENCE:

crl_calc_rt,pointer

INPUTS:

pointer: a pointer to a structure containg the CRL parameters

(as obtained from Dabax_Defaults('crl') )

KEYWORD PARAMETERS:

Group: the id of the called widget (to manage error window).

OUTPUTS:

performs the computation and creates the graph windows.

PROCEDURE:

1) Use dabax to get the refraction index and attenuation coefficient

2) Create the gaussian source using CRL_Ray_Source()

3) Trace the source trough the CRL using CRL_Ray_Tracing, which

calls either CRL_Ray_Cyl (for cylindrical lenses) or

CRL)Ray_Parabol (for parabolic lenses)

4) Calls CRL_Plot1 and/or CRL_Plot2 for creating the graphs

(image profile and image phase-space, respectively).

Reference: P.Elleaume - NIM A412 (1998) pp 483-506

(formula 9 for cylindrical lenses and formula 32 for parabolic ones)

EXAMPLE:

CRL_Calc_RT,Ptr_New(dabax_defaults('crl'))

MODIFICATION HISTORY:

Written by:	M. Sanchez del Rio (srio@esrf.fr), 98-12-22