----- file created by makedoc on: Thu Jul 20 11:13:08 2000

----- Documentation for /pgexp/applications1/XRayOptics/xop/src_idl/optics/xcrystal.pro -----

=================================   xcrystal   ================================

Xcrystal is a widget based graphical interface to calculate diffraction

profiles of perfect and mosaic crystals. It allows Bragg and Laue

geometries and (in the case of perfect crystals) diffracted and

transmitted intensity calculations.  The diffracted intensity can be

calculated as a function of either rock angle orphoton energy.

It uses the DABAX database to define the crystal structure and to

retrieve the scattering factors to build the crystal structure factors.

It interfaces diff_pat, a computer program written by M. Sanchez del Rio

(srio@esrf.fr) derived from the SHADOW application bragg.

DESCRIPTION OF THE CONTROLS IN THE MAIN WINDOW:

File:

Xcrystal input parameters: This option allows to save the current

parameters to a file for later loading. It also allows

to save the current parameters as defaults for being

used when the application is initialized. In the last

case, the file is named "application".xop (where

"application " is the name of the current XOP

application) and is written in the directory pointed

by the XOP_DEFAULTS_DIR environment variable (which

must be set). The parameter file is ASCII and can be

read and edited with care.

Quit: to exit from the program

Set_Parameters:

Set Parameters: to define the parameters for the calculation.

The same result is obtained pressing the "Set Parameters"

button in the main XCrystal window.

Please refer to the information under the HELP

button for a complete description of the parameters. After

pressing the ACCEPT button, diff_pat starts running.

Set Defaults: Sets the default parameters.

Show: Display results

Diffraction Curve: starts a graphical window with the calculation results.

Crystal Parameters: shows output parameters of the calculation.

Crystallographic Parameters: shows numerical values used to build the

crystal structure factor.

Help:   Shows the xcrystal help (this text).

COPYRIGHT:

xcrystal  belongs to XOP package and it is distributed within XOP.

PLEASE REFER TO THE XOP COPYRIGHT NOTICE BEFORE USING IT.

CREDITS:

Published calculations made with XOP should refer:

M. Sanchez del Rio and R. J. Dejus "XOP: Recent Developments"

SPIE proceedings vol. 3448, pp.340-345, 1998.

In addition, published calculations with the xcrystal_bent aplication

should also cite:

M. Sanchez del Rio, C. Ferrero and V. Mocella.

"Computer simulations of bent perfect crystal diffraction profiles"

SPIE proceedings vol. 3151, pp.312-323, 1997.

LAST MODIFICATION: msr/msr/99-03-16