----- file created by makedoc on: Thu Jul 20 11:13:23 2000

----- Documentation for /pgexp/applications1/XRayOptics/xop/src_idl/xopm/xop.pro -----

===============================  Xop   ========================================

XOP is a widget based interface which drives different programs that

calculates the synchrotron radiation source spectra

(bending magnet, wigglers and undulators) and the reflection and

transmission characteristics of optical elements as:

mirror, filters, flat crystals, bent perfect crystals, and multilayers.

XOP also contains programs for multi-purpose data analysis and

visualization.

XOP runs under most Unix machines and Windows (95/98 and NT).

The point-and-click interface is used as a driver for a variety

of codes from different authors written in different computer languages.

The XOP interface and many of its applications are written in IDL.

IDL is a programming language from  Research Systems:

Email: info@rsinc.com  WWW: http://www.rsinc.com/

Phone: (1) (303) 786-9900 Fax: (1) (303) 786-9909

The XOP interface largely simplifies the use of the codes and may be used

without prior knowledge of how to run a particular program.

XOP includes a flexible DAta BAse for X-ray applications (DABAX), which is a

compilations of tables for x-ray applications with a collection of codes

to access, visualize and process these tables.

DESCRIPTION OF THE MAIN WINDOW CONTROLS:

========================================

The main XOP window contains several entries in the menu-bar that are

starting points for the differents applications included in the interface.

The menus are fully customizable, thus the final aspect may vary from

one XOP installation to another. (See the file README_HOWTO in the

main XOP directory.)

The recommended menus (found in most installations) are:

FILE: (with the  EXIT entry to quit the program).

SOURCE: include programs for spectral calculation of synchrotron

radiation sources (undulator, wiggler and bending magnets).

OPTICS: include programs that simulate x-ray interaction with

materials and optical elements. Three groups are available:

TABLES: group of programs for x-ray cross sections, optical

constants, attenuation coefficients of materials. It includes

entries to all DABAX applications.

MIRRORS & FILTERS: Programs to calculate mirror reflectivities and

filter transmissions and to apply them to a source.

CRYSTALS: group of programs for crystal optics.

MULTILAYERS: group of programs for multilayer properties.

OBSOLETE: include programs already obsoleted by other xop

applications but they are kept for occasional use and

comparison with the new applications.

TOOLS: different tools available on-line.

HELP: Access to XOP documentation.

GENERAL INFORMATION:

====================

The application driven by Xop are very numerous and in few cases redundant.

The xop autors recommend and maintain only few of them. Nevertheless we may

keep in the interface of all the codes that have been used with xop (see

the OBSOLETED menu).

XOP and DABAX are fully extensible, and it is possible to create and

integrate new applications.

XOP is written in IDL and is supported for Unix and PC platforms.

XOP can be run in two modes:

Embedded mode: the default. The user has no access to the source code

and the IDL license is integrated (embedded) in XOP. XOP

is thus presented as a standalone application not requiring

any additional package or library.

Developer mode: for users that have a valid IDL license. The IDL code

is compiled "on the fly". Users have access to the IDL

code.

For more information read the README files in the main XOP directory

and consult the XOP man page under Unix.

COPYRIGHT:

==========

Please refer to the README_COPYRIGHT file in the manin XOP directory.

CREDITS:

========

Published calculations made with XOP should refer at least one of the

following publications:

M. Sanchez del Rio and R. J. Dejus "XOP: Recent Developments"

SPIE proceedings vol. 3448, 1998.

M. Sanchez del Rio and R. J. Dejus "XOP: A Multiplatform Graphical

User Interface for Synchrotron Radiation Spectral and Optics

Calculations" SPIE proceedings vol. 3152, pags. 148-157, 1997.

R.J. Dejus and M. Sanchez del Rio "XOP: A Graphical User Interface

for Spectral Calculations and X-Ray Optics Utilities"

Rev.  Sci. Instrum. 67(9) 1996 (Proceedings of the Conference on

Synchrotron Radiation Instrumentation '95).

LAST MODIFICATION: current version is XOP 2.0