----- file created by makedoc on: Thu Jul 20 11:13:13 2000

----- Documentation for /pgexp/applications1/XRayOptics/xop/src_idl/sources/xwiggler.pro -----

==============================  Xwiggler   ====================================

Xwiggler is a widget based graphical interface to calculate

the spectrum of a Wiggler Synchrotron Radiation source.

The Flux and Spectral Power values calculated with XWiggler

correspond to the integration in FULL vertical and horizontal

emission angles.

XWiggler interfaces the program wiggler_spectrum. This program has been

developed in the framework of the ray-tracing package SHADOW.

DESCRIPTION OF THE CONTROLS IN THE MAIN WINDOW:

File:

XWiggler input parameters: This option allows to save the current

parameters to a file for later loading. It also allows

to save the current parameters as defaults for being

used when the application is initialized. In the last

case, the file is named "application".xop (where

"application " is the name of the current XOP

application) and is written in the directory pointed

by the XOP_DEFAULTS_DIR environment variable (which

must be set). The parameter file is ASCII and can be

read and edited with care.

Write Files for XOP/Optics: Saves the current undulator

spectrum in the SRCOMPE (flux) and SRCOMPW (Power) files

to be used for the other XOP application from Optics

menu.

Quit: to exit from the program

Set_Parameters:

K(gap) Tool: This tool permits to calculate the deflection

parameter K as a function of the magnet gap.

Set Parameters: to define the parameters for the calculation.

The same result is obtained pressing the "Set Parameters"

button in the main XWiggler window.

Please refer to the information under the HELP

button for a complete description of the parameters. After

pressing the ACCEPT button, wiggler_spectrum starts running.

Set Defaults: Sets the default parameters.

Show: Display results

Spectra: Plots the calculated wiggler spectra.

Trajectories: Plots the calculated electron trajectories.

Parameters: Show the input parameters and some calculated values.

Help:   Shows the XWiggler help (this text).

COPYRIGHT:

xwiggler  belongs to XOP package and it is distributed within XOP.

PLEASE REFER TO THE XOP COPYRIGHT NOTICE BEFORE USING IT.

CREDITS:

Published calculations made with XOP should refer:

M. Sanchez del Rio and R. J. Dejus "XOP: Recent Developments"

SPIE proceedings vol. 3448, pp.340-345, 1998.

In addition, published calculations using the XWIGGLER aplication

should also cite:

C. Welnak, G.J. Chen and F. Cerrina, "SHADOW: a synchrotron

radiation X-ray Optics simulation tool",

Nucl. Instr. and Meth. in Phys. Res. A347 91994) 344

or

SHADOW WEB Page: http://www.xraylith.wisc.edu/shadow/shadow.html

LAST MODIFICATION: msr/msr/99-03-16