=================================  XnDeBe      ============================

XnDeBe is a widget based graphical interface to calculate Neutron

Scattering Factors, and their derived parameters

(refraction index, absorption cross section, reflectivity).

The data are taken from the DABAX-DABAN data base.

Possible calculations:

br [fm] (real part of the bound coherent scattering length)

bi [fm] (imaginary part of the bound coh. sca. length)

ScaXSec [barns] (Coherent scattering cross section)

delta [n=1-delta-i beta]

beta [n=1-delta-i beta]

Linear abs coeff (mu [cm^-1)]

Mass  abs coeff (mu [cm^2/g)]

Absorption Cross Section [barns]

Refractive index

Reflectivity

Most of these calculations can be made for unpolarized or polarized

(Up(+1/2) and Down(-1/2)) neutrons.

CUSTOMIZATION OF XnDeBe DABAN-DABAX INPUT PARAMETERS:

-If you want to add(remove) another DeBe* file (datafile in spec

format), just add(remove) it in any(all) directory of $DABAX_PATH.

Please consider that:

> In order to facilitate the fast access, the first time the

new file is called, a new file <file>.HIdl is created which

contains the files indeces. If you do not have write permission

on that directory, the file is not created, and the access is

much slower.

> Under Windows95/NT, a file containing the list of the files

MUST exist and is called "dabax.lis". This file must be updated

when adding new files. The file "dabax.lis" is optional under

UNIX. It may be used to define the order in which the files are

presented in the menus.

-If you want to modify the mixture table  list, do the following:

a) copy the Compounds.dat DABAX file to a given directory

(e.g., current directory ".")

b) Modify this file to add/remove/change the entries.

c) Redefine $DABAX_PATH to include your new directoty:

setenv  DABAX_PATH = .:$DABAX_PATH

d) Restart the application.

Please note that under Windows, the DABAX_PATH only can hold one

directory, so all changes must be done on the original DABAX

directory.

DESCRIPTION OF THE CONTROLS IN THE MAIN WINDOW:

File:

XnDeBe input parameters: This option allows to save the current

parameters to a file for later loading. It also allows

to save the current parameters as defaults for being

used when the application is initialized. In the last

case, the file is named "application".xop (where

"application " is the name of the current NOP

application) and is written in the directory pointed

by the XOP_DEFAULTS_DIR environment variable (which

must be set). The parameter file is ASCII and can be

read and edited with care.

Quit: to exit from the program

Set_Parameters:

Set Parameters: to define the parameters for the calculation.

The same result is obtained pressing the "Set Parameters"

button in the main XnDeBe window.

Please refer to the information under the HELP

button for a complete description of the parameters. After

pressing the ACCEPT button, XnDeBe start running and

presents a graphic display with results.

Set Defaults: Sets the default parameters.

Help:   Shows the XnDeBe help (this text).

COPYRIGHT:

XnDeBe  belongs to XOP/NOP package and it is distributed within NOP.

PLEASE REFER TO THE XOP/NOP COPYRIGHT NOTICE BEFORE USING IT.

CREDITS:

Published calculations made with XOP/NOP should refer:

M. Sanchez del Rio and R. J. Dejus "XOP: Recent Developments"

SPIE proceedings vol. 3448, pp.340-345, 1998.

LAST MODIFICATION: cb/cb/2001-11-15