----- file created by makedoc on: Thu Jul 20 14:46:21 2000

----- Documentation for /pgexp/applications1/XRayOptics/shadowvui/vui/xsh_wiggler_spectrum.pro -----

======================   XSh_Wiggler_Spectrum    ==============================

XSh_Wiggler_Spectrum is an widget based graphical interface to calculate

the spectrum of a Wiggler Synchrotron Radiation source.

It interfaces the program wiggler_spectrum. This program belongs to

the ray-tracing package SHADOW.

DESCRIPTION OF THE MAIN WINDOW CONTROLS:

FILE:

LOAD  XSH_WIGGLER_SPECTRUM INPUT FILE: Load  xsh_wiggler_spectrum

inputs from a binary file.

WRITE  XSH_WIGGLER_SPECTRUM  INPUT FILE: Writes the current inputs

to a binary file. This file can be loaded with the previously

described option.

WRITE FILES FOR Transmit: Saves the selected result file in

the Flux file XSH_SRCOMPE and the Power file XSH_SRCOMPW.

QUIT:     Exit the interface program and deletes temporary files.

SET_PARAMETERS:

K(Gap) Tool:  Opens the K(Gap) tool application that permits to

calculate the K value as a function of the wiggler gap.

Set Paramaters:  Opend the main window for parameter definition.

The effect of this menu option is identical to pressing

the "Set Parameters: button in the main xsh_wiggler_spectrum

window.

Set Defaults:  Initializes all the parameters to their default

values.

SHOW:       Plot Results (trajectories and Spectra) or Show Result

Parameters.

HELP:       Shows the xsh_wiggler_spectrum help text.

CUSTOMIZATION OF XSH_WIGGLER_SPECTRUM INPUT PARAMETERS:

The input parameters of xsh_wiggler_spectrum are hard-coded in the file

xsh_defaults.pro which, in general, is not available for

user-modifications.

It is possible to customize the default values by adding/changing

entries in the .xsh_defaults file.

Two copies of .xsh_defaults are sequentially read by the application:

the first one, sitting in $XSH_HOME, is the "installation" copy,

and the purpose is to place the defaults of the particular xop

installation. The system-administrator should do that.

The second .xsh_defaults file is placed in $HOME (user's home

directory). The users should place his/her defaults here and

will only affect his xop session.

If you are running XSH on a PC, the file to customize is

the "installation" one: %XSH_HOME%\.xsh_defaults

The entries in .xsh_defaults concerning the xsh_wiggler_spectrum

application are described below. Customize ONLY the ones that differ

from the default. This will save initialization time.

xsh_wiggler_spectrum.parameters.field='0'

; 0=Sinusoidal, 1=B from file,

; 2=B from harmonics

xsh_wiggler_spectrum.parameters.Nperiods=12

xsh_wiggler_spectrum.parameters.Ulambda=0.125

xsh_wiggler_spectrum.parameters.K= 14.

xsh_wiggler_spectrum.parameters.Energy= 6.04

xsh_wiggler_spectrum.parameters.Phot_energy_min= 100.0

xsh_wiggler_spectrum.parameters.Phot_energy_max= 100100.0

xsh_wiggler_spectrum.parameters.Phot_energy_step= 10000.0

xsh_wiggler_spectrum.parameters.Current= 100.

xsh_wiggler_spectrum.parameters.File: 'datafile'

xsh_wiggler_spectrum.kgap_tool.Br:1.57     ; K(GAP) Tool parameters.

xsh_wiggler_spectrum.kgap_tool.Bp:!pi

COPYRIGHT:

xsh_wiggler_spectrum (version 1.2) belongs to XSH package and it is

distributed within XSH.

PLEASE REFER TO THE XSH COPYRIGHT NOTICE BEFORE USING IT.

LAST MODIFICATION: msr/msr/97-12-05