SHADOW Namelist Documentation

This file contains a listing of the source and optical element namelists.

In the absence of a MENU mode, it will aid in single parameter changing

with the editor.  It is not intended to be a method to setup a source or

system.  For more complete definitions of source characteristics and

parameters, please see the User's Guide.

Variables preceded by `f' are typically flags indicating yes/no, or

multiple choice answers stored in the form of an integer.  Units of length

should be consistent between the source and optical elements and can be

anything unless otherwise specified.

Source Namelist:

$tsour

fdistr	=             2 - defines source angle distribution types:

Available options are: flat(1),uniform(2),

gaussian(3), synchrotron(4), conical(5), exact

synchrotron(6).

fgrid	=             0 - defines source modelling type in space and

momentum.  Options are (SPACE/MOMENTUM) :

RANDOM/RANDOM (0), GRID/GRID (1), GRID/RANDOM

(2), RANDOM/GRID (3), ELLIPSE in phase/RANDOM on

the ellipse (4), ELLIPSE in phase/GRID on the

ellipse (5).

fsour	=             0 - spatial source type/shape in X-Z plane.  Options

are: point (0), rectangle (1), ellipse (2),

gaussian (3).

fsource_depth	=             1 - defines the source depth (Y).  Options

are:  no depth (1), flat depth (2),

gaussian (3), synchrotron depth (4).

f_coher	=             0 - if generating the A vectors, defines

whether the light is incoherent (0), or

coherent (1). (f_polar = 1)

f_color	=             1 - photon energy distribution type.  Options

are:  single energy (1), mutliple

discrete energies, up to 10 energies

(2), uniform energy distribution (3).

f_phot	=             0 - defines whether the photon energy will be

specified in eV (0) or Angstroms (1).

f_pol	=             0 - for synchrotron and wiggler sources defines the

polarization component of interest: parallel (1),

perpendicular (2), total (3).

f_polar	=             1 - flag defining whether or not to generate

the A vectors:  yes (1), no (0).

f_opd	=             0 - flag defining whether or not to save the optical

paths (OPD):  yes (1), no (0).

f_wiggler	=             0 - source type.  Options:  regular/bending

magnet/synchrotron (0), wiggler (1),

undulator (2).

f_bound_sour	=             0 - flag defining whether or not to optimize

the source:  yes (1), no (0).

f_sr_type	=             0 - for synchrotron sources, distribution in

terms of: photons (0), or power

distribution (1). (fdistr=4,6)

istar1	=        478291 - seed for random number generator, odd.

npoint	=          5000 - number of random rays (0-5000).

ncol	=             0 - source generation routines will fill in the

number of columns in your source.

n_circle	=             0 - for fgrid=1,3 and fdistr=5; number of

grid points along each circle.

n_color	=             0 - for f_color=2, number of discrete lines

in energy, maximum = 10.

n_cone	=             0 - for fgrid=1,3 and fdistr=5; number of grid

points along the radius(number of concentric

circles).

ido_vx	=             1 - for fgrid=1,3; number of grid points in

horizontal angle distribution.

ido_vz	=             1 - for fgrid=1,3; number of grid points in vertical

angle distribution.

ido_x_s	=             1 - for fgrid=1,2; fsour=1 points along x,

fsour=2 number of concentric ellipses.

ido_y_s	=             1 - for fgrid=1,2; number of points along

depth (Y).

ido_z_s	=             1 - for fgrid=1,2; fsour=1 points along z,

fsour=2 number of concentric ellipses.

ido_xl	=             0 - for fgrid=4,5; number of sigma levels in X for

phase space ellipse source.

ido_xn	=             0 - for fgrid=5; number of rays/sigma level in X

ido_zl	=             0 - for fgrid=4,5; number of sigma levels in X for

phase space ellipse source.

ido_zn	=             0 - for fgrid=5; number of rays/sigma level in Z.

sigxl1	=  0.0000000000000000E+00 - for fgrid=4,5; values for sigma

sigxl2	=  0.0000000000000000E+00   levels in X.

sigxl3	=  0.0000000000000000E+00

sigxl4	=  0.0000000000000000E+00

sigxl5	=  0.0000000000000000E+00

sigxl6	=  0.0000000000000000E+00

sigxl7	=  0.0000000000000000E+00

sigxl8	=  0.0000000000000000E+00

sigxl9	=  0.0000000000000000E+00

sigxl10	=  0.0000000000000000E+00

sigzl1	=  0.0000000000000000E+00 - for fgrid=4,5; values for sigma

sigzl2	=  0.0000000000000000E+00   levels in Z.

sigzl3	=  0.0000000000000000E+00

sigzl4	=  0.0000000000000000E+00

sigzl5	=  0.0000000000000000E+00

sigzl6	=  0.0000000000000000E+00

sigzl7	=  0.0000000000000000E+00

sigzl8	=  0.0000000000000000E+00

sigzl9	=  0.0000000000000000E+00

sigzl10	=  0.0000000000000000E+00

conv_fact	=  0.0000000000000000E+00 - for fwiggler=1; conversion

factor from meters to user units.

cone_max	=  0.0000000000000000E+00 - for fdistr=5; maximum half

divergence. (radians)

cone_min	=  0.0000000000000000E+00 - for fdistr=5; minimum half

divergence. (radians)

epsi_dx	=  0.0000000000000000E+00 - for fdistr=4,6 or fwiggler=1;

in X, the distance from the waist

that the emittance value

corresponds to, signed.

epsi_dz	=  0.0000000000000000E+00 - for fdistr=4,6 or fwiggler=1;

in Z, the distance from the

waist that the emittance value

corresponds to, signed.

epsi_x	=  0.0000000000000000E+00 - for fdistr=4,6 or fwiggler=1;  the

beam emittance in units of radians*length

units used so far. (in X)

epsi_z	=  0.0000000000000000E+00 - for fdistr=4,6 or fwiggler=1; the

beam emittance (in Z) in units of

radians*length units used so far.

hdiv1	=  0.0000000000000000E+00 - horizontal divergence in +X (radians).

hdiv2	=  0.0000000000000000E+00 - horizontal divergence in -X (radians).

ph1	=  11160.00000000000      - photon energy:  f_color=1 desired

energy, f_color=2 first energy line,

f_color=3 minimum energy.

ph2	=  0.0000000000000000E+00 - photon energy: f_color=2 second energy

line, f_color=3 maximum energy.

ph3	=  0.0000000000000000E+00 - photon energy: f_color=2 third energy

line. (and so on for ph4 through

ph10).

ph4	=  0.0000000000000000E+00

ph5	=  0.0000000000000000E+00

ph6	=  0.0000000000000000E+00

ph7	=  0.0000000000000000E+00

ph8	=  0.0000000000000000E+00

ph9	=  0.0000000000000000E+00

ph10	=  0.0000000000000000E+00

bener	=  0.0000000000000000E+00 - for fdistr=4,6 or f_wiggler=1;

Electron beam energy (GeV).

pol_angle	=  0.0000000000000000E+00 - for f_polar=1; phase

difference in degrees.

pol_deg	=  0.0000000000000000E+00 - for f_polar=1; degree of

polarization (between 0 and 1).

r_aladdin	=  0.0000000000000000E+00 - for fdistr=4,6 or f_wiggler=1;

bending magnet radius in units

of length used for source

size, CCW rings negative.

r_magnet	=  0.0000000000000000E+00 - for fdistr=4,6 or f_wiggler=1;

bending magnet radius (m).

sigdix	=  0.0000000000000000E+00 - for fdistr=3; sigma (radians) for

horizontal divergence (gaussian angle

distribution).

sigdiz	=  0.0000000000000000E+00 - for fdistr=3; sigma (radians) for

vertical divergence (gaussian angle

distribution).

sigmax	=  0.0000000000000000E+00 - for fsour=3; sigma in X

sigmay	=  0.0000000000000000E+00 - for fsource_depth=3; sigma in Y

sigmaz	=  0.0000000000000000E+00 - for fsour=3; sigma in Z

vdiv1	=  6.0000000000000002E-05 - vertical divergence in +Z (radians).

vdiv2	=  6.0000000000000002E-05 - vertical divergence in -Z (radians).

wxsou	=  0.0000000000000000E+00 - for fsour=1,2; source width (X).

wysou	=  0.0000000000000000E+00 - for fsource_depth=2; source depth (Y).

wzsou	=  0.0000000000000000E+00 - for fsour=1,2; source height (Z).

file_traj	=  		  - for fwiggler=1,2; filename containing

''		    the electron trajectoryfor wigglers or

the CDF's for undulators. (from

make_id).

file_source	=  		  - filled in by source generation.

''

file_bound	=  		  - for f_bound_sour=1; file containing

''		    the output of reflag and histo3.

oe_number	=             0   - the last variables are set in the

idummy	=             0		    source generation and recorded in

dummy	=  0.0000000000000000E+00   END.00 file.  They should be left

f_new	=             0             alone.

$end

Optical Element Namelist:

$totoe1

fmirr	=             5 - mirror surface (3-d): spherical (1),

elliptical (2), toroidal (3), parabolical (4),

plane (5), codling slit (6), hyperbolical (7),

cone (8), polynomial (9).

f_torus	=             0 - for fmirr=3; mirror pole location:

lower/outer (concave/concave) (0)

(standard toroid),

lower/inner (concave/convex) (1),

upper/inner (convex/concave) (2),

upper/outer (convex/convex) (3).

fcyl	=             0 - flag for cylindrical mirror (1), regular mirror

(0).

f_ext	=             0 - flag for internal/SHADOW calculated (0)

parameters vs.  external/user defined

parameters (1).  For fmirr=8, f_ext=1.

fstat	=             0 - flag to move the source; yes(1), no(0).

f_screen	=             0 - flag: any screens? yes(1), no (0).

f_plate	=             0 - flag: if n_plates >= 1; image plates

orthogonal to optical axis. yes(1),

no(0).  All plates will have the same

orientation.

fslit	=             0 - flag: slit at continuation plane -- yes(1),

no(0).

fwrite	=             0 - files to save option: all files (0), mirror file

only -- mirr (1), image file only -- star (2), none

(3).

f_ripple	=             0 - flag: distorted surface - yes(1), no(0).

f_move	=             0 - flag: move mirror - yes (1), no (0).

f_thick	=             0 - for multilayers: does the thickness of

the multilayer vary as the cosine of the

angle from the pole of the mirror

- yes (1), no (0).

f_bragg_a	=             0 - flag: is the crystal asymmetric - yes

(1), no (0).

f_g_s	=             0 - for f_ripple=1 - type of distortion: sinusoidal

(0), gaussian (1), external spline (2), surface

roughness (3).

f_r_ran	=             0 - empty - read in from gaussian ripple

file in OE generation.

f_grating	=             0 - flag: is it a grating - yes (1), no (0),

if fzp=1, f_grating=1.

f_mosaic	=	      1 - if f_crystal=1; flag: mosaic crystal -

yes (1), no (0).

f_johansson	=	      0 - if f_crystal=1; flag: johansson geometry

- yes (1), no (0).

f_side	=             0 - for fmirr=4: focus location at image (0) or

source (1).

f_central	=             1 - flag: autotuning of grating or crystal -

yes (1), no (0).

f_convex	=             0 - flag: mirror surface concave (0) or

convex (1).

f_reflec	=             1 - reflectivity of surface: no reflectivity

dependence (0), full polarization

dependence (1), no polarization

dependence / scalar (2).

f_rul_abs	=             0 - for f_ruling=5: is the line density

absolute (0) or signed (1) from the

origin.

f_ruling	=             0 - for f_grating=1 - ruling type: constant

on X-Y plane (0), constant on mirror

surface (1), holographic (2), fan type

(3), reserved (4), polynomial line

density (5).

f_pw	=             2 - for f_ruling=2 - recording source types: both

spherical (0), source plane wave, image

spherical (1), source spherical, image plane

wave (2), both plane (3).

f_pw_c	=             0 - for f_ruling=2 - recording sources cylindrical:

none cylindrical (0), source cylindrical, image

spherical (1), source spherical, image

cylindrical (2), both cylindrical (3).

f_virtual	=             0 - for f_ruling=2 - source type

real/virtual: entrance slit real, exit

slit real (0), entrance slit real, exit

slit virtual (1), entrance slit virtual,

exit slit real (2), both virtual (3).

fshape	=             0 - for fhit_c=1: mirror shape rectangular (0), full

ellipse (1), ellipse with hole (2).

fhit_c	=             0 - flag: mirror dimensions finite: yes (1), no(0).

f_mono	=             0 - f_grating,f_central=1 - monochromator mount type:

TGM/Seya (0), ERG (1), Constant Incidence Angle (2),

Constant diffraction angle (3), Hunter (4).

f_refrac	=             0 - flag: reflector (0) or refractor (1).

f_default	=             0 - flag: focii coincident with continuation

plane - yes (1), no (0).

f_refl	=             0 - for f_reflec=1,2 - source of optical constants:

file generated by PREREFL (0), keyboard (1),

multilayers (2).

f_hunt	=             1 - for f_mono=4: first (1) or second (2) grating.

f_crystal	=             1 - flag: crystal -- yes (1), no (0).

f_phot_cent	=             0 - for f_central=1: tune to eV(0) or

Angstroms (1).

npoint	=             0 - filled in by OE generation.

ncol	=             0 - filled in by OE generation.

scr_number	=             0,            0,            0,            0,

0,            0,            0,            0,

0,            0

n_screen	=             0 - for f_screen=1: number of screens.

i_screen	=             0,            0,            0,            0,

0,            0,            0,            0,

0,            0 - for each screen, is it

before (1) or after (0) the mirror.

i_abs	=   0,            0,            0,            0,            0,

0,            0,            0,            0,            0

- for each screen, include absorption yes(1), no (0).

i_stop	=   0,            0,            0,            0,            0,

0,            0,            0,            0,            0

- for each screen with stop, obstruction (0), or aperture

(1).

i_slit	=   0,            0,            0,            0,            0,

0,            0,            0,            0,            0

- for each screen, is there an aperture stop yes(1), no (0).

k_slit	=   0,            0,            0,            0,            0,

0,            0,            0,            0,            0

- for each stop/slit, specify a shape: rectangular (0),

ellipse (1), external (2).  (at this point the external shape

is not yet implemented.)

istar1	=      12345701

cil_ang	=  0.0000000000000000E+00 - if fcyl=1: angle (degrees) of

cylinder axis from X axis.

ell_the	=  0.0000000000000000E+00 - for f_ext=1, fmirr=2: CCW

angle between semi-major axis

and mirror pole.

for f_ext=1, fmirr=7: angle

between major axis and the

vector from the hyperbola

center to the mirror pole.

n_plates	=             0 - number of extra image plates

ig_seed	=             0 - read in from gaussian ripple file in OE

generation.

mosaic_seed	=	8923471 - random number seed for mosaic crystal

calculations.

alpha	=  0.0000000000000000E+00 - mirror orientation angle (degrees).

ssour	=  0.0000000000000000E+00 - for f_default=0: distance from object

focus to the mirror pole.

theta	=  0.0000000000000000E+00 - for f_default=0: incidence angle

(degrees).

simag	=  0.0000000000000000E+00 - for f_default=0: distance from mirror

pole to image focus.

rdsour	=  0.0000000000000000E+00 - for fstat=1: source distance from

mirror pole.

rtheta	=  0.0000000000000000E+00 - for fstat=1: incidence angle

(degrees).

off_soux	=  0.0000000000000000E+00 - for fstat=1: offset in X in

Mirror Reference Frame (MRF).

off_souy	=  0.0000000000000000E+00 - for fstat=1: offset in Y in

MRF.

off_souz	=  0.0000000000000000E+00 - for fstat=1: offset in Z in

MRF.

alpha_s	=  0.0000000000000000E+00 - for fstat=1: source rotation

around Z (CCW +).

rlen1	=  0.0000000000000000E+00 - fshape=1: mirror half length +Y.

fshape=3: internal minor axis (Y).

rlen2	=  0.0000000000000000E+00 - fshape=1: mirror half length -Y.

fshape=2,3: external outline minor

axis (Y).

rmirr	=  0.0000000000000000E+00 - f_ext=1, fmirr=1: spherical mirror

radius.

axmaj	=  0.0000000000000000E+00 - f_ext=1, fmirr=2,7: semi-major axis

axmin	=  0.0000000000000000E+00 - f_ext=1, fmirr=2,7: semi-minor axis

cone_a	=  0.0000000000000000E+00 - f_ext=1, fmirr=8: cone half opening

(degrees).

r_maj	=  0.0000000000000000E+00 - f_ext=1, fmirr=3: tangential (major)

axis.

r_min	=  0.0000000000000000E+00 - f_ext=1, fmirr=3: sagittal (minor)

axis.

rwidx1	=  0.0000000000000000E+00 - fshape=1: mirror half width +X.

fshape=3: internal major axis (X).

rwidx2	=  0.0000000000000000E+00 - fshape=1: mirror half width -X.

fshape=2,3: external outline major

axis (X).

param	=  0.0000000000000000E+00 - f_ext=1, fmirr=4: parabola parameter P

of Y^2 = 2 P X.

hunt_h	=  0.0000000000000000E+00 - f_mono=4: monochromator height

(distance between beams.)

hunt_l	=  0.0000000000000000E+00 - f_mono=4: monochromator length.

blaze	=  0.0000000000000000E+00 - f_mono=4: blaze length (degrees).

ruling	=  0.0000000000000000E+00 - f_ruling=0,1: ruling density in l/cm.

f_ruling=3: ruling density at center

l/cm.

f_ruling=5: ruling density at the

origin, i.e. a0 of polynomial.

order	=  0.0000000000000000E+00 - diffraction order, negative inside

(European convention).

phot_cent	=  11160.00000000000 - for f_phot_cent=1: photon energy

(eV) to autotune grating/crystal to.

x_rot	=  0.0000000000000000E+00 - f_move=1: rotation about X axis

(degrees, +CW).

d_spacing	=  0.0000000000000000E+00 - calculated.

a_bragg	=  0.0000000000000000E+00 - f_bragg_a=1: angle between

crystal planes and surface.

spread_mos	=  0.400000000000000 - f_mosaic=1: mosaic spread FWHM

(degrees).

thickness	=  0.100000000000000 - f_mosaic=1: crystal thickness cm.

r_johansson	=  0.000000000000000E+00 - f_ext=1: johansson radius.

y_rot	=  0.0000000000000000E+00 - f_move=1: rotation about Y axis

(degrees, +CW).

z_rot	=  0.0000000000000000E+00 - f_move=1: rotation about Z axis

(degrees, +CW).

offx	=  0.0000000000000000E+00 - f_move=1: mirror offset in X (MRF).

offy	=  0.0000000000000000E+00 - f_move=1: mirror offset in Y (MRF).

offz	=  0.0000000000000000E+00 - f_move=1: mirror offset in Z (MRF).

sllen	=  0.0000000000000000E+00 - fslit=1: slit length (sagittal, X).

slwid	=  0.0000000000000000E+00 - fslit=1: slit width (dispersion, Z).

sltilt	=  0.0000000000000000E+00 - fslit=1: slit tilt (+CCW, degrees).

cod_len	=  0.0000000000000000E+00 - fmirr=6: codling slit length

(sagittal).

cod_wid	=  0.0000000000000000E+00 - fmirr=6: codling slit width

(effective, tangential).

$end

$totoe2

x_sour	=  0.0000000000000000E+00 - fstat=1: source offset in X (source

reference frame -SRF).

y_sour	=  0.0000000000000000E+00 - fstat=1: source offset in Y (SRF).

z_sour	=  0.0000000000000000E+00 - fstat=1: source offset in Z (SRF).

x_sour_rot	=  0.0000000000000000E+00 - fstat=1: source rotation about

X (SRF).

y_sour_rot	=  0.0000000000000000E+00 - fstat=1: source rotation about

Y (SRF).

z_sour_rot	=  0.0000000000000000E+00 - fstat=1: source rotation about

Z (SRF).

r_lambda	=  0.0000000000000000E+00 - f_phot_cent=1: Angstroms to

autotune grating/crystal to.

theta_i	=  0.0000000000000000E+00 - f_plate=1: elevation (degrees)

Y=0 plane rotated about the X

axis, +CCW.

alpha_i	=  0.0000000000000000E+00 - f_plate=1: azimuth (degrees)

Y=0 plane rotated about the Z

axis, +CCW.

t_incidence	=  10.00000000000000 - incidence angle (degrees), defining

optical axis.  (measured down from

the mirror normal.)

t_source	=  5000.000000000000 - source plane distance

t_image	=  150.0000000000000 - image plane distance

t_reflection	=  10.00000000000000 - reflection angle (degrees).

file_source	=

'BEGIN.DAT        - for first OE, filename of SOURCE

'		       array.  if unchanged, BEGIN.DAT.

file_rip	=

'NONE SPECIFIED   - for f_ripple=1, f_g_s=2: file

containing the external spline of

the surface, for f_g_s=3: file

'		       containing the power spectral

density function.

file_refl	=

'GAAS.SHA         - for f_crystal=1: file containing

'		       the crystal parameters.

for f_reflec=1,2 and f_refl=0: file

with optical constants.

for f_reflec=1,2 and f_refl=2: file

with thicknesses and refractive

indices for multilayers.

file_abs	=

'NONE SPECIFIED   - for each screen with absorption,

',		       file containing optical constants.

'NONE SPECIFIED                                                                  ',

'NONE SPECIFIED                                                                  ',

'NONE SPECIFIED                                                                  ',

'NONE SPECIFIED                                                                  ',

'NONE SPECIFIED                                                                  ',

'NONE SPECIFIED                                                                  ',

'NONE SPECIFIED                                                                  ',

'NONE SPECIFIED                                                                  ',

'NONE SPECIFIED                                                                  '

file_mir	=

'NONE SPECIFIED   - for fmirr=9(polynomial): file

'		       containing the polynomial description.

thick	=  0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00

- for each screen including absorption, thickness of the

film in cm.

rx_slit	=  0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00

- for each slit/stop in a screen, dimension in X.

d_plate	=  0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00

- for n_plates>=1: for each plate, the distance from the

mirror center to the plate.

rz_slit	=  0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00

- for each slit/stop in a screen, dimension in Z.

sl_dis	=  0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00,

0.0000000000000000E+00, 0.0000000000000000E+00

- for each screen, absolute distance from the mirror.

fzp	=             0 - flag: Fresnel Zone Plate -- yes(1), no(0).  set

f_grating=1.

holo_r1	=  0.0000000000000000E+00 - f_ruling=2: defining the

creation of the hologram:

entrance slit distance (cm).

holo_r2	=  0.0000000000000000E+00 - f_ruling=2: exit slit distance

(cm).

holo_del	=  0.0000000000000000E+00 - f_ruling=2: entrance slit

incidence angle (degrees).

holo_gam	=  0.0000000000000000E+00 - f_ruling=2: exit slit

incidence angle (degrees).

holo_w	=  0.0000000000000000E+00 - f_ruling=2: recording wavelength

(angstroms).

holo_rt1	=  0.0000000000000000E+00 - f_ruling=2: entrance slit

rotation angle (degrees).

holo_rt2	=  0.0000000000000000E+00 - f_ruling=2: exit slit rotation

angle (degrees).

azim_fan	=  0.0000000000000000E+00 - f_ruling=3: position of

ruling focus on mirror plane.

angle from Y, degrees, +CCW.

dist_fan	=  0.0000000000000000E+00 - f_ruling=3: distance from

grating center (cm).

coma_fac	=  0.0000000000000000E+00 - f_ruling=3: coma correction

factor, 0 if none.

alfa	=  0.0000000000000000E+00 - f_refl=1: alpha of the complex

dielectric constant defined by:

epsilon = (1-alpha)+i gamma

gamma	=  0.0000000000000000E+00 - f_refl=1: gamma of the above

equation.

r_ind_obj	=  1.000000000000000 - f_refrac=1: index of refraction in

object space.

r_ind_ima	=  1.000000000000000 - f_refrac=1: index of refraction in

image space.

rul_a1	=  0.0000000000000000E+00 - f_ruling=5: a1 coefficient of

polynomial.

rul_a2	=  0.0000000000000000E+00 - f_ruling=5: a2 coefficient of

polynomial.

rul_a3	=  0.0000000000000000E+00 - f_ruling=5: a3 coefficient of

polynomial.

rul_a4	=  0.0000000000000000E+00 - f_ruling=5: a4 coefficient of

polynomial.

x_rip_amp	=  0.0000000000000000E+00 - f_g_s=0: amplitude of

distortion along X axis.

x_rip_wav	=  0.0000000000000000E+00 - f_g_s=0: wavelength along X

axis.

x_phase	=  0.0000000000000000E+00 - f_g_s=0: phase for X axis; 0

implies maximum at origin.

y_rip_amp	=  0.0000000000000000E+00 - f_g_s=0: amplitutde along Y

axis.

y_rip_wav	=  0.0000000000000000E+00 - f_g_s=0: wavelength along Y.

y_phase	=  0.0000000000000000E+00 - f_g_s=0: phase for Y axis.

n_rip	=             0 - read in from gaussian ripple file in OE

generation.

oe_number	=             0 - the rest of the variables are filled

idummy	=             0           in during OE generation.

dummy	=  0.0000000000000000E+00

$end