----- file created by makedoc on: Thu Jul 20 11:13:10 2000

----- Documentation for /pgexp/applications1/XRayOptics/xop/src_idl/sources/xbfield.pro -----

==============================  Xbfield   =====================================

Xbfield is an widget based graphical interface to calculate

field of an tappered undulator insertion devices.

************************* IMPORTANT NOTICE ************************************

This utility (coming with the YAUP distribution), in the tapered case,

generates a B field assumed be an amplitude-modulated sinusoid, e.g.

Btot(z) = B(z)*sin(2*pi/period*z) and a linearly modulated gap is

assumed.  DG is the degree of gap taper:

gap=gzmin*(1.0+z*dg/ulen)

r=gap/per

ampl=0.95*3.44*exp(-r*(5.08-1.54*r)) (for Nb-Fe-B magnets) or

ampl=0.95*3.33*exp(-r*(5.47-1.80*r)) (for Sm-Co magnets). This

approximation comes from "6 GeV Synchrotron X-ray Source, Conceptual

Design Report Supplement A", LS-52, Argonne National Laboratory,

(March 1986), and you can also find it in the ESRF Foundation Phase

Report (The red book), pag CIV-314

If these parameters does not fit with the user's requirements, then

the user should either customize it or create another bfield

program which writes the output magnetic field in the format

accepted by yaup.

*******************************************************************************

DESCRIPTION OF THE CONTROLS IN THE MAIN WINDOW:

File:

XBField input parameters: This option allows to save the current

parameters to a file for later loading. It also allows

to save the current parameters as defaults for being

used when the application is initialized. In the last

case, the file is named "application".xop (where

"application " is the name of the current XOP

application) and is written in the directory pointed

by the XOP_DEFAULTS_DIR environment variable (which

must be set). The parameter file is ASCII and can be

read and edited with care.

Quit: to exit from the program

Set_Parameters:

Set Parameters: to define the parameters for the calculation.

The same result is obtained pressing the "Set Parameters"

button in the main XBField window.

Please refer to the information under the HELP

button for a complete description of the parameters. After

pressing the ACCEPT button, BFIELD starts running.

Set Defaults: Sets the default parameters.

Show: Display results

Plot Results: Plots the BFIELD results.

Show Numerical Results: Displays the BFIELD result file.

Help:   Shows the BFIELD help and the XBFIELD help (this text).

COPYRIGHT:

xyaup and xbfield belong to XOP package and are distributed within XOP.

PLEASE REFER TO THE XOP COPYRIGHT NOTICE BEFORE USING IT.

CREDITS:

Published calculations made with XOP should refer:

M. Sanchez del Rio and R. J. Dejus "XOP: Recent Developments"

SPIE proceedings vol. 3448, pp.340-345, 1998.

In addition, published calculations using the YAUP package

should also cite:

B. I. Boyanov, G. Bunker, J. M. Lee, and T. I. Morrison

"Numerical Modeling of Tapered Undulators"

Nucl. Instr. Meth. A339,  596-603, 1994

LAST MODIFICATION: msr/msr/99-03-16