----- file created by makedoc on: Thu Jul 20 11:12:40 2000

----- Documentation for /pgexp/applications1/XRayOptics/xop/src_idl/obsolete/xfilter.pro -----

===========================  Xfilter   =========================

Xfilter is a widget based graphical interface to calculate

the transmission in terms of energy vs flux or power of an x-ray

beamline. Its difference respect to XABREFC and XTRANSMIT is that

XFILTER includes the absorption by Compton and Rayleigh effects

in addition to the usual photoelectric one. The photoelectric

cross section is the only one considered in XABREFC and XRTRANSMIT.

XFILTER interfaces the program filter, built using the subroutine

library written by S. Brennan (Rev. Sci. Instrum. 63 (1) (1992))

which perfoms calculations besed on Cromer and Liberman and

McMasters cross section tables.

The main window contains four buttons:

QUIT: to exit from the program

SET PARAMETERS: to define the parameters for the calculation

these parameters are the following

NUMBER OF OPTICAL ELEMENTS to be used for the calculation.

min=1, max=5

SOURCE: Use a normalized source or a source from a file.

This file can be any 2-colums file with your

source spectrum and can also be created by the

xop/source programs and saved as for XOP/Optics.

ENERGY LIMITS AND POINTS : evident.

Then 5 lines, one for each filter. Each line has

SYMBOL OF ELEMENT: Capitalized, 2 letters.

FILTER THICKNESS in microns [if filter is selected].

SHOW:	Produces a graphic with the selected option

...

CUMULATIVE PROPERTIES or the flux or power of the source

after being reduced by the optical elements to the actual

one.

The two last options call XPLOT with the data of all

optical elements arranged in columns. Is therefore possible

to use XPLOT's overplotting capacities to generate

graphics containing all the curves. The data set

configuration is:

Column 1: Photon Energy [eV]

Column 2: Source Flux of Power

Column 3: Flux of Power after the 1st optical element

Column 4: Flux of Power after the 2nd optical element

Column 5: Flux of Power after the 3rd optical element

Column 6: Flux of Power after the 4th optical element

Column 7: Flux of Power after the 5th optical element

(for symmetrical perfect crystals and mosaic crystals

HELP:	Shows this text

-----------------------------------------------------------------------

Author: M. Sanchez del Rio (srio@esrf.fr) , ESRF, Mar 31, 1995

Modification history:

95/11/07 MSR adds file in/out. Modify input str to better arrange

the input widgets. Updates to xop 1.3b

96/01/19 MSR Makes more compact the show menu, by using data

selection inside xplot.

97/10/01 renames read_ascii by rascii

97/10/02 uses xop_wd, sdep() and makes other cosmetics.