================================   xtube_w   ===================================

xtube_w is an widget based graphical interface to calculate the spectum

of a Tungsten X-ray tube.

It uses a routine routine to generates spectra (photons/mm**2 per keV) for

one mAs and the output is calibrated to the output of a constant potential

generator in our laboratory (a Toshiba Model 2050 x-ray generator with a

"Rotanode" x-ray tube with housing model DRX-573HD-S and insert model

DR-5735H).

These spectra were calibrated to output (mR/mAs) values and HVL values

that were measured down the central axis of the x-ray beam.  The output

data are given in the manuscript:

"An accurate method for computer- generating tunsten anode x-ray spectra from

30 to 140 kV", JM Boone and JA Seibert, Medical Physics 24(11) pp.1661-1670,

November 1997.

The program interfaced by XTubeW has been downloaded from:

ftp://ftp.aip.org/epaps/medical-phys/E-MPHYA-24-1661

DESCRIPTION OF THE CONTROLS IN THE MAIN WINDOW:

File:

Xtube_W input parameters: This option allows to save the current

parameters to a file for later loading. It also allows

to save the current parameters as defaults for being

used when the application is initialized. In the last

case, the file is named "application".xop (where

"application " is the name of the current XOP

application) and is written in the directory pointed

by the XOP_DEFAULTS_DIR environment variable (which

must be set). The parameter file is ASCII and can be

read and edited with care.

Write Files for XOP/Optics: Saves the current undulator

spectrum in the SRCOMPE (flux) and SRCOMPW (Power) files

to be used for the other XOP application from Optics

menu.

Quit: to exit from the program

Set_Parameters:

Set Parameters: to define the parameters for the calculation.

The same result is obtained pressing the "Set Parameters"

button in the main Xtube_w window.

Please refer to the information under the HELP

button for a complete description of the parameters. After

pressing the ACCEPT button, xtube_w starts running.

Set Defaults: Sets the default parameters.

Show: Display results

Plot Results: Plots the xtube_w results.

Help:   Shows the xtube_w help (this text).

COPYRIGHT:

xtube_w  belongs to XOP package and it is distributed within XOP.

PLEASE REFER TO THE XOP COPYRIGHT NOTICE BEFORE USING IT.

CREDITS:

Published calculations made with XOP should refer:

M. Sanchez del Rio and R. J. Dejus "XOP: Recent Developments"

SPIE proceedings vol. 3448, pp.340-345, 1998.

In addition, published calculations using the xtube_w aplication

should also cite:

J.M. Boone and J.A. Seibert "An accurate method for

computer-generating tunsten anode x-ray spectra from 30 to 140 kV",

Medical Physics 24(11) pp.1661-1670, 1997.

LAST MODIFICATION: msr/msr/99-03-16