----- file created by makedoc on: Thu Jul 20 11:12:45 2000

----- Documentation for /pgexp/applications1/XRayOptics/xop/src_idl/dabax/f1f2_calc.pro -----

NAME:

F1F2_CALC

PURPOSE:

calculate the elastic Photon-Atom anonalous f1 and f2  coefficients

as a function of energy. It also gives the refractive index

components delta and beta (n=1-delta - i beta), the absorption

photoelectric coefficient and the reflectivities (s,p and unpolarized).

It users data from DABAX data-base.

CATEGORY:

X-Ray optics. DABAX data base.

CALLING SEQUENCE:

f1 = f1f2_calc(input,descriptor[,energy])

INPUTS:

input: a dabax input file as accepted by dabax_access().

descriptor: a string  with a description of the material.

The string is searched in the scan titles #S in order

to find the appropiated data.

OPTIONAL INPUTS:

energy: if undefined, it uses the standard energy grid,

which is the grid in the data file for the tabulated

cases, and  dindgen(100)/99.*3.0 for the parametrized

data. When energy is defined, this value is used as a grid

for the results (it interpolates in the case of

tabulated data)

KEYWORDS:

F: selects the output:

F=0 (default) returns a 2-col array with f1 and f2

F=1  returns f1

F=2  returns f2

F=3  returns delta  [n = 1 -delta -i beta]

F=4  returns betaf  [n = 1 -delta -i beta]

F=5  returns Photoelectric linear absorption coefficient

F=6  returns Photoelectric mass absorption coefficient

F=7  returns Photoelectric Cross Section

F=8  returns s-polarized reflectivity

F=9  returns p-polarized reflectivity

F=10  returns unpolarized reflectivity

THETA= the incident [grazing] angle in mrad (for reflectivity

calculations). It is set to 3.0 (default) when F>=8 and

THETA is undefined. It can be an array. In such a case, the

returned value os a matrix: out(theta,energy)

ROUGH= the rms roughness in Angstroms (for reflectivity

calculations. A Debye-Waller model is used.)

DENSITY= the density value to be used for the calculations. If

not defined, take the value given by atomic_constants().

GROUP = The widget ID of the widget that calls the routine.

This is passed to widget_message in its dialog_parent

keyword to center the window message.

OUTPUT:

out: an array with the values of the selected return parameter.

PROCEDURE:

Takes f1 and f2 from the DABAX files. It also takes the

Atomic constants from DABAX, and performs the classical

operations to calculate the other parameters.

EXAMPLES:

EXAMPLE1: f" vs. energy

delvar,energy ; cleans energy variable

f = f1f2_calc('f1f2_Henke.dat','Si',energy,f=2)

plot,energy,f,/xlog

EXAMPLE2: unpolarized refl vs. theta and energy

; create 20 points in the energy range [1000.,21000.] (eV)

energy = (findgen(20)+1)/20.0*10000.0+1000.0

; create 20 points in the theta range [1.0,6.0] (mrad)

theta = (findgen(20)+1)/20.0*5.0+1.0

out = f1f2_calc('f1f2_BrennanCowan.dat','Si',energy,f=10,theta=theta)

;plot

surface,out,theta,energy,az=285

MODIFICATION HISTORY:

96-07-10 Written by M. Sanchez del Rio (srio@esrf.fr)

96-09-27 MSR Many changes. Adds refractive index and

reflectivity calculations. Uses #UF1ADD to correct f1 with Z.

96-11-19 MSR Adds theta range option.

97-01-19 MSR Adpts for windows95.

97-02-06 MSR corrects a bug in the calculation of mu in cm^2/cm

units pointed out by M.J. Capitan.

97-10-16 MSR adds group keyword.