Last modification date of this file: Mon Nov 17 00:19:40 CST 1997

Roger J. Dejus (dejus@aps.anl.gov)

C+

C PROGRAM DESCRIPTION:

C  Program to calculate undulator spectra within the Bessel function

C  approximation for an ideal planar undulator or an ideal elliptical

C  undulator (including polarization in both cases).

C  The program may be executed from the xop interface.

C

C AUTHORS:

C  Roger J. Dejus

C  The Advanced Photon Source

C  Experimental Facilities Division

C  Argonne National Laboratory

C

C CREATION DATE:

C  25-MAR-1991

C

C INPUT PARAMETERS:

C  The input parameters are divided into sections related to the storage ring,

C  the undulator device, and the quantity to be calculated. Note: When modifying

C  parameters under the Xus interface, double click the field and make sure to

C  press the RETURN key so that the new parameter is accepted.

C Machine Parameters:

C  Storage ring energy 			(GeV)

C  Storage ring current			(mA)

C  RMS beam size (horizontal)		(mm)

C  RMS beam size (vertical)		(mm)

C  RMS beam divergence (horizontal)	(mrad)

C  RMS beam divergence (vertical)	(mrad)

C Undulator Parameters:

C  Period length			(cm)

C  Number of periods

C  Deflection parameter (hor.  field) Kx (= 0.0 for a regular planar device)

C  Deflection parameter (vert. field) Ky

C Scan Parameters:

C  Minimum energy			(eV)

C  Maximum energy			(eV)

C  Number of energy points

C Pinhole Parameters:

C  Distance from the source		(m)

C    (d=0.0 => angular units)

C  X-coordinate for center of pinhole	(mm) or (mrad)

C  Y-coordinate for center of pinhole	(mm) or (mrad)

C  X-size of pinhole (full width)	(mm) or (mrad)

C  Y-size of pinhole (full width)	(mm) or (mrad)

C    (for angular units (d=0.0) values are entered in mrad)

C    (X is for horizontal direction)

C    (Y is for the vertical direction)

C  Number of subdivisions of pinhole in X (max 50)

C  Number of subdivisions of pinhole in Y (max 50)

C    (for plotting 3d results with Xus, the X-size, Y-size, and the number of

C     of subdivisions in the two directions should be equal)

C

C Mode:

C  Depending on the mode selected, some of the pinhole parameters may be

C  set to different values by the program; see the output file us.plt.

C  MODE    1    Angular/spatial flux density distribution

C  MODE    2    Angular/spatial flux density spectrum

C  MODE    3    On-axis brilliance spectrum

C  MODE    4    Flux spectrum through a pinhole

C  MODE    5    Flux spectrum integrated over all angles

C  MODE    6    Power density and integrated power

C

C  Angular/spatial flux density distribution

C    - Flux distribution at the energy chosen as minimum energy.

C  Angular/spatial flux density spectrum

C    - Spectrum at any given point in space as selected by the X and Y

C      coordinate for the center of the pinhole. X is horizontal and Y is

C      vertical.

C  On-axis brilliance spectrum

C  Flux spectrum through a pinhole

C    - Spectrum through a pinhole centered at X-center and Y-center with

C      size X-size and Y-size.  The energy range is from the minimum to the

C      maximum energy.

C  Flux spectrum integrated over all angles.

C    -  The pinhole parameters have no significance here.

C  Power density and integrated power

C    -  Integrated over all energies, thus the energy parameters have no

C       significance here.

C

C Method:

C  METHOD  1    Non-zero emittance; finite-N

C  METHOD  2    Non-zero emittance; infinite-N

C  METHOD  3    Zero emittance;     finite-N

C  METHOD  4    Non-zero emittance; infinite-N + convolution (Dejus' approach)

C  METHOD 14    Non-zero emittance; infinite-N + convolution (Walker's approach)

C

C  Non-zero emittance; finite-N

C    - Use only for "Angular/spatial flux density distribution" and for

C      "Power density and integrated power".

C  Non-zero emittance; infinite-N

C    - For test purposes; do not use (will be removed from menu).

C  Zero emittance; finite-N

C    - Use for zero emittance calculations.

C  Non-zero emittance; infinite-N/convolution

C    - Generally, use for cases where emittance should be included.

C

C Harmonic Number:

C  IHARM   0    All harmonics

C  IHARM  -1    Lowest order harmonic (except MODE=6, include to -IHARM)

C  IHARM   I    I'th harmonic

C

C  All harmonics

C    - Selects all contributing harmonics (generally used).

C  Lowest order harmonic

C    - Selects the lowest order contributing harmonic.

C  Harmonic #

C    - Selects the harmonic number displayed.

C  Edit harmonic number

C    - Modifies the displayed harmonic number.

C

C Intrinsic Parameters:

C  Several parameters used in the calculations.  Usually not modified by the

C  user.  Please see me (RJD) for further information.

C

C Polarization:

C  The normalized Stokes parameters are calculated including the

C  unpolarized component.

C

C DESIGN ISSUES:

C  Program is based on the Bessel function approximation and is valid in the

C  far-field for an ideal sinusoidal magnetic field profile.

C

C COPYRIGHT:

C  Unless part of the XOP software package, this routine must only be used

C  at The Advanced Photon Source and must not be tranferred or used at any

C  other location without the written consent of the author.

C

C FILES USED:

C  Input file - us.dat  File in the user's current directory containing the

C                       input parameters.

C  Output file - us.plt File in the user's current directory containing the

C                       results of the calculation.  The header contains

C                       all input parameters and the calculated zero emittance

C			on-axis first harmonic energy (e1), corresponding

C                       wavelength (l1), total power (ptot), and the on-axis

C                       power density (pd).

C KEYWORDS:

C  Undulator Spectrum, Bessel Function Approximation

C

C LINK/LIBRARY ISSUES:

C  Calls routines BRIGHTE and HUNT.  BRIGHTE calculates the brilliance and HUNT

C  searches an array of real numbers (from Numerical Recipes).

C

C PORTABILITY ISSUES:

C  Runs on DEC 3000/400 AXP alpha (Unix v. 3.2c), SUNs (SUN-OS 4.1.3),

C  HP 9000/735-series (HP-UX 9.03).

C

C TIMING:

C  Execution times vary considerably depending on computer and the

C  quantity being calculated.  The zero emittance calculations are fast

C  (few seconds), whereas the non-zero emittance calculations may range from

C  seconds (on-axis brilliance) to an hour (flux spectrum through a pinhole).

C

C VERSION:

C  1.9

C

C MODIFICATION HISTORY:

C

C	 Date     | Name  | Description

C ----------------+-------+-----------------------------------------------------

C 06-JUL-1994     | RJD   | Modified value for E1MIN for angle-integrated

C                 |       | spectrum (MODE=5) to be non-zero; gamma*theta

C                 |       | corresponds to sqrt(200) (somewhat arbitrarily

C                 |       | chosen)

C ----------------+-------+-----------------------------------------------------

C 04-OCT-1994     | RJD   | Modified program to include polarization properties.

C                 |       | The four Stokes parameters are now calculated.

C                 |       | Program is for an ideal planar undulator or an ideal

C                 |       | elliptical undulator. Many other changes. The value

C                 |       | of the parameter IHARM has a different meaning.

C                 |       | IHARM=0 now gives 'all harmonics' and IHARM= <0

C                 |       | gives the lowest order harmonic except for the power

C                 |       | option. For the power option, a negative IHARM means

C                 |       | include all harmonics up to and including -IHARM.

C                 |       | This version is 1.6.

C ----------------+-------+-----------------------------------------------------

C 21-JUN-1995     | RJD   | Modified print-out of "Contributing harmonics" in

C		  |       | subroutine PRINT_OUT. Routine incorrectly calculated

C		  |       | IMIN and IMAX for METHOD 4 (Dejus' method) for

C		  |       | "Spectral distributions". The spectra and integrated

C		  |       | quantities were calculated correctly and are

C		  |       | unaffected by this modification.

C                 |       | The current version is 1.7.

C ----------------+-------+-----------------------------------------------------

C 04-JAN-1996     | RJD   | Modified the number of decimal places for the sigx1

C                 |       | and sigy1 variables to four in the printout. Added

C                 |       | one more digit for the emax variable to avoid

C                 |       | overflow on rare occasions. Formats 260 and 256 were

C                 |       | changed accordingly.

C                 |       | The current version is 1.8.

C ----------------+-------+-----------------------------------------------------

C 11-NOV-1997     | RJD   | Changed notation: Brightness -> Brilliance.

C                 |       | The current version is 1.9.

C ----------------+-------+-----------------------------------------------------

C [change_entry]

C-