===========================  Xws   =========================

Xws is an widget based graphical interface to calculate

spectral properties of a wiggler insertion device.

The main window contains four buttons:

QUIT: 	Exits from the program

DATA:   Defines the parameters for the calculation

SHOW:	Plots, Prints or Views the results

HELP:	Shows the Xws help text or the WS help text

-----------------------------------------------------------------------

Author: Roger J. Dejus (dejus@anlaps.aps.anl.gov), APS, May, 1994.

Modification history:

96/01/19 MSR changes labels Xtransmit -> XOP/Optics

97/01/23 MSR changes for Windows95, cosmetics (xhelp, widget_message...)

97/02/05 MSR adds help button in parameters window.

97/10/02 MSR uses sdep(), xop_wd and other cosmetics.

00/07/17 MSR introduces ws 1.5 in the interface.

C+

C PROGRAM DESCRIPTION:

C  Program to calculate wiggler and bending magnet spectra using the

C  Bessel function approximation. The program may be executed from the xop

C  interface.

C

C AUTHORS:

C  Roger J. Dejus

C  The Advanced Photon Source

C  Experimental Facilities Division

C  Argonne National Laboratory

C

C CREATION DATE:

C  17-FEB-1994

C

C INPUT PARAMETERS:

C  The input parameters are divided into sections related to the storage ring,

C  the wiggler device, and the quantity to be calculated.

C Machine Parameters:

C  Storage ring energy 			(GeV)

C  Storage ring current			(mA)

C Wiggler Parameters:

C  Period length			(cm)

C  Number of periods

C Note: For a bending magnet source: set N=0.5, and make Ky large and adjust

C       the period length accordingly. For example, put Ky=9.34 and calculate

C       the period length from, Period (cm) = 10.0/B0(T), where B0 is the known

C       strength of the magnetic field (in Tesla) for the bending magnet.  The

C       calculated power density (pd) is correct, but the total power (ptot)

C       is irrelevant. Typically make the extend of the pinhole small in the

C	horizontal direction (theta << Ky/gamma) as the intensity should

C	not depend on the horizontal offset. Check value of B0 (and critcal

C	energy EC0) in the plot file.

C  Deflection parameter (hor.  field) Kx (= 0.0 only; for elliptical wiggler

C  not yet implemented)

C  Deflection parameter (vert. field) Ky

C Scan Parameters:

C  Minimum energy			(eV)

C  Maximum energy			(eV)

C  Number of energy points

C Pinhole Parameters:

C  Distance from the source		(m)

C    (d=0.0 => angular units)

C  X-coordinate for center of pinhole	(mm) or (mrad)

C  Y-coordinate for center of pinhole	(mm) or (mrad)

C  X-size of pinhole (full width)	(mm) or (mrad)

C  Y-size of pinhole (full width)	(mm) or (mrad)

C    (for angular units (d=0.0) values are entered in mrad)

C    (X is for horizontal direction)

C    (Y is for the vertical direction)

C  Number of subdivisions of pinhole in X (max 50)

C  Number of subdivisions of pinhole in Y (max 50)

C

C Mode:

C  Depending on the mode selected, some of the pinhole parameters may be

C  set to different values by the program; see the output file ws.plt.

C  MODE    1    Angular/spatial flux density distribution

C  MODE    2    Angular/spatial flux density spectrum

C  MODE    3    On-axis brilliance spectrum (not yet implemented)

C  MODE    4    Flux spectrum through a pinhole

C  MODE    5    Flux spectrum integrated over all angles

C  MODE    6    Power density and integrated power

C

C  Angular/spatial flux density distribution

C    - Flux distribution at the energy chosen as minimum energy.

C  Angular/spatial flux density spectrum

C    - Spectrum at any given point in space as selected by the X and Y

C      coordinate for the center of the pinhole. X is horizontal and Y is

C      vertical.

C  On-axis brilliance spectrum (not yet implemented)

C  Flux spectrum through a pinhole

C    - Spectrum through a pinhole centered at X-center and Y-center with

C      size X-size and Y-size.  The energy range is from the minimum to the

C      maximum energy.

C  Flux spectrum integrated over all angles (wiggler only).

C    -  The pinhole parameters have no significance here.

C  Power density and integrated power

C    -  Integrated over all energies, thus the energy parameters have no

C       significance here.

C

C Polarization:

C  The normalized Stokes parameters are calculated including the

C  unpolarized component.

C

C DESIGN ISSUES:

C  Program calculates the spectra from the Modified Bessel functions.  See K.J.

C  Kim, in "Physics of Particle Accelerators", vol. 1, AIP Conference Proc. 184

C  Ed. R.G. Lerner, New York (1989), p. 583, Eq. (3.12).

C  The algorithm is based on a series expansion for small arguments Z

C  (Abramowitz & Stegun Eq. 9.6.2 and 9.6.10) and an asymptotic expansion for

C  large arguments (Eq. 9.7.2).

C  Reference: Handbook of Mathematical Functions, Eds. Milton Abramowitz and

C  Irene A. Stegun, Ninth Printing, Dover Publications, New York (1970).

C  NOTE: THE POLARIZATION PARAMETERS ARE PROVIDED ALTHOUGH NOT THOROUGHLY

C  TESTED - USE WITH CAUTION.

C

C COPYRIGHT:

C  Unless part of the XOP software package, this routine must only be used

C  at The Advanced Photon Source and must not be tranferred or used at any

C  other location without the written consent of the author.

C

C FILES USED:

C  Input file - ws.dat  File in the user's current directory containing the

C                       input parameters.

C  Output file - ws.plt File in the user's current directory containing the

C                       results of the calculation.  The header contains

C                       all input parameters and the calculated on-axis first

C			harmonic energy (e1), corresponding wavelength (l1),

C			total power (ptot), and the on-axis power density (pd).

C			See note above when using N=0.5 for bending magnet.

C KEYWORDS:

C  Wiggler Spectrum, Modified Bessel Function of Second kind.

C

C LINK/LIBRARY ISSUES:

C  The gamma function is needed.  Currently uses no library routines. The values

C  for gamma(2/3) and gamma(1/3) are stored as constants. May be substituted by

C  calls to the NAG library routine S14AAF.

C

C PORTABILITY ISSUES:

C  Runs on DEC 3000/400 AXP alpha (Tru64Unix v5.0), SUN (Solaris: SunOS

C  Release v5.6), and Windows 95/98/NT (Pentium and higher).

C

C TIMING:

C  Execution time is typically very fast but depends on the quantity being

C  calculated. Typically seconds to at the most minutes.

C

C VERSION:

C  1.5

C

C MODIFICATION HISTORY:

C

C	 Date     | Name  | Description

C ----------------+-------+-----------------------------------------------------

C 17-JUL-2000     | RJD   | Adopted from v1.4 which was never released to the

C                 |       | public. Turned off circular polarization for the

C                 |       | wiggler (which is only valid for EMW).

C                 |       | Current version is v1.5.

C ----------------+-------+-----------------------------------------------------

C [change_entry]

C-