----- file created by makedoc on: Thu Jul 20 11:13:14 2000

----- Documentation for /pgexp/applications1/XRayOptics/xop/src_idl/sources/xyaup.pro -----

==============================  Xyaup   =======================================

Xyaup is a widget based graphical interface to calculate spectra of a

tapered undulator insertion devices.

It interfaces the program YAUP 1.3.1 written by B. Boyanov (see Help/Yaup)

DESCRIPTION OF THE CONTROLS IN THE MAIN WINDOW:

File:

XYaup input parameters: This option allows to save the current

parameters to a file for later loading. It also allows

to save the current parameters as defaults for being

used when the application is initialized. In the last

case, the file is named "application".xop (where

"application " is the name of the current XOP

application) and is written in the directory pointed

by the XOP_DEFAULTS_DIR environment variable (which

must be set). The parameter file is ASCII and can be

read and edited with care.

Yaup file: This option allows to load/write the current parameter

and result files for further calculations. The used file

formats are those used by YAUP. Therefore, they

can be used to run YAUP without the interface or to

import YAUP files created outside the XYAUP application.

Write Files for XOP/Optics: Saves the current undulator

spectrum in the SRCOMPE (flux) and SRCOMPW (Power) files

to be used for the other XOP application from Optics

menu.

Quit and preserve files: Quits without cleaning current directory

(i.e., preserving the created data files.)

Quit: to exit from the program

BField:

from ASCII file:

Yaup needs a file with tha magnetic field. This may be

obtain from a user's file (column-formatted, containing

three colums: z, B(z), and phi(z), where the z s are

equidistant with step PERIOD/NPTS.

See Help/Yaup for definitions of PERIOD and NPTS.

There should be NPTS*NPER+1 lines in the ASCII file.

from XBFIELD Preprocessor: calls XBFIELD, an interface for the

program BFIELD distributed in the YAUP package. This program

creates the magnetic field starting from the undulator

oparameters.

from Linear B field: Creates a magnetic field modulated with a

straight line with the given B or K extrema.

Set_Parameters:

Set Parameters: to define the parameters for the calculation.

The same result is obtained pressing the "Set Parameters"

button in the main XYAUP window.

Please refer to the information under the HELP

button for a complete description of the parameters. After

pressing the ACCEPT button, YAUP starts running.

Set Defaults: Sets the default parameters.

Show: Display results

Plot B field: Plots the computed B field.

Plot Trajectory: Plots the electron trajectory in the

defined magnetic field.

Show Trajectory Numerical Data: display the trajectory

output file.

Plot Results: Makes 2-D or 3-D plots of the results,

depending on the mode and other defined parameters.

Show Results Numerical Data: display the yaup result file.

Help:   Shows the YAUP help and the XYAUP help (this text).

COPYRIGHT:

xyaup and xbfield belong to XOP package and are distributed

within XOP.

PLEASE REFER TO THE XOP COPYRIGHT NOTICE BEFORE USING IT.

CREDITS:

Published calculations made with XOP should refer:

M. Sanchez del Rio and R. J. Dejus "XOP: Recent Developments"

SPIE proceedings vol. 3448, pp.340-345, 1998.

In addition, published calculations using the XYAUP package

should also cite:

B. I. Boyanov, G. Bunker, J. M. Lee, and T. I. Morrison

"Numerical Modeling of Tapered Undulators"

Nucl. Instr. Meth. A339,  596-603, 1994

LAST MODIFICATION: msr/msr/99-03-16