----- file created by makedoc on: Thu Jul 20 14:46:17 2000

----- Documentation for /pgexp/applications1/XRayOptics/shadowvui/vui/xsh_abrefc.pro -----

=================================  XSh_AbRefC =============================

XSh_AbRefC is a widget based graphical interface to calculate

several properties of a given material (over a specified photon

energy range) that are important for physical optics calculations.

It interfaces the program ABREFC, part of the ray-tracing package SHADOW.

It creates intermediate files that are deleted when QUITing XSh_AbRefC.

The main window contains four buttons:

QUIT: to exit from the program

SET PARAMETERS: to define the parameters for the calculation

THICKNESS: thickness of the material in microns

(needed for computing the transmission).

DENSITY: Material density

INCIDENCE ANGLE: in degrees (for reflectivity)

PHOTON ENERGY: in eV, start end and step.

For each element:

Atomic symbol: Two capitalized letters.

Formula index.

SHOW: Display the selected option. Available options are:

DIELECTRIC FUNCTION: epsilon = 1 - alpha + i* beta

the user can display alpha or beta.

Units are farad/cm

ABSOPTION COEFFICIENT: Units are cm^(-1)

TRANSMISSION: Tha transmission through a film of thickness

d (as specified by the user):

T = exp(-d*mu*Phot_ener)

REFLECTIVITY: The s and p components of the reflectivity

R are computed from the Fresnel equations and the

dielectric function. Also available the average

between the s and p components.

REFRACTION INDEX: n^2 = epsilon. Only the real part is

available.

CROSS SECTION: for photoelectric effect. Units in cm^2

ELECTRON YIELD: as defined by B. Henke. In essence, the

primary electron yield is proportional to the

square of the electric field at the surface of

the material.

ENERGY LOSS: The rate at which energetic electrons lose

energy when travelling through a medium is

proportional to the imaginary part of the dielectric

function. This is obtained directly from the

linear medium response theory in the limit of

small q (Raether, 1960)

R(Delta_E) = - Imag(1/epsilon)

F1:	the atomic scattering factor f'

F2:	the atomic scattering factor f''

HELP:   Shows this text

----------------------------------------------------------------------

Author: S. Kesmarki, ESRF, Jun 1993

Modification history:

9/11/93 M. Sanchez del Rio: add HELP and cosmetics

16/5/94 M. Sanchez del Rio: adapt to a new version

of abrefc (called abrefcnew) which has

an extended optical library (> 100 keV) and

creates the F1 and F2 files with the atomic

scattering factors.

94/10/27  M. Sanchez del Rio: cosmetics.

94/11/28  MSR avoid the program crashes when try to

set #compounds>5 (remarked by N. Brooks)

95/05/01  Roger J. Dejus, XFD/APS. Added the case

'nothing' to the procedure xsh_abrefc_setparam_event to

avoid crashing when user enter a <CR>.

97/10/02  srio@esrf.fr makes use of xop_wd,/default

97/12/05  srio@esrf.fr adapts for SHADOW-GUI