----- file created by makedoc on: Thu Jul 20 14:46:21 2000

----- Documentation for /pgexp/applications1/XRayOptics/shadowvui/vui/xsh_transmit.pro -----

===========================  XSh_transmit   =========================

XSh_transmit is a widget based graphical interface to calculate

the transmission in terms of energy vs flux or power of an x-ray

beamline. Source is generated by XSh_SRComp, XSh_Wiggler_Spectrum or

eneterd from external files [called XSH_SRCOMPE (eV,#Phot/sec/0.1%bw)

and SRCPMPW (eV, Watts/eV)].

The reflectivity and transmission coefficients are obtained from

the Fresnel equations.

XSh_transmit is an interface for the program transmit, available

in the SHADOW ray-tracing package.

The main window contains four buttons:

QUIT: to exit from the program

SET PARAMETERS: to define the parameters for the calculation

these parameters are the following

NUMBER OF OPTICAL ELEMENTS to be used for the calculation.

min=1, max=5

RESULTS IN: Select Flux or Power for the presentation

of results.

Then 5 lines, one for each optica element [mirror or

filter]. Lines are hidden when not used. Each line has

KIND OF ELEMENT: Filter or Mirror

SYMBOL OF ELEMENT: Capitalized, 2 letters.

FILTER THICKNESS in microns [if filter is selected].

INCIDENCE ANGLE in degrees [if mirror is selected].

SHOW:	Produces a graphic with the selected option

These options are:

SOURCE a graph of the source spectrum.

DATA show a list of important parameters of the

optical elements in use.

LOCAL PROPERTIES [Reflectivity for mirrors and

Transmission for filters] of each defined element.

The selection of each oe. is done inside the Xplot

graphical window.

CUMULATIVE PROPERTIES or the flux or power of the source

after being reduced by the optical elements to the actual

one.

The selection of each oe. is done inside the Xplot

graphical window.

Is therefore possible to use XPLOT's overplotting capacities

to generate graphics containing all the curves.

HELP:	Shows this text

-----------------------------------------------------------------------

Author: M. Sanchez del Rio (srio@esrf.fr) , ESRF, Jan 1994

Modification history:

94/03/02 MSR removes the DENSITY entry. Uses the

tabulated values in p_table. Also removes the limitation

in the chemical element symbols that had to duplicate

the letter in the symbols with only one character.

94/10/27 MSR removes common blocks, call xop_defaults

95/11/07 MSR adds file in/out. Adapts to xop 1.3b

Date     | Name  | Description

----------------+-------+-----------------------------------------------------

07-AUG-1995     | RJD   | Modified the help text.

----------------+-------+-----------------------------------------------------

17-JAN-1996     | MSR   | Modified the help text.

----------------+-------+-----------------------------------------------------

19-JAN-1996     | MSR   | Makes the oe. selection inside xplot.

|       | Changes the help text.

----------------+-------+-----------------------------------------------------

97/01/16 srio@esrf.fr adapts for Windows95.

97/10/01 srio@esrf.fr renames read_ascii by rascii. Uses

shadow_defaults instead of xop_defaults. Uses SDEP().

97/10/02 srio@esrf.fr makes use of xop_wd,/default

97/12/05 srio@esrf.fr adapts for SHADOW-GUI. version 1.1