YAUP - Yet Another (Useless) Undulator Program

Version 1.3.1

MODIFICATION RECORD

- 12/92 - Initial version (without FFTs); calculates spectral and

angular distributions of the brightness and flux through

pinholes.

- 01/92 - FFT version.

- 04/92 - Bugs in module EXTRCT and trajectory calculation fixed.

Added degree-of-linear-polarization calculations.

- 06/92 - Add Hanning windows to reduce spectral leakage;

expert-user options; power-load calculations.

- 10/92 - New input-file format.  Life is easy now.(v1.0)

- 04/93 - Include particle beam sizes. (v1.1)

- 09/93 - Fix a bug in the flux calculations with finite

observation distance Some changes in the input file format.

(v1.1b)

- 11/93 - Switch back to double precision variables for temp

storage. Increases RAM requirements but improves "inter

platform" reproducibility of the results. (v1.2) Check (but do

not enforce) L/R symmetry criterion. (v1.2b) More flexible

scratch file names. (v1.2c)

- 12/93 - User-definable scratch file names.  Bug in spline

interpolations fixed (INTGRD). Add GNUPLOT and BASENAME. (v1.2d)

- 04/94 - Fix imsc bug. Rewrite parser routines.  Now compiles and

runs on an IBM RISC/6000 cluster. (v1.3)

- 04/94 - Fix minor output bugs in mode 7.  Fix integration bug in

mode 7 (v1.3.1)

- 07/94 - Some adjustments to the user interface (v 1.3.2)

WRITTEN BY

Boyan Boyanov

Department of Physics

Illinois Institute of Technology

boyan@tmnxt1.iit.edu

REFERENCE

B. I. Boyanov, G. Bunker, J. M. Lee, and T. I. Morrison

"Numerical Modeling of Tapered Undulators"

Nucl. Instr. Meth. A339,  596-603, 1994

DISCLAIMER

\begin{legalese}

This program is freeware (but *NOT* public domain). You may freely

copy and redistribute it. Permission is granted to modify the source

for your own purposes but NOT to redistribute the modifications without

permission of the author. If you use this program while doing scientific

research please cite this program in the acknowledgments of any resulting

publications.

There are absolutely no warranty on this program. The author takes no

responsibility for any damage caused by this program. The author takes

no responsibility for time lost if incorrect or misleading results are

produced by this program. If a warranty is required by law where you

intend to use this software permission to use this software there is

revoked.

\end{legalese}

Now that this junk is out of the way let's get on with the fun stuff.

TABLE OF CONTENTS

Section I .......................  Introduction

Section II ......................  Compilation Instructions

Section III .....................  New Features

Section IV ......................  Input File Format

Section V .......................  Output File Format

Section VI ......................  Program Description

Section VII .....................  Hints and Suggestions (PLEASE READ)

Section VIII ....................  Data File Formats

Section IX ......................  "Expert" Options

Section X .......................  Further Information

Appendix A ......................  Program Diagnostics

Appendix B ......................  Error Messages

I. INTRODUCTION

1 yawp or yaup \'yo�p\ vi

[ME yolpen]

(14c)

1: to make a raucous noise: SQUAWK

2: CLAMOR, COMPLAIN

� yawp-er n

2 yawp also yaup n

(1824)

1: a raucous noise: SQUAWK

2: something suggestive of a raucous noise; specif:

rough vigorous language

yawp  (or yaup) vb

1  syn    SQUALL 1, caw, ||quawk, squak, squawk

2  syn    GRIPE, ||beef, ||bellyache||||bitch, bleat,

||blow off, crab, fuss, squawk, yammer

Webster's Ninth New Collegiate Dictionary

Webster's Collegiate Thesaurus

First Digital Edition

Copyright (c) 1988, 1990

NeXT Computer, Inc.,

and Merriam-Webster Inc.

YAUP (Yet Another (Useless) Undulator Program) is a program that

calculates radiation patterns (spectral and angular distributions of

the brightness, the flux through a pinhole, spectral and angular power

distributions) and polarization properties of tapered and untapered

undulator radiation. The program has the following capabilities and

restrictions:

- the magnetic field B within the undulator is assumed to be a

vertically-polarized amplitude and/or frequency-modulated

sinusoid. The  B-field distribution is read in from an external

file. Random and or systematic errors in the magnitude and phase

of the field may be included.

- e-beam size and divergence may be included in a first-order

approximation as a convolution of the zero-emittance spectrum

with the e-beam distribution.

- up-down (vertical) symmetry of the angular distribution of the

brightness is always assumed but left-right (horizontal)

symmetry of the spectrum may or may not be enforced (see the

description of input parameter XSYM in sections IV and VII).

- besides tapered undulators the program may be used to calculate

distributions from untapered undulators.  These calculations,

however, are inefficient and programs designed specifically for

untapered undulators, e.g. URGENT and USER_UNDULATOR, are

usually much faster.  We recommend that the program be used for

untapered undulators at energies not higher than the fifth

harmonic only when field errors have to be accounted for (see

section VII).

-  The use of FFTs greatly enhances the performance of the program

(in terms of time needed to complete the calculation),

especially in the case of calculations in a broad spectral

range.

II.  COMPILATION INSTRUCTIONS

YAUP is written in FORTRAN and uses several non-standard but widely

available extensions - mainly DO ... END DO and DO WHILE ... END DO

constructions, COMPLEX*16 data types and its associated intrinsic

functions, and INCLUDE statements.  It calls no external library

routines so it should be fairly easily transportable.  With the

exception of (a large number of) insignificant warnings it passes all

FTNCHEK tests with flying colors.

As of this writing YAUP has been tested on the following machines:

* NeXTStation (black), Absoft 3.1 compiler (BSD UNIX 4.3)

* VAX 3100-3600, DEC FORTRAN compiler (VMS 5.1 or later)

* Macintosh Quadra 610, Language Systems 3.0 compiler (System 7.1)

* IBM RISC System/6000 cluster, IBM AIX XL Compiler/6000 (IBM AIX 3.2)

The program is distributed as a single module named YAUP.F. The file

YAUP.INC is an include file that contains definitions of essential

parameters and common blocks and should be placed in the same

directory as YAUP.F at the time of compilation.

THE PROGRAM SHOULD BE COMPILED WITH THE STATIC MEMORY OPTION TURNED ON.

IF YOUR COMPILER IS CASE SENSITIVE IT IS SAFER TO FOLD ALL VARIABLE

NAMES TO LOWER (OR UPPER) CASE.  BEWARE OF OPTIMIZING COMPILERS - THE

"OPTIMIZATIONS" MAY LEAD TO IMPROPER EXECUTION AND SHOULD BE AVOIDED

UNLESS YOU ABSOLUTELY TRUST YOUR COMPILER'S OPTIMIZER.  THE FIRST

THING YOU SHOULD DO IF YAUP DOES NOT EXECUTE PROPERLY IS TURN OFF ALL

OPTIMIZATIONS.

A VERY important parameter in YAUP.INC that may need customizing to

your compiler is LWORD.  YAUP uses unformatted direct-access files to

store intermediate results and the various FORTRAN compilers handle

these files in different ways. Generally the value of LWORD should be

set equal to the number of bytes in the direct-access  storage unit,

which for most compilers is 1.  The VAX FORTRAN compilers, however,

use 4-byte portions (words).  SET LWORD TO 4 PRIOR TO COMPILATION IF

YOU USE VAX FORTRAN OR SOME OTHER COMPILER WITH FUNNY IDEAS ON HOW THE

RECL LENGTHS SHOULD BE SPECIFIED.   The program is distributed with

LWORD set to 1.  It should run on any system with this setting but

the amount of free disk space needed for successful execution may be

unnecessarily large.

Some compilers (such as the older versions of MacFortran) do not allow

direct-access records longer than 1024 bytes.  YAUP may not run on

such systems.

Another possible customization concerns the way your compiler handles

output string editing.  YAUP prints information on the progress of the

calculations to the screen and in doing so it uses '+' (overprint)

character editing. If your compiler interprets the first character of

the output string as a control character you need not do anything (VAX

and MPW (LangSys) FORTRAN, for example).  If this is not so, special

action may be required (this is the case with many, if not most, Unix

compilers). Absoft FORTRAN for NeXTStep, for example, requires the

statement

OPEN(UNIT=6,ACTION='PRINT')

added to the very beginning of YAUP.F to ensure that the

screen-directed output is handled properly.  Check your compiler

manula and inser the appropriate code if necessary.

YAUP needs somewhat large amounts of RAM and free disk space to compile

and run properly.  At least 8 Mb RAM are needed for compilation on

most machines, 5-6 Mb of free RAM should be available at runtime, and

UP TO 260 Mb of free disk space (for temporary data storage) may be

needed IF THE PROGRAM IS PUSHED TO EXTREMES.  The amount of disk space

needed depends on the input parameters, but approximately

2*8*NXP*NYP*NE bytes of disk space will be needed for temporary data

storage (for a description of NXP, NYP and NE see section IV).  If

anything, this is a very rough underestimate.

For FORTRAN gurus only: The FORTRAN compiler on AIX machines handles

IOSTAT in a very weird fashion.  When an I/O error is encountered, e.g.

attempt to read a characted where a number is expected, IOSTAT is not

set to a non-zero value, as is the case with most other compilers I

have had to deal with.  Instead, an error message is printed to the

terminal and the error is ignored. This interferes with YAUP's internal

error-checking routines.  If someone knows of a fix, e.g. a compiler

switch, I'd like to hear about it.  Send email to boyan@tmnxt1.iit.edu.

III. NEW FEATURES

This section describes the new features in YAUP relative to version

1.2.  Skip this section if this is the first time you are using YAUP.

- The L/R symmetry criterion (see B. I. Boyanov et al., NIM A, Feb

1, 1994) is checked, but not enforced.  That means that the

program will warn you when you are using XSYM=YES at dangerously

high energies, but will not force you to switch to XSYM=NO.

- New keywords: BASENAME, GNUPLOT.  See section IX.

- A serious bug in the spline interpolation procedures has been

fixed. The bug caused problems in calculations with large values

of NXP and/or NYP only.

- A bug in the I/O routines has been fixed.  (imsc -> imisc

conflict)

- The parser routines have been rewritten and are hopefully more

robust.  There are no changes in the input file format and the

user interface.

- The behavior of the program in MODE 7 has been fixed up.

- NXP and NYP are no longer under user control in MODE 1 and may be

set automatically in other modes.

- The default value of UPDATE is 1 (i.e. UPDATE=0 defaults to UPDATE=1).

Use UPDATE=-1 to turn off updating.

IV. INPUT FILE FORMAT

In the rest of this document keywords and file names are shown

capitalized.  This is done for clarity and is not required.  If you

are using YAUP on a system with case-sensitive file names (e.g. UNIX)

use LOWER CASE for the file names. The case of the keywords is

irrelevant as the program capitalizes all input internally.

Input is read from the file YAUP.INP which is expected to be in the

current directory.  The input consists of tab, space or

comma-delimited keywords followed by values. On any given line in the

input file all text that appears after any of the characters %,!,#,

or ; is treated as a comment (ignored).

The values are separated from the keywords by one or more spaces, tabs

equal signs.  A list of valid keywords follows :

PERIOD, NPER, NPTS

EMIN, EMAX, NE

ENERGY, CURRENT, SIGX, SIGY, SIGX1, SIGY1

DISTANCE, XPC, YPC, XPS, YPS, NXP, NYP

MODE, NSIG, TRAJECTORY, XSYM, HANNING

BFILE, TFILE

QUIET,END

The order in which the keywords appear in YAUP.INP is not important (as

long as ALL keywords are defined) but for the sake of clarity they may

be grouped in the following categories :

LINE 1 : specifies the undulator

-  PERIOD is the magnet period in centimeters (real)

-  NPER is the number of periods (integer, 150 max)

-  NPTS is the number of points PER PERIOD at which the trajectory

is to be calculated (integer, 10 min, 100 max, 40 sugg.). The

magnetic-field modulation and phase-error functions (see eq. 1)

must then be given at NPTS points per period (NPTS*NPER-1 points

total).

LINE 2 : defines the photon energy range

-  EMIN is the photon starting energy in eV (real)

-  EMAX is the final photon energy in eV (real)

-  NE is the number of points in energy at which the spectrum will

be calculated (integer, used in spectral calculations, 500 max)

LINE 3 : particle beam parameters

-  ENERGY is the electron energy in GeV (real)

-  CURRENT is the beam current in Amps (real)

-  SIGX is the horizontal beam size in mm (real)

-  SIGY is the vertical beam size in mm (real)

-  SIGX1 is the horizontal beam divergence in mrad (real)

-  SIGY1 is the vertical beam divergence in mrad (real)

LINE 4 :  pinhole size and position

-  DISTANCE is the distance to the observation point in meters. If

=0 then all pinhole sizes (defined below) are assumed to be in

angular units (mrad).  Otherwise the pinhole sizes are in

millimeters AND ALL BRIGHTNESS/POWER DENSITY DATA IS PER MM^2

RATHER THAN PER MRAD^2, E.G. PH/S/MM^2/0.1%BW.

-  XPC is the horizontal position of the pinhole center in mrad or

mm (real)

-  YPC is the vertical position of the pinhole center in mrad or

mm (real)

-  XPS is the horizontal pinhole size in mrad or mm (real) -  YPS

is the vertical pinhole size in mrad or mm (real)

-  NXP is the number of pinhole subintervals in the horizontal

direction (integer, 100 max)

-  NYP is the number of pinhole subintervals in the vertical

direction (integer, 100 max)

LINE 5 : control parameters

-  MODE=0-7 specifies the calculation mode (integer)

MODE=0 - trajectory calculation only. The settings of

all parameters on lines 2-4 are irrelevant in

this case.

MODE=1 - spectrum of the brightness from EMIN to

EMAX at NE points  at position XPC,YPC.  In this

case XPS,YPS, NXP, and NYP are irrelevant (see

Section VII).

MODE=2 - angular distribution of the brightness at

EMIN. In this case EMAX and NE are irrelevant.

MODE=3 - spectral AND angular distribution of the

brightness.

MODE=4 - spectrum of the flux through a pinhole at

NE points in the range [EMIN,EMAX].

MODE=5 - spectral distribution of the radiated power

at XPC,YPC.  In this mode XPS, YPS, NXP and NYP

are irrelevant (see Section VII).

MODE=6 - spectral AND angular distribution of the

radiated power.

MODE=7 - angular distribution of the radiated power

integrated from EMIN to EMAX.  In this case NE

determines the number of energy points at which the

spectrum will be calculated prior to the integration.

-  NSIG is the number of standard deviations of beam divergence to

include in the calculation (integer, max=5). NSIG=0 gives the

zero-emittance spectrum (in this case SIGX, SIGY, SIGX1, SIGY1

are irrelevant).

-  TRAJECTORY  is a trajectory-calculation flag

TRAJECTORY=OLD - read the trajectory from the disk

file specified by the keyword TFILE (see below).

TRAJECTORY=NEW - read the B-field distribution from a

file specified by the keyword BFILE and calculate

the trajectory  without saving it.

TRAJECTORY=NEW+KEEP - read the B-field distribution from

a file specified with the keyword BFILE, calculate

the trajectory and store it in a file specified by

the keyword TFILE.

-  XSYM=YES,NO flags whether horizontal symmetry of the spectrum

should be enforced (integer). Vertical symmetry is always

enforced. XSYM=NO - do not enforce horizontal symmetry XSYM=YES

- enforce horizontal symmetry

-  HANNING=0,1,...,NPER/2 is the number of periods at the entrance

and exit of the undulator over which a Hanning window will be

applied to the solution for the trajectory prior to the FFT.  If

HANNING=0 no window will be  applied (default).  USE WITH

EXTREME CAUTION.

LINE 6 : I/O filename specifications

-  BFILE - when TRAJECTORY=NEW or NEW+KEEP this keyword must

specify a complete directory specification of the disk file

containing the B-field distribution (<80 chars).  If

TRAJECTORY=OLD the value of BFILE is irrelevant and may be

omitted.  THE FILE NAME MUST BE ENCLOSED IN DOUBLE QUOTES.

-  TFILE - when TRAJECTORY=OLD or NEW+KEEP this keyword must

specify a complete directory specification of the disk file

containing the trajectory or to which the trajectory will be

written. The maximum length of the filenames is 80 characters.

THE FILE NAME MUST BE ENCLOSED IN DOUBLE QUOTES.  If TRAJECTORY

= NEW this keyword may be omitted.

NOTE:  unless told otherwise YAUP converts all input to upper case. On

case-sensitive systems (e.g. UNIX) this may cause problems with the

interpretation of file names.  In such cases (or any time you want to

keep the file names case-sensitive and/or want to use file names

containing blanks) enclose the file names in DOUBLE quotes, e.g.

BFILE= "/users/myname/myfile.b"

LINE 7: Miscellaneous keywords

- QUIET=YES,NO - when YAUP encounters a word that it does not

recognize it will complain loudly.  You may prevent it from

doing so by using the QUIET flag.  By default QUIET is set to

NO. Setting QUIET=YES will cause all unrecognized keywords to be

ignored.  It is your responsibility to avoid all typos and

accidental occurrences of text containing keywords.  It is

probably a good idea to set QUIET=NO (default) and comment out

all additional text in YAUP.INP

- END - all input following the keyword END is ignored

EXAMPLE 1.  Here is a sample input file

---------------------YAUP.INP begins here-------------------

;Magnet parameters

period=3.3      NPER = 75       npts=20

#Photon energy

Emin=4000.  eMax = 6000.        nE=300  ;you may mix cases as you wish

%Storage ring

ENERGY=7.0              CURRENT=0.1

SIGX=0.308              SIGY=0.085

SIGX1=0.024             SIGY1=0.009

!Pinhole (mm or mrad)

DisTaNcE = 50. ; all pinhole sizes are in mm now,

; case of keyword does not matter

XPC=0.000       XPS=2.000       NXP=50    ; this text is ignored

YPC=0.000       YPS=2.000       NYP=50

%Calculation parameters

MODE=4          NSIG=2  TRAJECTORY=NEW

XSYM=yes        HANNING=0

;Filenames

BFILE = "test.b"

;TFILE = "test.trj"

END

Note that four types of comment characters are allowed, even in the

middle of a line.  You may put anything your heart desires after an

END statement without using any comment characters, just like this

text.

---------------------YAUP.INP ends here-------------------

In this case TRAJECTORY=NEW (calculate without saving), so there is no

need to specify TFILE.  Note that comments may appear anywhere in the

input file and that empty lines are ignored. Care must be taken to

avoid comments which are identical to keywords when QUIET=YES.

EXAMPLE 2. Another input file

---------------------YAUP.INP begins here-------------------

;Magnet parameters

PERIOD=3.3      NPER = 75       NPTS=20

;Photon energy

EMIN=4000.  EMAX = 6000.        NE=300

;Storage ring

ENERGY=7.0              CURRENT=0.1

SIGX=0.308              SIGY=0.085

SIGX1=0.024             SIGY1=0.009

;Pinhole (mm or mrad)

DISTANCE = 0.  ; all pinhole sizes are now in mrad

XPC=0.000       XPS=0.000       NXP=0

YPC=0.000       YPS=0.100       NYP=50

;Calculation parameters

MODE=2          NSIG=2  trajectory=NEW+KEEP

XSYM=yes        HANNING=0

;Filenames

BFILE = "test.b"

TFILE = "test.trj"

---------------------YAUP.INP ends here-------------------

Here TRAJECTORY=NEW+KEEP (calculate and save) so both input (for the

B-field) and output (for the trajectory) filenames must be specified.

The same is true if MODE=0 regardless of the setting of TRAJECTORY

Note that you may omit the END keyword when you don't need it.

EXAMPLE 3. Yet another sample input file.

-------------------YAUP.INP begins here-------------------

;Magnet parameters

PERIOD=3.3      NPER = 75       NPTS=20

;Photon energy

EMIN=4000.  EMAX = 6000.        NE=300

;Storage ring

ENERGY=7.0              CURRENT=0.1

SIGX=0.308              SIGY=0.085

SIGX1=0.024             SIGY1=0.009

;Pinhole (mm or mrad)

DISTANCE = 50.

XPC=0.000       XPS=0.100       NXP=50

YPC=0.000       YPS=0.100       NYP=50

;Calculation parameters

MODE=4          NSIG=2  TRAJECTORY=old

XSYM=yes        HANNING=0

;Filenames

BFILE = "testbri"

TFILE = "test.trj"

---------------------YAUP.INP ends here-------------------

EXAMPLE 4. A bad example

-------------------YAUP.INP begins here-------------------

QUIET=yes

Magnet parameters - this will not cause an error

PERIOD=3.3      NPER = 75       NPTS=20

;Photon energy

EMIN=4000.  EMAX = 6000.        NE=300

QUIET=no

Storage ring - this text will make YAUP complain

ENERGY=7.0              CURRENT=0.1

SIGX=0.308              SIGY=0.085

SIGX1=0.024             SIGY1=0.009

;Pinhole (mm)

DISTANCE = 50.

XPC=0.000       XPS=0.000       NXP=0

YPC=0.000       YPS=0.100       NYP=0

;Calculation parameters

MODE=1          NSIG=2  TRAJECTORY=old

XSYM=yes        HANNING=0

; MODE=1 so YPS does not matter

;Filenames

BFILE = "test.b"

TFILE = "test.trj"

---------------------YAUP.INP ends here-------------------

In this example QUIET is initially set to YES so the line "Magnet

parameters..." will be ignored.  Later in the file, however, QUIET is

set back to NO and the line "Storage..." will cause the program to

complain.

Extensive error checking of the input stream is performed but it is

mostly for syntax errors and missing keywords.  Obvious errors like

negative ENERGY or variables out of bounds will be reported but no

other attempt is made to see if the input data makes any sense.

V. OUTPUT FILE FORMAT

All results are written to a file called YAUP-N.OUT where N is an

integer from 0 to 9.  Initially the program tries to open a file

called YAUP-0.OUT.  If the file already exists in the current

directory the program attempts to open the file YAUP-1.OUT and so on,

until a unique file name YAUP-N.OUT if found.  If the files YAUP-0.OUT

through YAUP-9.OUT already exist the program terminates with the

error message

*************

opnout:: cannot open an output file

please delete all yaup-*.* files

*************

You will have to manually delete at least one of the YAUP-*.OUT files

before attempting to restart the program.  This inconvenience is

intended to protect the results of lengthy calculations from

accidental deletion (you can tell that I did that many times, can't

you :-)).

The output file is straight ASCII and may be plotted with the program

of your choice.

VI. PROGRAM DESCRIPTION

The program assumes that the tapered undulator is a periodic array of

magnets with alternating poles and a magnet gap that varies along the

undulator axis z in a user-specified fashion.  Thus the magnet field

within the undulator is some amplitude-modulated sinusoid :

Btot(z)=B(z)*sin(2*pi/L*z+phi(z)).	(1)

Here Btot is the total field, z is the longitudinal coordinate (along

the undulator axis), L is the undulator period and phi(z) accounts for

any phase errors that may be present in the field (amplitude errors

should be included in B(z)).

The user specifies the function B(z) (in tesla) and phi(z) (in

radians).  The  equations of motion are solved numerically subject to

the boundary condition that the transverse displacement at z=0 and

z=N*L is zero (no beam steering).  Zero beam divergence is assumed in

these calculations.

Once the trajectory is available it is used to calculate the energy

radiated by a single electron traversing that trajectory in a

user-specified angular and energy range.  In order to minimize the

computation time the program tries to use certain symmetries of the

spectrum to reduce the angular range to the possible minimum.  It is

therefore very likely that the angular range of the actual

calculations (printed on the screen) will differ significantly from

the angular range requested by the user, but as long as the FINAL

results are in the requested range this should be no cause for alarm.

As mentioned earlier beam emittance may be accounted for in a first

order approximation by convoluting the zero emittance spectrum with

the e-beam velocity distribution.  Up to five standard deviations of

beam divergence may be included in the calculations at this time

which should be more than enough for all practical purposes.

VII. HINTS AND SUGGESTIONS (PLEASE, PLEASE READ)

If a pinhole covers both positive and negative angles the program may

not calculate the brightness over the requested angular range but will

rather use the allowed reflection symmetries of the spectrum to

minimize the angular range of the calculations.  In such case there

will be APPROXIMATELY (see below) NXP/2 and NYP/2 points in the

minimal angular range.  Any adjustments to the input parameters are

written to the output file so that the values saved there reflect the

ACTUAL parameters used in the calculations.

The parameter XSYM flags whether to assume reflection symmetry of the

spectrum with respect to the vertical (yz) plane.  This is a valid

symmetry for untapered undulators (if no field errors are present). In

every other case the presence or absence of this symmetry should be

verified (calculate an angular distribution of the brightness at

several energies with XSYM=NO for an on-axis pinhole and compare the

results for positive and negative x-angles) and XSYM=YES should be

used whenever possible. For an approximate analytical criterion see

the reference cited at the beginning of this document.  This criterion

is checked internally and whenever it is not satisfied and XSYM is set

to YES the program will issue a warning message:

************************************************

*EMAX may be too high.  Consider using XSYM=NO.*

************************************************

In order to calculate the convolutions with the beam divergence

efficiently (NSIG>0, all modes), the step sizes for the brightness

calculations and the sampling of the e-beam distribution have to be

identical and fixed.  Since the number of sampling points for the

particle beam distribution is limited to 78 in each direction, some

minor adjustment of the step and pinhole sizes may be needed.  The

program performs all necessary  adjustments to NXP, NYP, XPS and YPS

automatically and saves them in the output file.  The values written

to the output file are the ones used in the calculation.

Since the user-requested pinhole must be covered in an integer number

of steps minor adjustments to the pinhole sizes may also be

necessary.  This is done automatically and all changes are recorded in

the output file.

To account properly for beam emittance (NSIG>0) the program needs a

"sufficiently dense" angular/spatial mesh of points.  This is

especially true for calculations involving untapered undulators and

tapered undulators with a small amount of taper, where the

zero-emittance spectrum is a rapidly oscillating function of the angle

between the observation direction and the undulator axis.  An

angular/spatial mesh may be considered "sufficiently dense" if

doubling the number of points in angle in the X and/or Y directions

(i.e. NXP and NYP) leaves spectrum reasonably unchanged.

Insufficiently dense angular/spatial mesh will result in spurious

oscillations in the calculated spectra.

Usually NSIG=2 will do a decent job and NPTS=40 is more than enough.

Unnecessarily large values for these parameters will slow down the

execution of the program considerably and may lead to insufficient

averaging (and smoothing) of the zero- emittance spectrum.

If NXP and/or NYP=0,1 and XPS and/or YPS>0, the stepsize for the

angular-distribution calculations is chosen internally.  It is

highly recommended that you set NXP and NYP to zero when MODE=2,3,4,6.

Note that when MODE=1,5 XPS and YPS are forced to zero and the angular

stepsize is ALWAYS controlled internally (if NSIG>0).

It is a good idea to leave a blank line at the end of the input file.

If this is not done the program may respond with an error message on

some systems.

Only minor error checking of the input stream is performed.  If

inconsistent input parameters are given the results produced by the

program (if any) most likely will make no sense.  It is the user's

responsibility to ensure that the input is "reasonable" and the output

behaves "as expected" to certain changes of the input parameters. THIS

IS NOT A COMMERCIAL PRODUCT AND IS NOT DESIGNED TO BE "FOOL-PROOF".

As mentioned in the introduction, it is not recommended to use this

program for calculations of untapered undulators.  URGENT will handle

such devices much more efficiently. Programs such as URGENT, however,

cannot account for field errors or fringe fields of any kind.  If this

is what you want, YAUP may be used to do the calculations but it must

be kept in mind that the zero-emittance spectrum of untapered

undulators is a  wildly oscillating function of the angles (both

horizontal and vertical) and in some cases the convolution with the

beam emittance may not  provide sufficient "statistics" to wash out

these oscillations unless the stepsize is sufficiently small (this is

especially true for large values of NSIG and/or XPS/YPS when

MODE=2,3,4,6). In such cases it is HIGHLY desirable that you set

NXP=NYP=0 and NSIG=2 whenever possible.  If you insist on using

your own NXP/NYP, please run YAUP once with NXP=NYP=0 and note the

step sizes (in both the X and Y directions) that the program chooses.

We recommend that the step sizes for all subsequent calculations with

this configuration (i.e. number of poles, magnet period, beam energy,

AND photon energy range) does not exceed these values more than two

times whenever possible.

Use HANNING *very* cautiously.  When in doubt use HANNING=0.  Whenever

HANNING>0 *always* inspect the trajectory visually.

When interrupted the program leaves the internal files YAUP.CFG,

YAUP.S0, and YAUP.S1 on the disk.  These files must be removed

manually before restarting the program. For more info see section IX.

In some cases the program will terminate with an error message

suggesting that you use certain values of NCRIT and/or MFFT. (see

section IX).  Under normal operation (i.e. when you are not abusing

the program) they will not be encountered. If you use the suggested

values the program will probably produce results but you should be

** VERY VERY ** SUSPICIOUS of them.

VIII. DATA FILE FORMATS

The B-field file should be written in double precision to an

unformatted FORTRAN file as follows :

IMPLICIT DOUBLE PRECISION (A-H,O-Z)

.

.

.

NPTST=NPER*NPTS+1

DZ=PERIOD/NPTS

BMIN=1.D30

DO I=1,NPTST

Z=(I-1)*DZ

< calculate B(z) and phi(z) >

IF (B(I).LT.BMIN) BMIN=B(I)

END DO

OPEN(1, FILE=filename, STATUS='UNKNOWN',FORM='UNFORMATTED')

WRITE (1) PERIOD, NPER, NPTS,BMIN

WRITE(1) ( B(I), PHI(I), I=1,NPTST )

CLOSE(1)

.

.

.

In the above code NPTST is the total number of points at which the

trajectory will be calculated, PERIOD is the undulator period (in cm)

), and B (in tesla) and PHI are the values of the modulating function

and the phase errors (as defined by eq.(1)) at points Z=(I-1)*DZ,

I=1,NPTST, DZ=PERIOD/NPTS. BMIN is the minimum value of B(I),

I=1,...,NPTST.  The values of PERIOD, NPER and NPTS in the B-field

file and YAUP.INP must be the same or the program will terminate with

an error message.

A program called BFIELD.F is distributed with YAUP.F. BFIELD.F is a

standalone module that generates a B-field file for an undulator with

a linearly tapered gap in a format acceptable to YAUP.  The user must

specify the gap GZMIN at Z=Zmin=0 and the amount of gap taper

DG=(GZMAX-GZMIN)/GZMIN (either positive or negative).  The program

interpolates the gap linearly between GZMIN and GZMAX.  The field

strength dependence is specific to the proposed APS undulators but the

user may modify this section of the program according to his/her

specific needs.  Zero phase errors are assumed. See the file BFIELD.F

for more details.

The format of the trajectory file is similar to that of the B-field

file.  YAUP uses the following code to save the trajectory (when

ITRAJ=SAVE or MODE=0) :

IMPLICIT DOUBLE PRECISION (A-H,O-Z)

.

.

.

KMIN=0.934*PERIOD*BMIN

EFUND=949.*ENERGY**2/(PERIOD*(1.D0+KMIN**2/2.D0))

OPEN(1, FILE=filename,STATUS='UNKNOWN',FORM='UNFORMATTED')

WRITE(1) PERIOD,NPER,NPTS,ENERGY,EFUND

WRITE(1) ( CT(I),X(I),BETAX(I),BETAZ(I), I=1,NPTST )

CLOSE(1)

.

.

.

where PERIOD,NPER,NPTS,and NPTS are as before, ENERGY is the electron

energy in GeV, EFUND is the on-axis fundamental energy (in eV) of  a

untapered undulator with K=KMIN=0.934*PERIOD*BMIN, and CT (in

centimeters), X (in cm), BETAX, and BETAZ are the usual quantities as

a function of the longitudinal coordinate Z=(I-1)*DZ, I=1,NPTST.

Contact the authors at the addresses given at the end of this document

if you need a program that converts the trajectory and B-field files

back to ASCII format and vice-versa.

IX. "EXPERT" options.

This section deals with somewhat technical questions that concern

primarily the fine-tuning of the program execution.  It is intended

for "expert" users.  The use of the capabilities described here is

strictly optional.  If you intend to use the program as a black box

you probably do not need to read what follows.

The execution of YAUP is controlled by a number of parameters: STATUS,

UPDATE, RESOLUTION, NCRIT, MFFT, BW, and GNUPLOT.  The most

time-consuming portion of the program is the calculation of the

zero-emittance Stoke's coefficients S0 and S1. The values of these

coefficients as a function of energy and angle are written to two

files named YAUP.S0 and YAUP.S1 (defaults).  The status of the current

job is saved in the "status file" YAUP.CFG. If the program is

interrupted at any time, these three files (and the ORIGINAL YAUP.INP)

may be used to continue the calculations from the point of

interruption instead of starting from scratch. The default location

of these files is the current directory.  If you would prefer to have

them in a scratch directory (e.g. /tmp on UNIX systems) or rename

them altogether, you may do so through the use of the BASENAME keyword

(see below).  On some systems you can avoid counting these files

against your disk quota by doing this.

- BASENAME="filespec", if present, determines the basename of all

output files produced by YAUP. Here "filespec" is a file

specification (enclosed in double quotes as usual) such that

"filespec.txt" is a valid file name.  Examples of valid

filespec's are "/tmp/yaup" (UNIX), "c:\tmp\yaup1" (DOS),

"[.junk]yaup2" (VMS), "HD20:junk:yaup3" (Mac). If a basename is

given program output will be directed to the file

"filespec-n.out" (instead of YAUP-n.OUT), the Stokes'

coefficients will be written to "filespec.s0" and "filespec.s1"

(instead of YAUP.S0 and YAUP.S1), and the configuration file

name will be "filespec.cfg" (instead of YAUP.CFG). This is

particularly useful if you want to run several YAUP (batch) jobs

in the same directory. Note, however, that input is ALWAYS read

from YAUP.INP.

- The handling of YAUP.S0, YAUP.S1 and YAUP.CFG (which will be

jointly referred to as "temporary files") is controlled by the

STATUS keyword:

STATUS=NEW - start a new job.  Delete all temporary files

when done.

STATUS=NEW+KEEP - start a new job. Keep all temporary files

when done.

STATUS=OLD - continue an interrupted job from the point of

interruption.  All temporary files must already exist on

disk and are deleted at the end of the job.

STATUS=OLD+KEEP - continue an interrupted job from the point of

interruption.  All temporary files must already exist on

disk and are saved at the end of the job.

If the files were intentionally saved (with STATUS=%%%+KEEP), program

execution will resume with the 2D convolution with the beam emittance.

You may use this option to calculate the flux through a pinhole once

you have calculated the spectral and angular distributions of the

brightness in the pinhole (and vice versa).  If the file YAUP.INP

differs in any way (besides the values of MODE, TRAJECTORY, STATUS, and

UPDATE) from the original, the program will terminate with the message

*************

status:: invalid checksum found in "yaup.cfg"

cannot continue

*************

If STATUS=NEW or NEW+KEEP and any of the files YAUP.S0, YAUP.S1, and

YAUP.CFG already exist in the current  directory the program will

terminate with an error message.  You will be expected to delete these

files manually.

- UPDATE controls the frequency with which the temporary files are

updated (i.e. closed and then reopened).  If UPDATE>=0 the actual

frequency of the updates will depend on the input parameters

but approximately every UPDATE*NYP points the files will be

closed and then reopened. In the special case NXP/NYP=0 (see

section VII) the program will calculate the values of NXP/NYP

internally and will update the temporary files every UPDATE*NYP

points. When UPDATE<0 an update is forced only once - when the

zero-emittance calculations are completed (there may be other

unforced updates but that depends on the amount of temporary

data and the size of the file buffers on your system). A small

(positive) value of UPDATE will force frequent updates and will

minimize the data loss in case of a power failure or unintended

interruption but will slow down program execution somewhat.

Obviously the highest update frequency corresponds to UPDATE=1.

UPDATE=0 defaults to UPDATE=1.

- RESOLUTION controls the internal scaling of the angular/spatial

stepsize when  NXP/NYP=0 (see section VII). The program calculates

the angular width ANGW of the zero-emittance first-harmonic lobe

of an untapered undulator with the same hardware parameters tuned

to EMAX and sets the angular stepsize to ANGW/RESOLUTION. The

default is RESOLUTION=6.

- In order to avoid power-aliasing problems YAUP will try to

increase the sampling rate of the function that is FFTd until

the Nyquist energy is at least ECRIT=NCRIT/2*EFUND+EMAX eV,

where NCRIT is a parameter currently set to 20 and EFUND is

defined in section VIII. The spurious contributions to the

spectrum at any energy E the will then come from energies not

lower than E+NCRIT*EFUND. By default NCRIT=20.  This means that

for untapered undulator calculations the power aliasing

contributions will come from harmonics at least 20 orders higher

than the highest harmonic in the energy range of interest.  In

plain English, if you are calculating the undulator spectrum in

an energy range containing the fifth, sixth, and seventh

harmonic, the power aliasing contributions will come at most

from the 27th harmonic.

- To account for the beam emittance the program convolutes the

angular distribution of the brightness at each energy with the

electron divergence/spatial distribution. Since the energy

stepsize in energy of the FFTd spectrum varies with the

observation direction the spectrum must be interpolated so as

to find the values of the zero- emittance brightness at the

user-requested energies. Typically the zero-emittance spectrum

of the brightness is a sinc-like oscillating function of energy

and a good interpolation will be possible only if the energy

mesh is "sufficiently dense". The program controls the energy

stepsize of the FFT internally (it has nothing to do with the

user-specified energy stepsize as determined from EMIN, EMAX,

and NE).  The choice for an energy stepsize is  based on the

values the parameters MFFT and NCRIT and is adjusted so that

there will be approximately MFFT points per oscillation in the

spectrum.  MFFT must be chosen so that a sufficient number of

points is available to perform the interpolation described

above. The default value is 8.  Anything less than 4 seems

unacceptable (to me). MFFT MUST BE A POWER OF 2. THE PROGRAM

DOES NOT CHECK THAT.

- BW is the bandwidth of the results.  The defaults is BW=0.001

(e.g. ph/s/0.1% BW(/mrad^2 or /mm^2))

- GNUPLOT=XLINES,YLINES,NONE allows formatting of the output file

for GNUPLOT compatibility. When plotting Z=F(X,Y) 3D data

GNUPLOT requires that each set of lines in the input file that

defines a contour in the XY plane be separated from the

preceding and following sets by a blank line.  Obviously this

option is useful only when calculating 2D distributions.

XLINES - a blank line will be left after every NXP*NYP lines.

YLINES - a blank line will be left after every NYP lines

NONE - no additional formatting is performed.  This is equivalent

to omitting the GNUPLOT keyword.

In some cases the program will terminate with an error messages

suggesting that you use certain values of NCRIT and/or MFFT. Under

normal operation (i.e. when you are not abusing the program) they will

not be encountered. If you use the suggested values the program will

probably produce results but you should be ** VERY VERY ** SUSPICIOUS

of them.

EXAMPLE 5.  A demo YAUP.INP that uses the expert options.

---------------------YAUP.INP begins here-------------------

;Magnet parameters

PERIOD=3.3      NPER = 75       NPTS=20

;Photon energy

EMIN=3000.  EMAX = 600000.        NE=300

;Storage ring

ENERGY=7.0              CURRENT=0.1

SIGX=0.308              SIGY=0.085

SIGX1=0.024             SIGY1=0.009

;Pinhole (mm or mrad)

DISTANCE = 0.

XPC=0.050       XPS=.100       NXP=50

YPC=0.030       YPS=.100       NYP=50

;Calculation parameters

MODE=4          NSIG=2  TRAJECTORY=old

XSYM=yes		HANNING=0

;Filenames

BFILE = "planar-1.76-bf"

TFILE = "planar-1.76-trj"

BASENAME = "/tmp/hello"

;END

;Advanced keywords

STATUS=old+keep		UPDATE=2          RESOLUTION=12

NCRIT=25          	MFFT=0            BW=0.01

---------------------YAUP.INP ends here-------------------

In this hypothetical input file STATUS=old+save, UPDATE=2,

RESOLUTION=12, NCRIT=25, MFFT1=8 (default), and BW=0.01 (1%BW).

Setting any parameter equal to zero signals the program to use the

defaults (as defined in the parameter descriptions above). Here

BASENAME="/tmp/hello" so output will be written to /tmp/hello-0.out

and the temporary files will be /tmp/hello.s0, /tmp/hello.s1, and

/tmp/hello.cfg

X. Further information

You may copy and distribute this program freely as well as modify it

for your purposes but you may NOT distribute hacked copies.  Instead

send all constructive comments to the me and I (bib) will try to

incorporate them into future releases (if any).  Your work will be

properly credited. Also see the disclaimer in the beginning of this

document. To  request your own copy send Email to:

boyan@tmnxt1.iit.edu

boyaboy@karl.iit.edu

boyaboy@iitvax        (Bitnet)

or contact

Boyan Boyanov

Department of Physics

Illinois Institute of Technology

Chicago,IL 60616 USA

phone   (312) 567-3398

fax     (312) 567-3396

This is the only way you are sure to have the latest (and hopefully the

least buggy) version of the program as well as any future upgrades.

Please direct all bug reports, suggestions for improvements, and

requests for further assistance (preferably via Email) to the above

addresses.  Do not be afraid to complain.  The program satisfies my

own needs in its present form and unless I receive substantial

feedback I will make no effort to change/improve anything.

XI. Acknowledgments

Some parts of this program  as well as this document itself have drawn

heavily from URGENT (R. Walker and B. Diviacco) and its supporting

documentation.  The author is greatly indebted to R. Walker

(Synchrotrone Trieste) for kindly giving his permission for this.

Thanks are due to Pathikrit Bandyopadhyay for thoroughly abusing the

code :-). The development of this program was supported in part by a gift

from Amoco Corporation.

To those of you who had the patience to read through this entire piece

of crap - thank you. Let me know if it was worth the effort.

APPENDIX A. Program diagnostics

Several diagnostic files are provided with the program:

testbf.txt - sample b-field configuration

txt2u.f - program to convert taper33.txt into binary format

yaup.inp - sample input file

yaup-0.out - output generated with yaup.inp and testbf.txt

To perform program diagnostics:

1) Compile txt2u.f.  The same precautions should be used when

compiling TXT2U as when compiling yaup, i.e. use static allocation,

fold all variables to lower/upper case, and do not optimize.

2) Run txt2u with the following input:

------------------------------------------------------------

txt2u - convert text b-field files to YAUP format.

Input file name ? testbf.txt

Output file name ? test.bf

Magnet period (cm) ? 3.3

Number of periods ? 75

Number of points per period ? 40

Working...

Done.

------------------------------------------------------------

TXT2U should create a b-field file called test.bf

3) Run YAUP with the provided YAUP.INP.  This should produce the

following output on your screen

------------------------------------------------------------

YAUP 1.3.1 - Yet Another (Useless) Undulator Program

Last modified on APR-13-94

Send bug reports to boyan@tmnxt1.iit.edu

------------------------------------------------------------

Reading B-field distribution (in tesla) from file test.bf

Writing output to yaup-0.out

------------------------------------------------------------

Beginning calculation of electron trajectory as a function

of the longitudinal  coordinate. The total undulator length

is  247.5 centimeters.

Pass #     z(m)     % done        error (cm)

3       2.474     100.0        2.249587E-03

Trajectory calculations completed.

Error after the final pass (cm) :   -7.657119E-11 ( 0.00 % )

Initial betax                   :    9.092205E-05

Final   betax                   :    7.673175E-05

------------------------------------------------------------

Beginning zero-emittance brightness calculations.

emin =   7000.0   emax =  10000.0   ne = 201

xmin =   -.0480   xmax =   0.0000   nx =  30

ymin =   -.0180   ymax =   0.0000   ny =  12

x (mrad/mm)  y (mrad/mm)  % done

0.0000       0.0000      100.0

------------------------------------------------------------

Convoluting with the beam divergence.

energy (eV)            % done

10000.              100

------------------------------------------------------------

This calculation takes about 10 minutes on a 25 MHz 68040 NeXTStation.

If the output file (e.g. YAUP-1.OUT) does not reproduce the provided

YAUP-0.OUT with sufficient accuracy (1-2 % at the very least) you may

need to make some adjustments. Send DETAILED email to boyan@tmnxt1.iit.edu,

and I will try to help you solve the problem.

APPENDIX B. Error messages.

Some nice day when I have nothing better to do I'll describe all error

messages here.  Don't count on it though :-).

07/94 Chicago bib