Last modification date of this file: Mon Nov 17 00:09:32 CST 1997

Roger J. Dejus (dejus@aps.anl.gov)

C+

C PROGRAM DESCRIPTION:

C  Program to calculate on-axis brilliance tuning curves for an ideal undulator

C  insertion device (regular planar device or a helical device). The program

C  uses the Bessel function approximation which is valid for an ideal device,

C  e.g., no magnetic field errors. The effect of the particle beam emittance and

C  the beam energy spread is taken into account.

C

C AUTHORS:

C  Roger J. Dejus

C  The Advanced Photon Source

C  Experimental Facilities Division

C  Argonne National Laboratory

C

C CREATION DATE:

C  22-APR-1996

C

C INPUT PARAMETERS:

C  The input parameters are divided into sections related to the storage ring,

C  the undulator device, and the brilliance calculation.

C Machine Parameters:

C  ENERGY		Storage ring energy              (GeV)

C  CUR		        Storage ring current             (mA)

C  SIGE		        Energy spread (sigma(E)/E)

C  SIGX		        RMS beam size (horizontal)       (mm)

C  SIGY		        RMS beam size (vertical)         (mm)

C  SIGX1	        RMS beam divergence (horizontal) (mrad)

C  SIGY1	        RMS beam divergence (vertical)   (mrad)

C Undulator Parameters:

C  PERIOD	        Undulator period length          (cm)

C  N			Number of periods

C Scan Parameters:

C  EMIN			Lower energy limit for FIRST harmonic (eV)

C  EMAX			Upper energy limit for FIRST harmonic (eV)

C  NE	                Number of energy points

C  IHMIN		Minimum harmonic of interest

C  IHMAX		Maximum harmonic of interest

C  IHSTEP		Step size for the harmonics

C Brilliance Parameters:

C  IHEL			Type of device; ihel=0: regular planar, ihel=1: helical

C  METHOD               Method

C  			METHOD={0,1}Non-zero emittance;

C                                   infinite-N +convolution (Dejus' approach)

C  			METHOD=2    Non-zero emittance;

C                                   infinite-N +convolution (Walker's approach)

C  			METHOD=3    Non-zero emittance; finite-N (Walker's)

C

C  IK			Print K-values & powers; ik=0: quiet, ik=1: print

C  NEKS			Number of energy points for peak search: Default 100, or

C			use 0 for default.

C COMMON BLOCKS:

C  None.

C

C DESIGN ISSUES:

C  The down shift in energy due to the beam emittance is most noticable for

C  small values of K, and therefore, for each harmonic, the energy shift is

C  calculated for K = Kmin. The shifted peak is then used to define the energy

C  interval over which the on-axis brilliance is calculated. The peak of this

C  function is stored and subsequently saved in a file vs. energy (eV). The

C  user enters the scanning range in energy (eV) for the first harmonic, even in

C  the case when higher harmonics are chosen (using IHMIN > 1). The beam energy

C  spread is included by using a straightforward convolution at the fixed energy

C  of the peak. Beam energy spreads typically in the range 1% to 0.01% can be

C  used. It is valid to set the beam energy spread to zero. The number of energy

C  points for peak search NEKS is used at each K-value and would typically be

C  set to the default value unless the down shift in energy is being sought with

C  high accuracy. For high accuracy typically use NEKS = 1000.

C

C COPYRIGHT:

C  This routine must only be used at The Advanced Photon Source and must not

C  be tranferred or used at any other location without the consent of the

C  author.

C

C FILES USED:

C Input files

C  tc.dat

C Output files

C  tc.plt

C

C KEYWORDS:

C  Undulator Tuning Curve, Undulator On-Axis Brilliance.

C

C LINK/LIBRARY ISSUES:

C  Calls routine USB that calls routines BRIGHTE and HUNT.

C  BRIGHTE calculates the brightness and HUNT searches an array of real

C  numbers (from Numerical Recipes).

C

C PORTABILITY ISSUES:

C  Runs on DEC 3000/400 AXP alpha (Unix 4.0b) and SUN (Solaris: SunOS Release

C  5.5.1)

C

C TIMING:

C  Generally, the execution is fast. For example, the first three odd

C  harmonics (1, 3, 5) for Undulatar A at the APS over the full tuning range is

C  calculated in about 10 s using the default parameters for the infinite-N

C  method with convolution and zero beam energy spread. The finite-N method is

C  about 10 times slower. Introduction of the beam energy spread increases the

C  execution time by typically 40%. The timing above is given for the default

C  value of NEKS (=100) and the number of points/harmonic ne = 20.

C

C VERSION:

C  1.9

C

C MODIFICATION HISTORY:

C	 Date     | Name  | Description

C ----------------+-------+-----------------------------------------------------

C 10-MAY-1996     | RJD   | Tuning variable is K.

C ----------------+-------+-----------------------------------------------------

C 21-MAY-1996     | RJD   | Changed tuning variable to E1 and added descriptive

C                 |       | text describing the input/output. First official

C                 |       | release, v1.2.

C ----------------+-------+-----------------------------------------------------

C 13-MAR-1997     | RJD   | Added beam energy spread using a straightforward

C                 |       | convolution at the fixed energy of the peak.

C                 |       | Modified calculation of the variable nek. Added the

C                 |       | variable ik which controls printing of K-values.

C                 |       | Current version is v1.3.

C ----------------+-------+-----------------------------------------------------

C 18-MAR-1997     | RJD   | Modified upper limit of peak search from 1.0 to

C                 |       | fc2 = 1.002. Added parameter SPECLIM which defines

C                 |       | the minimum Brilliance to retain in the calculation.

C                 |       | Current version is v1.4.

C ----------------+-------+-----------------------------------------------------

C 21-MAR-1997     | RJD   | Modified lower limit of peak search from fc =

C                 |       | 0.990d0*ep/eiz to fc = 0.985d0*ep/eiz so that the

C                 |       | the peak of the higher odd harmonics will be found

C                 |       | (or is more likely to be found) when the beam energy

C                 |       | spread is taken into account.

C                 |       | Current version is v1.5.

C ----------------+-------+-----------------------------------------------------

C 15-JUL-1997     | RJD   | Added information about total emitted power and

C                 |       | on-axis power density to the printout when the print

C                 |       | variable ik is set to 1. Current version is v1.6.

C ----------------+-------+-----------------------------------------------------

C 29-SEP-1997     | RJD   | Added printout of zero emittance energy (first

C                 |       | column in output file). Current version is v1.7.

C ----------------+-------+-----------------------------------------------------

C 06-OCT-1997     | RJD   | The parameter NEKS which determines the number of

C                 |       | energy points for the peak search at each K-value

C                 |       | was added to the input file. Default = 100 (or enter

C                 |       | 0). Min and max is 100 and 10000, respectively.

C ----------------+-------+-----------------------------------------------------

C 14-NOV-1997     | RJD   | The variable neks declared as integer*4.

C                 |       | Current version is v1.9.

C ----------------+-------+-----------------------------------------------------

C-